UNIVERSIDAD AUTO´ NOMA DEL ESTADO DE ME´ XICO

FACULTAD DE INGENIER´IA

Disen˜o y construcci´on de un prototipo para medir la trucha arco´ıris dentro de un flujo de agua procesando im´agenes digitales

TESIS

Que para obtener el grado de Maestro en Ciencias de la Ingenier´ıa

Presenta:

Ing. Jos´e Manuel Miranda Contreras

Director de Tesis:

Dr. Marcelo Romero Huertas

Toluca, Me´xico; Octubre 2016

# Resumen

En el proceso de crianza tradicional de la trucha arco´ıris, el granjero extrae individualmente los peces de los tanques de crianza y estima su longitud de manera visual, para determinar si debe cambiarlos a otro estanque acorde a  su  talla.  Lamentablemente,  esta  medicio´n  visual  no  es  exacta  y  genera maltrato y estr´es al esp´ecimen.

En  esta  tesis  se  presenta  el  disen˜o,  construccio´n  y  evaluacio´n  de  un  nove- doso prototipo para medir la longitud de la trucha arco´ıris mientras nada contracorriente en un flujo de agua, como alternativa al m´etodo tradicional de  medicio´n  por  inspeccio´n  visual.  Este  prototipo  esta  integrado  por  siete componentes: alimentador de peces, iluminacio´n, visio´n, distribucio´n, cana- lizacio´n; as´ı como, caja de intermitencia y difusor de agua.

Con  el  desarrollo  de  esta  investigacio´n  se  presenta  una  alternativa  para  la medicio´n de la trucha arco´ıris, que en un futuro permita automatizar el pro- ceso  y  mejore  la  exactitud  de  la  medicio´n  manual,  as´ı  como,  disminuya  la manipulacio´n y estr´es de los espec´ımenes durante el proceso de crianza.

Para  estimar  la  longitud  de  la  trucha  arco´ıris  se  implementaron  dos  m´odu- los, deteccio´n y medicio´n, para procesar los videos adquiridos por la c´amara instalada dentro del prototipo. El primer mo´dulo detecta la presencia de un pez dentro de una imagen y en caso afirmativo activa el mo´dulo de medicio´n. Para la estimacio´n de la longitud se procesa la imagen del pez para detectar los puntos (x, y) que forma la silueta del cuerpo del pez y con estos puntos interpolar una curva de ajuste para estimar su longitud.

La funcionalidad de este prototipo se ha evaluado y refinado en diferentes experimentos realizados en una granja acu´ıcola de trucha arco´ıris, ubicada en el Valle del Potrero, municipio de Ocoyoacac, Estado de M´exico, M´exico.

1

2

Para reportar los resultados experimentales en esta tesis, se video grabaron diez  truchas  arco´ıris  de  talla  adulta  nadando  contra  corriente  a  trav´es  del componente de canalizacio´n del prototipo propuesto. Posteriormente, se pro- cesaron los videos capturados en la experimentacio´n para evaluar la exactitud en la estimacio´n de las longitudes. En el procesamiento de la ima´genes cap- turadas se reportan dos resultados principales, la detecci´on y medicio´n de los peces.

En los videos adquiridos los peces realizaron 46 nados, en los que el mo´dulo de deteccio´n obtuvo un porcentaje de deteccio´n exitoso de 96.7 % que representa un  total  de  244  ima´genes  de  peces.  Por  su  parte,  el  m´odulo  de  medici´on obtuvo un error absoluto medio de 1.413 cm y un error relativo medio de

5.206 %  sobre  las  longitudes  estimadas  en  las  244  im´agenes,  los  cuales  son resultados competitivos con el estado del arte.

# Tabla de contenido

- Introducci´on 	5

- Protocolo de investigacio´n 	9

  - Introduccio´n 	9

    - Antecedentes y motivaciones del proyecto	10

  - Estado del arte	11

  - Planteamiento del problema	13

  - Justificaci´on 	14

  - Meta de ingenier´ıa	15

  - Objetivos	15

    - Objetivo general	15

    - Objetivos espec´ıficos 	15

  - Metodolog´ıa 	16

  - Plan de trabajo	16

  - Infraestructura requerida	18

  - Contribuciones esperadas	19

- Measuring rainbow trout by using simple statictics	23

3

TABLA DE CONTENIDO	4

- A prototype to measure rainbow trout’s length using image processing	43

- Conclusiones y trabajo futuro	73

  - Conclusiones	73

  - Trabajo futuro	75

Cap´ıtulo 1 Introduccio´n

La  piscicultura  estudia  el  cultivo  de  organismos  acua´ticos  tanto  en  aguas oce´anicas  como  continentales,  la  cual  implica  intervenci´on  en  los  procesos de  crianza  para  mejorar  la  produccio´n  (FAO,  2016).  Esta  actividad  se  ha incrementado  dra´sticamente  a  nivel  mundial  tanto  en  produccio´n  como  en consumo en las u´ltimas d´ecadas.

La crianza de peces necesita supervisio´n para ordenar los espec´ımenes durante sus diferentes etapas de crecimiento. Dicho proceso de ordenamiento se puede realizar con base en diferentes caracter´ısticas para separar los peces en grupos consistentes (Saberioon et al., 2016; Dowlati et al., 2012; Mathiassen et al., 2011).

Por  lo  tanto,  los  investigadores  est´an  buscando  nuevos  m´etodos  o  t´ecnicas que permitan mejorar la crianza de los peces (Mathiassen et al., 2011; Dow- lati et al., 2012). Para esto, es necesario monitorear, ordenar y clasificar a los peces durante las diferentes etapas de crecimiento con el fin de evitar proble- mas durante la crianza de peces tales como: bajo crecimiento, cantidad de alimento y medicina requerida, canibalismo y competencia desleal (Dowlati et al., 2012; Zion, 2012; Blanco, 1993).

Tradicionalmente,  se  utilizan  m´etodos  manuales  para  ordenar  los  peces  en sus diferentes etapas de crecimiento, los cuales causan estr´es y dan˜o f´ısico al pez. Estas t´ecnicas convencionales de ordenamiento son laboriosas e invasivas, donde el granjero solo verifica de manera visual ciertas caracter´ısticas del pez

5

CAP´ITULO 1.   INTRODUCCIO´ N 	6

y lo ordena con base en estas (Romero et al., 2015; Zion, 2012; Saberioon et al., 2016).

En  las  u´ltimas  d´ecadas  se  ha  extendido  el  uso  de  visio´n  por  computadora como un m´etodo no invasivo y con resultados prometedores. Debido a esto, el  procesamiento  de  ima´genes en  la  piscicultura es  un  a´rea de  investigacio´n activa.  Particularmente,  la  clasificacio´n  de  peces  utilizando  procesamiento de ima´genes a trav´es de criterios predefinidos como sexo, longitud, taman˜o, masa, g´enero, forma, piel y composiciones qu´ımicas (cantidad de sal, grasa, etc.),  se  observan  m´as  utilizados  en  el  ordenamiento  de  peces  muertos  que en la planeacio´n y mejora del proceso de crianza de especies vivas (Dowlati et al., 2012; Zion, 2012; Saberioon et al., 2016; Mathiassen et al., 2011).

En  granjas  pequen˜as  generalmente  se  utilizan  estanques  de  tierra  al  aire libre para la crianza de peces. En estos estanques los peces tienen libertad de movimiento y esta´n expuestos a las variaciones de iluminacio´n ambiental. Debido a estas condiciones, los granjeros necesitan extraer a los peces para ordenarlos de acuerdo a su talla (Saberioon et al., 2016).

Por lo antes mencionado, el objetivo general y la meta de ingenirer´ıa de esta tesis son:

Objetivo general

Disen˜ar y construir un prototipo para medir la longitud de la trucha arco´ıris de  talla  adulta  dentro  de  un  flujo  de  agua,  procesando  ima´genes  digitales capturadas en l´ınea.

Meta de ingenier´ıa

Disen˜ar y construir un prototipo basado en procesamiento de ima´genes digi- tales  cuyas  caracter´ısticas t´ecnicas  permitan  medir  la  longitud  de  la  trucha arco´ıris de talla adulta al desplazarse dentro de un flujo de agua.

Alcances

Las granjas de crianza de trucha arco´ıris identifican tres tallas principales hasta su maduracio´n: alev´ın (2 a 5 cm), juvenil (5 a 12.5 cm) y adulto (12.5 a 30 cm) (Woynarovich et al., 2011). Debido al taman˜o de la trucha arco´ıris en su talla alev´ın y juvenil, por practicidad, los granjeros inspeccionan vi- sualmente  por  grupos  capturados  en  pequen˜os  recipientes  a  fin  de  separar los espec´ımenes mas grandes para cambiarlos de estanque. Con el muestreo solicitado para la evaluacio´n experimental reportada en el Cap´ıtulo 3 de esta

CAP´ITULO 1.   INTRODUCCIO´ N 	7

tesis, se tiene evidencia no solo de la inspeccio´n en grupos sino tambi´en del posible  traslape  entre  las  tallas  alev´ın  y  juvenil  (Clasificacio´n  realizada  por el granjero). Por el contrario, se observa una inspecci´on visual uno a uno de los peces adultos, por ser ´esta la verificacio´n que determina si el pez esta listo para ser consumido.

Considerando que el granjero debe inspeccionar fuera del estanque a las tru- chas  en  un  tiempo  de  1  a  3  segundos  para  evitar  que  el  esp´ecimen  muera, esta  investigacio´n  tuvo  como  premisa  el  disen˜o  de  un  prototipo  para  medir la longitud de la trucha arco´ıris mientras se desplaza en un flujo de agua.

Por  lo  antes  mencionado,  esta  investigacio´n  se  centro´  en  el  ana´lisis  de  la

trucha arco´ıris de talla adulta, para con esto determinar la viabilidad de la medicio´n  propuesta  en  el  objetivo  general  de  esta  tesis.  Igualmente,  consi- derando que la talla adulta es la mas grande que puede alcanzar la trucha arco´ıris, de resultar viable este estudio, el disen˜o del prototipo se podr´ıa ajus- tar para medir tallas alev´ın y juvenil. Por otro lado, el taman˜o de la trucha adulta facilita su separacio´n para ingresarlo al prototipo.

El  prototipo  propuesto  se  disen˜o´  con  base  a  las  dimensiones  f´ısicas  de  la

trucha  arco´ıris  de  talla  adulta.  Los  espec´ımenes  son  individualmente  mani- pulados e insertados de forma indirecta al prototipo donde se desplazan en un flujo de agua a fin de ser analizados.

Las caracter´ısticas analizadas de los peces pueden ser diversas tales como: taman˜o, g´enero, especie, peso y forma (Dowlati et al., 2012). En el caso tra- dicional de separacio´n de la trucha arco´ıris, se estima u´nicamente la longitud de manera visual por parte del granjero. De manera ana´loga al proceso tra- dicional de separacio´n, en esta investigacio´n se propone estimar la longitud del pez durante su desplazamiento como u´nica caracter´ıstica para validar el prototipo.

Organizacio´n de la tesis

Con base en los art´ıculos 57, 59 y 60 del Reglamento de los Estudios Avanza- dos de la Universidad Aut´onoma del Estado de M´exico, esta tesis de grado se desarrolla en la modalidad por art´ıculo especializado a ser publicado y esta integrada por los siguientes cap´ıtulos:

El Cap´ıtulo II - “Protocolo de investigacio´n”, presenta una versi´on actualiza- da del protocolo de tesis registrado ante la Secretar´ıa de Estudios Avanzados

CAP´ITULO 1.   INTRODUCCIO´ N 	8

de esta universidad.

El Cap´ıtulo III - “Measuring rainbow trout by using simple statictics”, pre- senta un cap´ıtulo publicado en el libro Emerging Trends in Image Processing, Computer Vision and Pattern Recognition (Romero et al., 2015). En el cual se  reporta  una  evaluacio´n  cuantitativa  en  la  medicio´n  de  la  trucha  arco´ıris utilizando un prototipo piramidal desarrollado como trabajo previo a esta investigaci´on.

El Cap´ıtulo IV - “A prototype to measure rainbow trout’s length using image processing”, incluye el art´ıculo de journal enviado a revisi´on y posible publi- cacio´n  en  la  revista  Aquacultural  Engineering  de  Elsevier.  En  este  art´ıculo se  describe  el  disen˜o  y  evaluacio´n  de  un  prototipo  para  estimar  la  longitud de la trucha arco´ıris mientras nada en un flujo de agua.

El Cap´ıtulo V - “Conclusiones y trabajo futuro”, presenta las conclusiones obtenidas con el desarrollo de esta investigacio´n e ilustra posibles oportuni- dades de trabajo futuro.

# Cap´ıtulo 2

Protocolo  de investigacio´n

### Introducci´on

Durante  la  crianza  de  la  trucha  arco´ıris  un  proceso  esencial  es  la  medicio´n de su longitud, con la cual los granjeros clasifican al pez a trav´es de su talla que puede ser alev´ın, juvenil y adulto (Blanco, 1993). Tradicionalmente, en las granjas de pequen˜os productores de la zona central de M´exico (e.g. Valle de Toluca, la Marqueza, Nevado de Toluca), esta´ actividad se lleva a cabo de manera manual por los granjeros (Romero et al., 2015).

El uso de tecnolog´ıa para mejorar el proceso de medicio´n de peces, espec´ıfica- mente t´ecnicas de procesamiento de ima´genes se muestra como una manera factible,  econ´omica  y  precisa  para  realizar  esta  tarea  durante  la  crianza  de peces en granjas pisc´ıcolas (de Dios et al., 2003; Romero et al., 2015; Shafry et al., 2012; Dowlati et al., 2012; Viazzi et al., 2015).

En  esta  tesis  se  propone  disen˜ar  y  construir  un  prototipo  para  medir  la longitud de la trucha arco´ıris dentro de un flujo de agua. En este prototipo los peces se desplazara´n uno a uno por el canal, capturando ima´genes digitales, con las cuales, se estima la longitud del esp´ecimen.

El uso de tecnolog´ıa en la medicio´n de la trucha arco´ıris supone una optimi- zacio´n  al  proceso  de  crianza  que  se  realiza  de  forma  manual  en  las  granjas pisc´ıcolas de pequen˜os productores de la zona central de M´exico.

9

CAP´ITULO 2.   PROTOCOLO DE INVESTIGACIO´ N 	10

##### Antecedentes y motivaciones del proyecto

Dentro  del  cultivo  de  peces,  la  importancia  de  la  medicio´n  del  esp´ecimen durante la crianza para realizar una clasificacio´n con base a su talla, radica en factores que afectan el crecimiento y la cantidad de alimento y medicamento requerido, as´ı como propiciar el canibalismo y la competencia desleal (Blanco, 1993; Gallego et al., 2010; Dowlati et al., 2012; Hsieh et al., 2011; Ibrahim and Wang, 2009; de Dios et al., 2003; Viazzi et al., 2015).

A la fecha, la tendencia a utilizar tecnolog´ıa para incrementar y mejorar el proceso de produccio´n de peces, espec´ıficamente el ana´lisis y procesamiento de ima´genes es un a´rea de investigacio´n activa con resultados prometedores. (Dowlati et al., 2012; Romero et al., 2015; Hsieh et al., 2011; Ibrahim and Wang, 2009; Costa et al., 2013; White et al., 2006; de Dios et al., 2003; Viazzi et al., 2015).

Sin embargo, en las granjas de pequen˜os productores (e.g. en el municipio de Ocoyoacac, Estado de M´exico), el proceso de medicio´n de la trucha arco´ıris con el cual se clasifica en tallas durante su crianza se realiza de forma manual. Este proceso consiste en la manipulacio´n de los espec´ımenes uno a uno para estimar su longitud de forma visual y con base a la experiencia del piscicultor se separan en grupos consistentes (Romero et al., 2015).

El inter´es y motivacio´n de este proyecto consiste en desarrollar un prototipo para medir la longitud de la trucha arco´ıris dentro de un flujo de agua, tomando como referencia el trabajo realizado por Romero et al. (2014). En este trabajo se desarrollo´ y present´o un novedoso prototipo para estimar la longitud de la trucha arco´ıris utilizando procesamiento de ima´genes digitales.

Finalmente, los hallazgos obtenidos a la fecha indican que la incorporacio´n de tecnolog´ıa permite mejorar el proceso de crianza de la trucha arco´ıris, para medir y clasificar automa´ticamente al pez con base a la longitud estimada del esp´ecimen  utilizando  procesamiento  de  im´agenes  digitales.  Esto  representa una oportunidad de mejora a los procesos empleados en granjas del municipio de Ocoyoacac, Estado de M´exico (Romero et al., 2015).

CAP´ITULO 2.   PROTOCOLO DE INVESTIGACIO´ N 	11

### Estado del arte

La clasificacio´n y medicio´n de peces ha atra´ıdo a la comunidad cient´ıfica para proponer y desarrollar nuevas tecnolog´ıas que ayuden a eficientar los procesos de crianza. Es de esperarse entonces una cantidad considerable de literatura publicada. A continuaci´on se describen investigaciones relevantes en el ´area.

Zion  et  al.  (1999,  2000)  creo  un  m´etodo  para clasificar  tres diferentes espe- cies  de  peces,  carpa  comu´n,  tilapia  y  salmones.  La  t´ecnica  reportada  para la  clasificacio´n  se  baso  en  momentos  invariantes  y  algunas  consideraciones

geom´etricas  como  la  longitud  y  el  a´rea  del  pez.  Los  resultados  mostrados

indican  un  0.950,  0.997  y  0.983  de  coeficiente  de  correlacio´n  de  la  longitud real con respecto a la longitud estimada de los peces, de las tres especies estudiadas respectivamente.

El trabajo mostrado por Karplus et al. (2003), se enfoco en crear y analizar tres tipos de aparatos para inducir el nado del pez lebistes a trav´es de canales estrechos, usando la respuesta positiva a la luz y al flujo de agua de este pez. El objetivo de este estudio consistio´ en preparar a los peces para ser inspeccio- nados y ordenados por un sistema de visio´n por computadora. Los resultados obtenidos indican que el nado pudo ser inducido en estos peces: 96 % y 100 % de desplazamientos exitosos en machos y hembras, respectivamente.

White et al. (2006) creo´ un sistema basado en visio´n por computadora (The CatchMeter) para identificar y medir diferentes especies de peces muertos, capturados en botes pesqueros. En este sistema, el pez es transportado a lo largo de una banda en donde son capturadas ima´genes a color RGB de vista superior. Este trabajo calcula la orientacio´n y longitud de los peces utilizando momentos invariantes. Para discriminar entre las diferentes especies se utiliza ana´lisis  discriminante  del  color.  Este  sistema  es  capaza  de  identificar  si  el pez  es  de cuerpo  alargado  o redondo  con  un 100 %  de precisio´n. Se  reporta una  clasificacio´n  de  99.9 %  entre  siete  diferentes  especies  de  peces,  con  una desviacio´n esta´ndar de 1.2 mm en las mediciones de longitud.

Utilizando  visio´n  est´ereo,  Costa  et  al.  (2006)  propusieron un  sistema  remo- to para estimar la longitud de peces en jaulas marinas. Costa et al. (2006) propusieron  una  red  neuronal  para  corregir  los  errores  de  medicio´n  y  desa- rrollaron un algoritmo para filtrar las ima´genes utilizando ana´lisis el´ıptico de Fourier para obtener las coordenadas del contorno del cuerpo del pez. Los

CAP´ITULO 2.   PROTOCOLO DE INVESTIGACIO´ N 	12

resultados  en  el  error  de  la  estimacio´n  de  la  longitud  fueron  de  5 %  de  la longitud real.

Zion  et  al.  (2007)  crearon  un  algoritmo  para  ordenar  carpa  comu´n,  tilapia y salmones. Se realizo´ una captura de ima´genes digitales de vista lateral de cada pez mientras estos nadan a trav´es de un canal trasparente. El algoritmo extrae  el  taman˜o y  las  caracter´ısticas invariantes del  contorno  de  cada  pez.

Para  realizar  la  separacio´n  se  utilizo´  un  clasificador  bayesiano,  mostrando

resultados  en  la  precisio´n  de  98.9 %,  94.2 %  and  97.7 %  para  las  especies mencionadas   respectivamente.

Dada la importancia de obtener la longitud del pez durante su crianza, Shafry et al. (2012) crearon un m´etodo llamado FileDI (Fish Length Digital Image) para  obtener  la  longitud  de  los  peces  a  partir  de  ima´genes  digitales.  Este m´etodo  utiliza  teor´ıa  o´ptica  y  t´ecnicas  de  procesamiento  de  ima´genes  para obtener la longitud de los peces. Para identificar los peces en la imagen se detectan las esquinas distintivas en el cuerpo del pez en una vista lateral, se estima la longitud del pez en pixeles (longitud de la cola a la cabeza) y con base a teor´ıa ´optica se obtiene la medida real del pez.

Adicionalmente al estado del arte presentado, existen sistemas comerciales para medir la longitud de los peces, contarlos y estimar su peso. Como es el caso del Riverwatcher Fish Counter de VAKI (VAKI, 2016), el cual mide la longitud de los peces. Este sistema es colocado en r´ıos en el paso de los peces, donde se capturan ima´genes de vista lateral para estimar su longitud. La precisio´n reportada por este sistema es de 98 % y 95 % para contar y medir respectivamente.

AutoFish Sorting (Technology, 2015) es un sistema para clasificar peces ba- sado en la longitud del pez. Este sistema desliza a los peces a trav´es de una rampa  donde  se  capturan  ima´genes  de  vista  superior  para  estimar  la  lon- gitud  del  pez  utilizando  procesamiento  de  ima´genes,  logrando  un  precisio´n reportada en la medicio´n del error de 1 mm de la longitud total.

El estado del arte muestra que el ana´lisis y procesamiento de ima´genes en la estimacio´n de la longitud del esp´ecimen es un camino factible para optimizar el proceso de clasificacio´n con base a su talla en las granjas acu´ıcolas (Dowlati et al., 2012; Viazzi et al., 2015).

CAP´ITULO 2.   PROTOCOLO DE INVESTIGACIO´ N 	13

### Planteamiento del problema

Actualmente, la medicio´n de la longitud de la trucha arco´ıris en las granjas de Valle de Toluca se realiza de forma visual, en donde, el granjero extrae uno a uno los peces, los mide visualmente y en base a su experiencia o mejor juicio los coloca en estanques de acuerdo a su talla (Romero et al., 2015). Desafortunadamente, el proceso realizado carece de exactitud (Mejia, 2013; Dowlati et al., 2012).

Como alternativa a la medicio´n visual, existen sistemas semi-automa´ticos que miden al pez durante la crianza. Ejemplos de estos sistemas son Fish Counter y AutoFish System que miden al pez utilizando procesamiento de ima´genes digitales (Vaky, 2014; Technology, 2015). El objetivo de estos sistemas con- siste en medir al pez durante el proceso de crianza manteniendo el bienestar del esp´ecimen durante el proceso. Para cumplir su objetivo se incorpora un flujo de agua constante para el paso de los peces.

Un  trabajo  previo  a  esta  investigacio´n  en  la  medicio´n  y  clasificacio´n  de  la trucha arco´ıris esta reportado en Romero et al. (2015), cuyos resultados su- gieren una alternativa para resolver el problema de la medicio´n y clasificacio´n manual en las granjas del Valle de Toluca durante el proceso de crianza. Los resultados experimentales y el conocimiento adquirido son los elementos con los  que  se  continua  esta  investigacio´n,  iniciando  con  la  identificacio´n  de  al- gunas limitantes que son discutidas a continuacio´n.

Este  prototipo  cuenta  con  sistema  de  canalizacio´n  sin  flujo  de  agua  y  el ingreso de los peces al prototipo es de forma manual. Esto es, se introducen uno a uno los peces y se capturan ima´genes para su posterior procesamiento. En  este  sistema  el  bienestar  del  pez  se  ve  afectado  por  la  manipulacio´n,  al ser  introducidos  y  retirados  uno  a  uno  del  prototipo.  Adem´as,  la  falta  de oxigenacio´n  y  flujo  de  agua  en  el  prototipo,  pone  en  riesgo  la  vida  de  los espec´ımenes utilizados durante la experimentacio´n.

De la trucha arco´ıris no se espera un comportamiento cooperativo para su medicio´n, tomando en cuenta esto, los sistemas comerciales (Vaky, 2014; Fai- vre, 2014) aspiran a los peces para hacerlos pasar por su sistema. Esto puede considerarse  como  una  forma  agresiva  de  manipulacio´n,  al  ser  arrastrado  a trav´es del sistema para su ana´lisis.

CAP´ITULO 2.   PROTOCOLO DE INVESTIGACIO´ N 	14

Por otra parte, en Romero et al. (2015) se muestran errores en la clasifi- cacio´n  de  la  trucha  arco´ıris  en  sus  diferentes  tallas  por  parte  del  granjero, observa´ndose  traslapes  en  las  tallas  clasificadas  manualmente.  Es  decir,  la medicio´n y por lo tanto la clasificacio´n manual de los peces a la fecha carece de exactitud.

### Justificaci´on

Como se menciono´ anteriormente, durante el proceso de crianza de la trucha arco´ıris,  la  medicio´n  de  su  longitud  en  granjas  de  pequen˜os  productores  de la  zona  central  de  M´exico  se  realiza  de  forma  manual,  representando  estr´es y  maltrato  a  los  espec´ımenes.  Por  lo  tanto,  esta  tesis  propone  el  disen˜o  y evaluaci´on  de  un  prototipo  para  estimar  la  longitud  de  la  trucha  arco´ıris dentro de un flujo de agua, utilizando procesamiento de ima´genes digitales.

En  un  sistema  para  la  medicio´n  de  peces  durante  la  crianza,  es  esencial contar con una t´ecnica no agresiva para desplazar el esp´ecimen. Una forma de realizar esto es proponer una t´ecnica pasiva que promueva el desplazamiento en los canales aprovechando el nado contra corriente de la trucha arco´ıris, evitando  as´ı  la  agresio´n  de  sistemas  comerciales  para  medir  al  esp´ecimen (Vaky, 2014; Technology, 2015). Sin embargo, este nado contra corriente solo implica la fuerza necesaria para permanecer fija y evitar con ello que la corriente arrastre al pez. Esto sugiere que el nado contra corriente de este esp´ecimen no necesariamente conlleva un desplazamiento.

Por  lo  antes  mencionado,  se  tiene  la  necesidad  de  disen˜ar  y  construir  un prototipo  f´ısico  que  permita  evaluar  la  medicio´n  de  la  longitud  de  la  tru- cha arco´ıris dentro de un flujo de agua, promoviendo una t´ecnica pasiva de desplazamiento, problema que no ha sido investigado a la fecha (Dowlati et al., 2012; Romero et al., 2015; Hsieh et al., 2011; Ibrahim and Wang, 2009; de Dios et al., 2003).

CAP´ITULO 2.   PROTOCOLO DE INVESTIGACIO´ N 	15

### Meta  de ingenier´ıa

Disen˜ar y construir un prototipo basado en procesamiento de ima´genes digi- tales  cuyas  caracter´ısticas t´ecnicas  permitan  medir  la  longitud  de  la  trucha arco´ıris de talla adulta al desplazarse dentro de un flujo de agua.

### Objetivos

##### Objetivo general

Disen˜ar y construir un prototipo para medir la longitud de la trucha arco´ıris de  talla  adulta  dentro  de  un  flujo  de  agua,  procesando  im´agenes  digitales capturadas en l´ınea.

##### Objetivos  espec´ıficos

      - Proponer las caracter´ısticas t´ecnicas del disen˜o de los componentes f´ısi- cos y lo´gicos del prototipo de medicio´n.

      - Analizar  experimentalmente  la  t´ecnica  de  Romero  et  al.  (2015)  para adaptar su uso en la medicio´n de la longitud de la trucha arco´ıris dentro de un flujo de agua.

      - Experimentar una t´ecnica no agresiva para promover el desplazamiento de la trucha arco´ıris en un flujo de agua que no comprometa la vida del pez durante el proceso de medicio´n.

      - Integrar  los  componentes  f´ısicos  y  lo´gicos  del  prototipo  para  evaluar su  funcionalidad  en  la  estimaci´on  de  la  longitud  de  la  trucha  arco´ıris dentro de un flujo de agua.

      - Evaluar la precisio´n de la medici´on de la trucha arco´ıris dentro de un flujo de agua con base a la talla estimada por el prototipo.

CAP´ITULO 2.   PROTOCOLO DE INVESTIGACIO´ N 	16

### Metodolog´ıa

Para el desarrollo de esta investigacio´n se propone la metodolog´ıa de ana´lisis y s´ıntesis:

An´alisis funcional y de requerimientos

Colectar y analizar el estado del arte en las investigaciones relevantes de procesamiento de ima´genes digitales aplicado a la estimaci´on de la longitud de peces.

An´alisis de factibilidad

Evaluar experimentalmente propuestas que promuevan el desplazamien- to del pez dentro de un flujo de agua.

Disen˜o modular

Desarrollar un modelo f´ısico (prototipo) que permita medir la lon- gitud de la trucha arco´ıris dentro de un flujo de agua.

Desarrollar el software requerido de procesamiento de ima´genes di- gitales que permita medir la longitud de la trucha arco´ıris dentro de un flujo de agua.

Integracio´n modular

Integrar el prototipo f´ısico de la trucha arco´ıris y el software de medi- ci´on.

Experimentacio´n

Evaluar  experimentalmente  el  prototipo  de  medicio´n  de  la  trucha  ar- co´ıris en una granja piscicola.

### Plan de trabajo

Para  el  desarrollo  de  este  proyecto  de  investigacio´n  se  desarrollo´  un  plan

de  trabajo  a  dos  an˜os  (ver  Figura  2.1),  considerando  las  siguientes  tareas principales:

An´alisis funcional y de requerimientos.

CAP´ITULO 2.   PROTOCOLO DE INVESTIGACIO´ N 	17

Revisio´n del estado del arte en la medici´on de la longitud de la trucha arco´ıris  por  procesamiento  de  im´agenes  digitales  con  base  en  carac- ter´ısticas  de forma.

Desarrollo del modelo f´ısico  (prototipo).

Desarrollo del software de medicio´n de la trucha arco´ıris.

Integracio´n del prototipo de medicio´n.

Evaluaci´on experimental del prototipo en una granja pisc´ıcola.

An´alisis de resultados y definicio´n de trabajo futuro.

Redaccio´n del documento de tesis y evaluacio´n de grado.

Figura 2.1: Cronograma de actividades a realizar para el desarrollo del pre- sente trabajo de investigacio´n.

CAP´ITULO 2.   PROTOCOLO DE INVESTIGACIO´ N 	18

Adicionalmente al desarrollo de la investigacio´n se cursaron y aprobaron las materias del programa en Ciencias de la Ingenier´ıa de la siguiente manera:

Ago.-Dic., 2014  Revisio´n del estado del arte en la medicio´n de la longitud de la trucha arco´ıris por procesamiento de ima´genes digitales, adema´s de la acreditacio´n de los cursos: Metodolog´ıa de la investigacio´n, con el Dr. Marco Antonio Ramos Corchado, Investigacio´n I, con el Dr. Mar- celo Romero Huertas, Algoritmos y complejidad, con el Dr. Jos´e Ray- mundo Marcial Romero, Estad´ıstica, con el M. en C.C H´ector Alejan- dro Montes Venegas y Matem´aticas avanzadas, con el Dr. Juan Carlos Acosta  Guadarrama.

Feb.-Jun., 2015  An´alisis funcional y de requerimientos, Desarrollo del mo- delo  f´ısico  (prototipo),  desarrollo  y  evaluacio´n  del  software  de  medi- ci´on,  adema´s  de  la  acreditacio´n  de  los  cursos:  Investigacio´n  II,  con  el Dr. Marcelo Romero Huertas, Seminario de investigacio´n I.

Ago.-Dic., 2015  Evaluacio´n del modelo f´ısico, Integracio´n y evaluacio´n del prototipo de medicio´n, Evaluacio´n experimental del prototipo en gran- jas acu´ıcolas, adema´s de la acreditacio´n de los cursos: Investigacio´n III, con el Dr. Marcelo Romero Huertas, Seminario de investigacio´n II.

Ene.-Ago., 2016  An´alisis de resultados y definicio´n de trabajo futuro, Re- daccio´n  del  documento  de  tesis  y  evaluacio´n  de  grado,  adema´s  de  la acreditacio´n de los cursos: Investigacio´n IV, con el Dr. Marcelo Rome- ro Huertas, Seminario de investigacio´n III.

### Infraestructura   requerida

Para iniciar este trabajo de investigacio´n alguna infraestructura b´asica es:

C´amaras  digitales  con  resoluci´on  m´ınima  de  640x480  pixeles,  m´ınimo 30 Fps.

Las c´amaras sera´n utilizadas para la adquisicio´n de las ima´genes digi- tales.

CAP´ITULO 2.   PROTOCOLO DE INVESTIGACIO´ N 	19

Materiales diversos para la construccio´n del prototipo (modelo f´ısico), tales como: vidrio, la´mparas, acr´ılico, madera, etc. usados en los com- ponentes que integran el prototipo.

Espec´ımenes de trucha arco´ıris viva.

La trucha arco´ıris es necesaria para la evaluacio´n experimental del pro- totipo.

Lenguaje de programacio´n y herramientas como Matlab V.2013 con los Toolbox Image Processing, Computer Vision System y Image Acquisi- tion.

Se utilizara´ Matlab como lenguaje de desarrollo y sus toolbox para el desarrollo del software de clasificacio´n.

Acceso a bibliotecas digitales especializadas, tales como: Aquacultural Engineering, Fisheries Research.

Las bibliotecas digitales marcadas muestran las tendencias e investiga- ciones relacionadas al trabajo.

Equipo  de  c´omputo  con  4GB  en  RAM,  disco duro  de  1TB,  m´ınimo  4 procesador de 3.2 GHz.

El equipo de c´omputo es necesario para el procesamiento de las ima´ge- nes digitales de la trucha arco´ıris.

### Contribuciones esperadas

Con el desarrollo de esta tesis se visualizan dos contribuciones originales:

Desarrollo de un prototipo innovador que permita estimar la longitud de la trucha arco´ıris dentro de un flujo de agua, procesando ima´genes digitales en l´ınea.

Evaluaci´on experimental del prototipo de medici´on in situ para cuan- tificar el error en la estimaci´on de la longitud de la trucha arco´ıris.

# Bibliograf´ıa

Blanco, M. (1993). La trucha: cria industrial. Acribia S. A., 2nd edition. Costa, C., Antonucci, F., Boglione, C., Menesatti, P., Vandeputte, M.,   and

Chatain, B. (2013). Automated sorting for size, sex and skeletal anomalies of cultured seabass using external shape analysis. Aquacultural Enginee- ring, 52:58 – 64.

Costa, C., Loy, A., Cataudella, S., Davis, D., and Scardi, M. (2006). Extrac- ting fish size using dual underwater cameras. Aquacultural Engineering, 35(3):218 – 227.

de Dios, J. R. M., Serna, C., and Ollero, A. (2003). Computer vision and robotics techniques in fish farms. Robotica, 21:233 –  243.

Dowlati, M., de la Guardia, M., Dowlati, M., and Mohtasebi, S. S. (2012). Application of machine-vision techniques to fish-quality assessment. TrAC Trends in Analytical Chemistry, 40:168 – 179.

Faivre       (2014).	Fish      counters.	Consultado      de http://www.faivre.fr/index.php/en/products/fish-counters. Accesado el 20/04/2015.

FAO          (2016).	Aquaculture.	Consultado       de http://www.fao.org/aquaculture/en/.  Accesado el 18/04/2016.

Gallego, I., Carrillo, R., Garc´ıa, D., Sasso, L. F., Guerrero, J., Carrillo, R. A., Garc´ıa, D., Burrola, C., White, L., Manjarrez, J. F., Zepeda, C., Aguilar, X.,  and  Sa´nchez,  A.  (2010).   Maestro sistema producto trucha Estado de M´exico [Versi´on electr´onica].  1st edition.

20

BIBLIOGRAF´IA 	21

Hsieh, C.-L., Chang, H.-Y., Chen, F.-H., Liou, J.-H., Chang, S.-K., and Lin, T.-T. (2011). A simple and effective digital imaging approach for tuna fish length measurement compatible with fishing operations. Computers and Electronics in Agriculture, 75(1):44 – 51.

Ibrahim, M. Y. and Wang, J. (2009). Mechatronics applications to fish sorting part 1: Fish size identification. In 2009 IEEE International Symposium on Industrial Electronics, pages 1978 – 1983.

Karplus, I., Gottdiener, M., and Zion, B. (2003). Guidance of single guppies (poecilia reticulata) to allow sorting by computer vision. Aquacultural Engineering, 27(3):177 – 190.

Mathiassen, J. R., Misimi, E., Bondø, M., Veliyulin, E., and Østvik, S. O. (2011). Trends in application of imaging technologies to inspection of fish and fish products. Trends in Food Science and Technology, 22(6):257 – 275.

Mejia, S. (2013). Clasificacio´n y conteo de la trucha arco´ıris utilizando visio´n artificial: revisio´n literaria y ana´lisis.  Tesis de Ingenier´ıa en la Universidad Aut´onoma del Estado de M´exico.

Romero, M., Miranda, J., Montes, H., and Acosta, J. (2014). A statistical measuring system for rainbow trout. In Proceedings on the International Conference on Image Processing, Computer Vision and Pattern Recogni- tion, page 384 – 390.

Romero, M., Miranda, J. M., and Montes-Venegas, H. A. (2015). Measu- ring Rainbow Trout by Using Simple Statistics, chapter 3, pages 39 – 53. Emerging Trends in Image Processing, Computer Vision and Pattern Re- cognition. Elsevier Inc.

Saberioon, M., Gholizadeh, A., Cisar, P., Pautsina, A., and Urban, J. (2016). Application of machine vision systems in aquaculture with emphasis on fish: state-of-the-art and key issues. Reviews in Aquaculture, 8(2):1 – 19.

Shafry, M. R., Rehman, A., Kumoi, R., Abdullah, N., and Saba, T. (2012). Filedi framework for measuring fish lenght from digital images. Interna- tional journal of the physical Science, 7(4):607 – 618.

BIBLIOGRAF´IA 	22

Technology, N. M. (2015). Northwest marine technology. Consultado de http://www.nmt.us/products/afs/afs.shtml.  Accedido el 26/05/2016.

VAKI, A. S. L. (2016). The riverwatcher fish counter. Consultado de http://www.riverwatcher.is.  Accedido el 15/06/2016.

Vaky       (2014).	Biomass      daily.	Consultado      de http://www.vaki.com/Products/BiomassDaily.  Accesado el 20/04/2015.

Viazzi, S., Hoestenberghe, S. V., Goddeeris, B., and Berckmans, D. (2015). Automatic mass estimation of jade perch scortum barcoo by computer vision. Aquacultural Engineering, 64:42 – 48.

White, D., Svellingen, C., and Strachan, N. (2006). Automated measure- ment of species and length of fish by computer vision. Fisheries Research, 80(2–3):203 – 210.

Woynarovich, A., Hoitsy, G., and Moth-Poulsen, T. (2011). Small-scale rain- bow trout farming. Food and Agriculture Organization of the United Na- tions.  Consultado  de http://www.fao.org/docrep/015/i2125e/i2125e.pdf.

Zion, B. (2012). The use of computer vision technologies in aquaculture – a review. Computers and Electronics in Agriculture, 88:125 – 132.

Zion, B., Alchanatis, V., Ostrovsky, V., Barki, A., and Karplus, I. (2007). Real-time underwater sorting of edible fish species. Computers and Elec- tronics in Agriculture, 56(1):34 – 45.

Zion, B., Shklyar, A., and Karplus, I. (1999). Sorting fish by computer vision.

Computers and Electronics in Agriculture, 23(3):175 – 187.

Zion, B., Shklyar, A., and Karplus, I. (2000). In-vivo fish sorting by computer vision. Aquacultural Engineering, 22(3):165 – 179.

# Cap´ıtulo 3

Measuring rainbow trout by using simple statictics

A continuacio´n se presenta un cap´ıtulo publicado en el libro Emerging Trends in Image Processing, Computer Vision and Pattern Recognition (ISBN:978- 0-12-802045-6) de Elsevier. En este cap´ıtulo de libro se reporta una ex- perimentacio´n  in  situ  estad´ısticamente  significativa  del  prototipo  piramidal desarrollado dentro del proyecto de investigacio´n SIyEA 3274/2012M Sistema inteligente para contar, medir y clasificar peces basado en visi´on artificial.

Para  que  el  sustentante  de  esta  tesis  continuara  con  su  investigacio´n  en  el a´rea, se consideraron los resultados y oportunidades de mejora obtenidos en esta experimentacio´n in situ.

23

Executive Editor: Steve Elliot

Editorial Project Manager: Kaitlin Herbert Project Manager: Anusha Sambamoorthy Designer: Ines Maria Cruz

Morgan Kaufmann is an imprint of Elsevier 225 Wyman Street, Waltham, MA 02451, USA

Copyright # 2015 Elsevier Inc. All rights reserved.

No part of this publication may be reproduced or transmitted in any form or by any means, electronic or mechanical, including photocopying, recording, or any information storage and retrieval system, without permission in writing from the publisher. Details on how to seek permission, further information about the Publisher’s permissions policies and our arrangements with organizations such as the Copyright Clearance Center and the Copyright Licensing Agency, can be found at our website: www.elsevier.com/permissions.

This book and the individual contributions contained in it are protected under copyright by the Publisher (other than as may be noted herein).

Notices

Knowledge and best practice in this field are constantly changing. As new research and experience broaden our understanding, changes in research methods, professional practices, or medical treatment may become necessary.

Practitioners and researchers must always rely on their own experience and knowledge in evaluating and using any information, methods, compounds, or experiments described herein. In using such information or methods they should be mindful of their own safety and the safety of others, including parties for whom they have a professional responsibility.

To the fullest extent of the law, neither the Publisher nor the authors, contributors, or editors, assume any liability for any injury and/or damage to persons or property as a matter of products liability, negligence or otherwise, or from any use or operation of any methods, products, instructions, or ideas contained in the material herein.

Library of Congress Cataloging-in-Publication Data

A catalogue record for this book is available from the Library of Congress

British Library Cataloguing-in-Publication  Data

A catalogue record for this book is available from the British Library

For information on all Morgan Kaufmann publications visit our website at http://store.elsevier.com/

This book has been manufactured using Print On Demand technology. Each copy is produced to order and is limited to black ink. The online version of this book will show color figures where appropriate.

ISBN: 978-0-12-802045-6

## Contents

Contributors	xxi

Acknowledgments	xxix

Preface	xxxi

Introduction	xxxv

PART 1 IMAGE AND SIGNAL PROCESSING

###### CHAPTER 1	Denoising camera data: Shape-adaptive noise reduction for color filter array image data	3

- Introduction	3

- Camera Noise	4

- Adaptive Raw Data Denoising	6

  - Luminance Transformation of Bayer Data	6

  - LPA-ICI for Neighborhood Estimation	7

  - Shape-Adaptive DCT and Denoising via Hard Thresholding	7

- Experiments: Image Quality vs System Performance	8

  - Visual Quality of Denoising Results	9

  - Processing Real Camera Data	10

- Video Sequences	14

  - Implementation Aspects	15

- Conclusion	15

References	16

###### CHAPTER 2	An approach to classifying four-part music

###### in multidimensional space	19

- Introduction	19

  - Related Work	19

  - Explanation of Musical Terms	19

- Collecting the Pieces—Training and Test Pieces	20

  - Downloading and Converting Files	21

  - Formatting the MusicXML	21

- Parsing MusicXML—Training and Test Pieces	23

  - Reading in Key and Divisions	24

  - Reading in Notes	24

  - Handling Note Values	25

  - Results	26

- Collecting Piece Statistics	26

  - Metrics	26

v

vi	Contents

- Collecting Classifier Statistics—Training Pieces Only	28

  - Approach	29

- Classifying Test Pieces	29

  - Classification Techniques	30

  - User Interface	31

  - Classification Steps	31

  - Testing the Classification Techniques	32

  - Classifying from Among Two Composers	32

  - Classifying from Among Three Composers	33

  - Selecting the Best Metrics	33

- Additional Composer and Metrics	34

  - Lowell Mason	34

  - Additional Metrics	36

- Conclusions	37

References	37

Further Reading	37

###### CHAPTER 3	Measuring rainbow trout by using simple statistics	39

- Introduction	39

- Experimental Prototype	40

  - Canalization System	41

  - Illumination System	41

  - Vision System	42

- Statistical Measuring Approach	42

- Experimental Framework	43

  - Testing Procedure	44

- Performance Evaluation	48

- Conclusions	52

Acknowledgments	52

References	53

###### CHAPTER 4	Fringe noise removal of retinal fundus

###### images using trimming regions	55

- Introduction	55

  - Image Processing	56

  - Retinal Image Processing	57

- Methodology	58

  - Implementation	60

- Results and Discussion	62

- Conclusion	62

References	63

###### CHAPTER

3

Measuring rainbow trout by using simple statistics

Marcelo  Romero,  Jose´   Manuel  Miranda,  Hector  A. Montes-Venegas

Facultad de Ingenierı´a,  Universidad Auto´noma  del Estado de Me´xico,    Toluca,

Estado de Me´xico,  Mexico

#### 1 INTRODUCTION

Generally, small farms use a manual measuring and counting process when cultivat- ing rainbow trout [1–4]. There are many reasons to perform such classification, but the most important are to feed the trout according to its size and to avoid cannibalism into the tanks [4]. Problems when doing a manual classification are, indistinctively, the stress and physical damage causes to the specimen when manipulated by the farmer. Moreover, we believe that this classification approach is not accurate, where the trout is taken from the water using a net and visually the farmer decide whether or not the trout should be changed to another tank.

Mexico, as well as many other countries in the world, has large hydric areas, which are ideal for aquaculture [5,6]. Taking advantage of both, its altitude and nat- ural water resources, the State of Mexico (Mexico) has particular interest in increas- ing the trout’s production as a sustainability and economic strategy for local small farmers [7]. Hence, this is a good opportunity to integrate technology to optimize the trout’s production in this region.

That is the reason which motives our research interest in the field, where we have accomplished some results, including a research project [3] and a couple of bachelor in science dissertations [1,2]. In this paper we robustly evaluate our experimental procedure to measure rainbow trout [8] in a small farm located in the Valley of Toluca, Mexico [9], where we have observed a manual classification process as illus- trated in Figure 1.

Therefore, robust experimental results are presented in this publication by using our statistical system [8] and a state-of-the-art rainbow trout image database spe- cially collected for this article. These data corpus were collected by capturing 20 images for each of 30 specimens per size (fry, fingerling, and table-fish), counting 1800 rainbow trout images.

Some related work is observed in the literature. Hsieh et al. [10] proposed a tech- nique to measure dead tuna fish using a colour pattern. In this work, the fish length is

Emerging Trends in Image Processing, Computer Vision and Pattern Recognition

http://dx.doi.org/10.1016/B978-0-12-802045-6.00003-X	39

© 2015 Elsevier Inc. All rights reserved.

40	CHAPTER 3 Measuring rainbow trout by using simple statistics

FIGURE 1

Manual measuring-classification process generally done in small farms in central Mexico. Note that this small farms use lined earth tanks.

estimated by proportional relationship between the fish body pixel length and an image reference scale. Ibrahim and Wang [11] measure four dead fish classes by con- structing a central line along the fish body from horizontal and vertical views of the fish’s body. Finally, a commercial counting and measuring system is observed in Vaki System [12]; however, there is no further information about its classification procedure.

The rest of this article is as follows. First, Section 2 describes our novel prototype designed for this research. Then, Section 3 introduces our statistical measuring approach. After that, Section 4 details our experimental framework. Next, Section 5 shows our performance evaluation. Finally, Section 6 concludes this article and draws some venues for our future work.

#### 2 EXPERIMENTAL PROTOTYPE

In this section, we describe our experimental prototype, which has been designed as part of this research.

This novel prototype is essential to collect useful trout’s 2D images; therefore, we have meticulously designed it. Figure 2 shows our experimental prototype, which has evolved from a traditional squared glass fish-cube (prototype version 1). We observed relevant issues from our first prototype and that knowledge was experimen- tally analyzed to obtain our second model. Note that our two prototypes have been experimentally evaluated in a trout farm; so, we have gathered special knowl- edge about handling the rainbow trout.

2 Experimental prototype	41

- (f)

Juvenil

(b)

(c)

(e)

(d)

FIGURE 2

Our experimental scenario for measuring rainbow trout. (a) Statistical approach within

a personal computer, (b) vision system, (c) canalization system, (d) illumination system,

- specimen to be measured, and (f) database.

Then, as observed in Figure 2, our experimental prototype consists of three main components: canalization, illumination, and vision which are aim to collect RGB trout images using a standard personal computer.

  - CANALIZATION SYSTEM

We have design a novel canalization system based on opaque-glass within our pro- totype. This canalization system poses two main properties. The first property is regarded to its trapezoidal shape, which has been decided according to the digital camera’s vision field principle. As long as such trapezoidal shape avoids reflection to be captured when taking a digital image. As its second property, we can mention that this is a two-canal tray, which prevents occlusion by taking only one fish per canal and it allows capturing two rainbow trout images per shot.

###### ILLUMINATION SYSTEM

To assist our vision system, we have integrated an illumination system, which dis- tributes light in a uniform way at the bottom of the canalisation system. To do this, a light source is located to an appropriate high to distribute light uniformly over  an

42	CHAPTER 3 Measuring rainbow trout by using simple statistics

acrylic diffuser. The light-source’s high was defined by using a bisection approach and measuring the light intensity projected into the diffuser with photo resistors. We integrated this diffuse illumination to increase contrast into the image and highlight- ing the trout’s body.

###### VISION SYSTEM

In order to explore economical technology for our classification system, we have used a standard 2D LifeCam Studio [13] camera in this experimentation. This camera is able to capture RGB-images with a maximum resolution of 1920 × 1080 pixels. In this prototype, this RGB camera is located at the top of the canalization system to capture downward-view images of the trout. Its high is proportional to the canaliza- tion base length to avoid extra data to be captured.

#### STATISTICAL  MEASURING  APPROACH

In this section, we present our statistical approach to measure rainbow trout.

Considering the rainbow trout natural swimming movement against the water flow and observing the trout from a downward point of view, we hypothesized that a third- order curve could approximate the trout’s body within the water.

Different procedures can be followed to obtain a third-order curve equation. However, we prefer a simple but effective solution that could be executed online after a trout’s image is captured.

Then, given n sample points (x,y) which depict the trout body, we apply minimum squares to compute a polynomial third-order equation [14]:

y3 ¼ a0 þ a1x þ a2x2 þ a3x3	(1)

where a0, a1, a2, a3 are constants that gain their values by solving the [4 × 4] equation system (2):

n	n	n	n

Xyi ¼ na0 þ a1Xxi þ a2Xx2 þ a3Xx3

i¼1

n

i¼1

n	n

i

i¼1

n

i

i¼1

n

Xxiyi ¼ a0Xxi þ a1Xx2 þ a2Xx3 þ a3Xx4

Xx2

i¼1 n

i

i¼1 n

i

i

i¼1 n

i

i

i¼1 n

i

x

i¼1 n

(2)

i yi ¼ a0

Xx3

i¼1 n

i yi ¼ a0

i¼1

Xx2 þ a1

i¼1 n

i

Xx3 þ a1

i¼1

Xx3 þ a2

i¼1 n

i

Xx4 þ a2

i¼1

Xx4 þ a3

i¼1 n

i

Xx5 þ a3

i¼1

X 5

i

i¼1 n

x

X 6

i

i¼1

The equation system (2) can be easily solved using the matrix notation, AX ¼ B, or more specifically: X ¼ BA—1.

After this computation, we obtained the best regression curve that adjusts the trout’s body captured into an RGB image.

- Experimental framework	43

Then, we observe that this regression curve is related to the trout’s length, which could be estimated by computing the Euclidean distance among the points within the regression curve.

Finally, given a probe-length (li) a classification can be done by comparing against training lengths. For this research, such comparison is performed by comput- ing the Mahalanobis distance [15] from training fry, fingerling, and table-trout lengths:

di ¼

li — x sx

(3)

Hence, a probe-trout ti is classified through its estimated length li by comparing its Mahalanobis distance di against a predefined threshold, which in fact is the number of standard deviations that is expected to be li to the training mean length (x).

#### 4  EXPERIMENTAL FRAMEWORK

This section presents the experimental framework to illustrate how rainbow trout is measured using our statistical approach.

As show in Figure 3, after an RGB image is taken by our prototype, we are fol- lowing a five-stage image processing to get the trout’s contour. As explained in Section 3, we are measuring the trout’s length using this contour.

To classify the trout’s image within an image, we are performing four main steps. Firstly, an RGB image of the trout is taken using our prototype (Section 2). Secondly,

FIGURE 3

Processing an incoming RGB image to estimate the trout’s length using our statistical approach.

44	CHAPTER 3 Measuring rainbow trout by using simple statistics

this RGB image is processed to obtain the trout’s contour. Thirdly, the trout’s length is estimated by applying our statistical method (Section 3) to the trout’s contour. Finally, using that estimated length, the trout is classified using a binary classifica- tion approach.

In Section 4.1, we provide more detail about our image processing step.

4.1  TESTING PROCEDURE

As illustrated in Figure 2, we have implemented a novel functional prototype which allows us to gather RGB images. After that, as shown in Figure 3, RGB images are processed using standard algorithms in the literature [16,17] until we obtain the trout’s contour. Next, we apply our statistical approach to that contour, so we can estimate the trout’s length. Finally, a binary classification approach is taken to clas- sify the trout within the income image. We now detail our experimental procedure:

- As mentioned before, we have collected a trout-image database using our prototype (illustrated in Figure 4) in a farm. This database was created

FIGURE 4

Functional prototype used within our measuring system. This prototype includes an illumination source, a pyramidal canalization compartment and a 2D camera.

4 Experimental framework	45

Table 1 Experimental Data Images

| Trout Size | Specimens | Images Per Specimen | Total Images |
| --- | --- | --- | --- |
| Fry | 30 | 20 | 600 |
| Fingerling | 30 | 20 | 600 |
| Table-fish | 30 | 20 | 600 |
| Grand total | 90 |  | 1800 |

Table 2 Training and Testing Sets

| Trout Size | Training | Testing |
| --- | --- | --- |
| Fry | 150 | 300 |
| Fingerling | 150 | 300 |
| Table-fish | 150 | 300 |
| Total | 450 | 900 |

using 30 fry, 30 fingerling, and 30 table-fish specimens, capturing 20 images per specimen. This state-of-the-art rainbow trout image database (see Table 1) was recently collected for this publication. However, for this experiment we are using only 450 images per size.

- From our database, separate training and testing sets are defined (see Table 2). Thus, 450 images for training and 900 images for testing are used. Specifically, we have three training data sets, containing 150 images per size, namely,  fry, fingerling, and table-fish trout. In every case, we selected the first

five-captured images for each of the 30 specimens per size to be part of the training set. Then, we use the next 10-captured images for each of the 30 specimens for testing. By doing this, we have three testing sets (one per trout- size) containing 300 images of fry, fingerling, and table-fish, respectively.

- From these 450 training images, training data are gathered, which in fact consist of training lengths, the arithmetic mean, and the standard deviation for each size.

- For each testing trout image, estimated lengths are gathered as illustrated in Figures 3 and 5. To do this, we gather an RGB image using our prototype. Then, we execute a five-stage image processing: gray-scaling, filtering, thresholding, closing, and contouring. Next, we estimate the trout’s length by applying our statistical approach to the contour obtained above. Finally, using this estimated length we classify the trout as fry, fingerling, or table-fish.

- To speed up our image-processing step, our vision system gathers [640 × 360] pixels RGB-images.

- Captured RGB values are converted into a grayscale by forming weighted sums of the R, G, and B components [16]:

46	CHAPTER 3 Measuring rainbow trout by using simple statistics

(a)

(b)

(c)

(d)

(e)	(f)

FIGURE 5

(g)

Image processing performed to measure a rainbow trout using our statistical approach.

  - RGB incoming image sensed by the vision system; (b) gray-scaling; (c) filtering;

(d) thresholding; (e) closing; (f) contouring; and (g) trout’s length estimated by a third order regression curve (plotted as dash line).

0:2989 m R þ 0:5870 m G þ 0:1140 m B	(4)

- Noise reduction is performed in every grayscale image by using a [3 × 3] Gaussian low-pass filter and s¼ 0.6

1	2       2	2

ð	Þ ¼

G x, y		e—ðx  þy Þ=2s

2ps2

(5)

- A binary image is obtained by using a 0.245 threshold, which was calculated experimentally from training rainbow trout images.

4 Experimental framework	47

- The trout’s body is emphasized by using a closing operation, first erosion, and then dilation with a [5 × 8] mask. This operation is the key to eliminate small clusters of pixels around the trout’s body cluster.

- The trout’s contour is obtained by removing interior pixels. In this case, a pixel is set to 0 if all its four-connected neighbours are 1, thus leaving only the boundary pixels on as shown in Equation (6):

1

If   1  x   1

1

Then

1

1  0  1

1

(6)

- Using the trout’s contour, we apply our statistical measuring approach detailed in Section 3.

- By definition, the trout’s size is estimated by computing the Mahalanobis distance from this estimated length to training data (Equation 3).

- For classification in this experiment, imagine that the complete testing-trout set (900 in total) is passed through a grid one by one in three steps. First, the grid is sized to filter only fry-trout. Then, every trout able to pass this grid is labelled as a fry-trout. Second, for the rest of the testing set, the grid is now sized to filter fingerling-trout. Every trout that is able to pass this grid is labelled as fingerling-trout. Third, for the rest of the testing set, the grid is now sized to filter table-fish trout.

Remember that we are computing the Mahalanobis distance and this allows us to easily implement the approach above using fry, fingerling, and table-fish training data, respectively. Referring as training data the arithmetic mean and the standard deviation from each size.

Another advantage in using Mahalanobis distance is that we can make our classification process as rigid as we decide, by defining a threshold in number of standard deviations.

Then, in this article we are reporting classification figures from one to three standard deviations.

- We are considering this experiment as a binary classification problem, as illustrated in Table 3. By doing this, we are collecting true positive (TP), false positive (FP), false negative (FN), and true negative (TN) frequencies [18].

- Using values in Table 3, performance figures are generated by computing accuracy, repeatability, specificity, recall, and precision metrics when

Table 3 Binary Classification

|  | Actual Positive | Actual Negative |
| --- | --- | --- |
| Predicted positive Predicted negative | TP FN | FP TN |

48	CHAPTER 3 Measuring rainbow trout by using simple statistics

classifying as fry, fingerling, and table-fish trout. In every case, we are evaluating using as threshold from one to three standard deviations.

Accuracy, a degree of veracity, is a measurement of how well the binary classification test correctly identifies a rainbow trout’s size.

TP þ TN

Accuracy ¼ TP

þ TN

þ FP þ

(7)

FN

Repeatability, a degree of reproducibility, is an indicator about how robustly a rainbow trout size can be identified:

TP

þ

Repeatability ¼ TP

(8)

FP

Specificity, a degree of speciality, rates how negative rainbow trout’s size is correctly identified:

TN

þ

Specificity ¼ TN

(9)

FP

Recall measures the fraction of positive examples that are correctly labelled:

TP

þ

Recall ¼ TP

(10)

FN

Precision measures that fraction of examples classified as positive that are truly positive:

TP

Precision ¼ TP

(11)

þ FP

#### 5  PERFORMANCE EVALUATION

We now present performance figures when using our statistical model to measure rainbow trout in a farm.

As observed in Figure 5, our statistical approach’s performance to measure a rain- bow trout depends on our image processing stage. However, according to our exper- imental results, we believe that we have addressed main issues about capturing and processing an RGB image within our system.

As we have mentioned before, we consider this as a binary classification problem as indicated in step 13 in our experimental procedure (Section 4.1). Thus, the complete testing lengths (computed from 900 images) are compared against fry training data, using Mahalanobis distance. Then, if a testing length falls into a predefined threshold (one to three standard deviations), the respective testing trout is marked as fry-trout. Next, all remaining testing lengths are compared against fingerling training data using Mahalanobis distance as well. Again, if the testing length falls into a predefined thresh- old (one to three standard deviations), we label the respective testing trout as fingerling-trout. Then, every remaining testing length is compared against table-fish

5 Performance evaluation	49

training data using Mahalanobis distance. Similarly, if the testing length falls into a predefined threshold (one to three standard deviations), the testing trout is labelled as table-fish-trout.

Then, as prescribed in Table 3 we count TP, FP, TN, and FN frequencies, which are summarized from Tables 4 to 6. Hence, by using these values we are able to com- pute accuracy, repeatability, and specificity metrics, which are presented from Tables 7 to 9 and illustrated in Figure 6.

Finally, we are computing recall and precision metrics. Tables 10 and 11 sum- marize these results and Figure 7 shows recall and precision results at three standard deviations.

Observing our experimental results when classifying our testing set, we score the best precision at three standard deviations: 95.93%, 93.21%, and 96.25% for fry, fin- gerling, and table-fish trout, respectively.

Table 4 Frequency when Classifying a Probe Set as Fry Trout at Three Standard Deviations

|  | 1Sx | 2Sx | 3Sx |
| --- | --- | --- | --- |
| TP | 201 | 259 | 276 |
| FP | 71 | 147 | 234 |
| TN | 529 | 453 | 366 |
| FN | 99 | 41 | 24 |

Table 5 Frequency when Classifying as Fingerling Trout at Three Standard Deviations

|  | 1Sx | 2Sx | 3Sx |
| --- | --- | --- | --- |
| TP | 149 | 108 | 39 |
| FP | 40 | 27 | 36 |
| TN | 357 | 309 | 279 |
| FN | 82 | 50 | 36 |

Table 6 Frequency when Classifying as Table-Fish Trout at Three Standard Deviations

|  | 1Sx | 2Sx | 3Sx |
| --- | --- | --- | --- |
| TP | 165 | 261 | 257 |
| FP | 7 | 19 | 10 |
| TN | 141 | 58 | 48 |
| FN | 126 | 21 | 0 |

50	CHAPTER 3 Measuring rainbow trout by using simple statistics

Table 7 Accuracy when Classifying a Probe Set at Three Standard Deviations

| Testing as | 1Sx (%) | 2Sx (%) | 3Sx (%) |
| --- | --- | --- | --- |
| Fry | 81.11 | 80.57 | 69.70 |
| Fingerling | 79.11 | 84.41 | 88.86 |
| Table-fish | 71.33 | 81.54 | 96.83 |

Table 8 Repeatability when Classifying a Probe Set at Three Standard Deviations

| Testing as | 1Sx (%) | 2Sx (%) | 3Sx (%) |
| --- | --- | --- | --- |
| Fry | 73.90 | 78.84 | 95.93 |
| Fingerling | 63.79 | 80.00 | 93.21 |
| Table-fish | 54.12 | 52.00 | 96.25 |

Table 9 Specificity when Classifying a Probe Set at Three Standard Deviations

| Testing as | 1Sx (%) | 2Sx (%) | 3Sx (%) |
| --- | --- | --- | --- |
| Fry | 88.17 | 89.92 | 95.27 |
| Fingerling | 75.50 | 91.96 | 75.32 |
| Table-fish | 61.00 | 88.57 | 82.76 |

FIGURE 6

Accuracy, repeatability, and specificity performance when classifying fry, fingerling, and table-fish trout at three standard deviations using our statistical measuring system.

5 Performance evaluation	51

Table 10 Recall when Classifying Testing Sets at Three Standard Deviations

| Testing as | 1Sx (%) | 2Sx (%) | 3Sx (%) |
| --- | --- | --- | --- |
| Fry | 67.00 | 64.50 | 56.70 |
| Fingerling | 86.33 | 68.35 | 92.55 |
| Table-fish | 92.00 | 52.00 | 100.00 |

Table 11 Precision when Classifying Testing Sets at Three Standard Deviations

| Testing as | 1Sx (%) | 2Sx (%) | 3Sx (%) |
| --- | --- | --- | --- |
| Fry | 73.90 | 78.84 | 95.93 |
| Fingerling | 63.79 | 80.00 | 93.21 |
| Table-fish | 54.12 | 52.00 | 96.25 |

FIGURE 7

Recall-precision metrics when classifying fry, fingerling, and table-fish trout at three standard deviations using our statistical measuring system.

Furthermore, when classifying those testing lengths, we score a 100% recall and 96.25% precision when classifying table-fish trout at three standard deviations.

These experimental results are not only motivated, but also valuable evidences that indicate effectiveness in our classification system.

52	CHAPTER 3 Measuring rainbow trout by using simple statistics

FIGURE 8

Processing a table-fish trout image for classification using our statistical measuring system.

#### 6 CONCLUSIONS

In this article, we have robustly evaluated our statistical system to measure rainbow trout in farm using computer vision as observed in Figure 8. This novel technique [8] is a simple but effective statistical method, which has been evaluated in a small farm in central Mexico named Rincon del Sol [9].

For this research, we have designed and implemented a functional prototype to collect RGB trout images. This prototype includes canalization, illumination, and vision components that have been meticulously assembled. Also, we believe that this prototype could be easily integrated into a mechanical system to interconnect lined earth tanks in farms.

Our experimental results encourage our research as they have shown that our classification system is effective, where 95.93%, 93.21%, and 96.25% precisions are observed when classifying fry, fingerling, and table-fish trout, respectively.

It is important to observe that, although our statistical approach has been inspired to measure rainbow trout, this approach can be applied to other fishes grown in farms. As part of our future work, we are integrating water flow and stereovision into our prototype to investigate two main issues: the light reflection into the water and the presence of turbulence. Our final aim is to implement an economical classification

system for small farms.

#### ACKNOWLEDGMENTS

Authors thank to the Research Department of the Autonomous University of the State of Mexico (UAEMex) for its  financial  support  through  the  research  project  SIyEA/  32742012 M. Their gratitude also goes to the Mexican Council for Science and Technology (CONACyT) for the scholarship granted to Jose Manuel Miranda (634478).

References	53

REFERENCES

[1]  Mejia S. Conteo y clasificacio´n de la trucha arco´ıris utilizando visio´n artificial: revisio´n literaria  y  ana´lisis  [BSc.  Final  dissertation].  Mexico:  Engineering  Department,  Auto- nomous University of the State of Mexico;   2013.

[2] Miranda J. Prototipo de un sistema clasificador de la trucha arco´ıris utilizando un modelo estad´ıstico de su longitud obtenido de ima´genes 2D [BSc thesis]. Mexico: Engineering Department, Autonomous University of the State of Mexico;    2014.

[3] Romero M, Vilchis A, Portillo O. Intelligent system to count, measure and classify fishes using computer vision. Research project SIyEA 32742012 M. Mexico: Autonomous University of the State of Mexico;   2012.

[4] Woynarovich A, Hoitsy G, Moth-Poulsen T. Small-scale rainbow trout farming. FAO Fisheries and aquaculture technical paper, Food and Agriculture Organization of the United  Nations; 2011.

[5]  CONAGUA, Comisio´n Nacional del Agua. Concesio´n de Aprovechamiento de Aguas Superficiales. Water National Council; 2006. [Online] Available from: http://www.con agua.gob.mx/Contenido.aspx?n1¼5&n2¼101&n3¼302&n4¼302. [Accessed: 13 Sep- tember 2014].

[6]  DOF, Diario Oficial de la Federacio´n. Ley de aguas nacionales. DCVII(14): 27–95. Fed- eral Oficial News. [Online] Available from: http://dof.gob.mx/nota_detalle.php? codigo¼670229&fecha¼12/05/2004. [Accessed: 13 September 2014];   2004.

[7] Gallego I, Carrillo R, Garc´ıa D, Sasso I, Guerrero J, Burrola C, et al. Programa maestro, sistema producto trucha del Estado de Me´xico. Government Plan for Rainbow Trout Production. Mexico, Autonomous University of the State of Mexico; 2007.

[8] Romero M, Miranda JM, Montes HA, Acosta JC. A statistical measuring system for rainbow trout, In: Proceedings on the international conference on image processing, computer vision and pattern recognition. Las Vegas NE, United States of America; 2014.

[9] Rincon del Sol. Small Trout Farm. La Marqueza, State of Mexico, Mexico; 2014. [Vis- ited: 20 August  2014].

[10] Hsieh C, Chang H, Chen F, Liou J, Chang S, Lin T. A simple and effective digital imag- ing approach for tuna fish length measurement compatible with fishing operations. Com- put  Electron  Agric 2011;75:44–51.

[11] Ibrahim M, Wang J. Mechatronics applications to fish sorting part 1: fish size identifi- cation. industrial electronics (ISIE 2009). In: IEEE international symposium; 2009.

[12] Vaki System. Bioscanner Fish Counter. [Online] Available from: http://www.vaki.is/ Products/BioscannerFishCounter/. [Accessed: 10 May 2014];   2013.

[13] LifeCam Studio. LifeCam Studio. [Online] Available from: http://www.microsoft.com/ hardware/en-us/p/lifecam-studio. [Accessed: 13 September 2014];    2014.

[14] Murray S, Schiller J, Srinivasan R. Probability and Statistics. 4th ed. USA: Mc Graw-Hill; 2012.

[15] Duda R, Hart P, Stork D. Pattern classification. 2nd ed. USA: Wiley Interscience; 2001. [16]  Gonzales R, Woods R. Digital image processing. 3rd ed. USA: Prentice Hall; 2008.    [17]     Gonzales R, Woods R, Eddins S. Digital image processing using Matlab. 2nd ed. USA:

Prentice-Hall; 2009.

[18] Davis J, Goadrich M. The relationship between precision-recall and ROC curves. In: Pro- ceedings of the 23rd International Conference on Machine Learning; 2006.

# Cap´ıtulo 4

A prototype to measure rainbow trout’s length using image processing

En este cap´ıtulo se presenta el manuscrito que reporta el trabajo de inves- tigacio´n en la medici´on de la trucha arco´ıris por procesamiento de ima´genes digitales, el cual se envio´ para su revisio´n y posible publicacio´n en la revista internacional Aquacultural Engineering (ISSN: 0144-8609) de Elsevier. Esta revista internacional tiene un factor de impacto 1.381 y se encuentra indexa- da en: Scopus, Science Direct, Mendeley, Evolve, Knovel, Reaxys y Clinical Key.

43

Manuscript Details

Manuscript number	AQUE_2016_75

Title	A prototype to measure rainbow trout's length using image processing

Article type	Full Length Article Abstract

In rainbow trout farming, automatic measuring for classification is an open problem, when in most of small farms this work is done manually within a laborious process and achieving inaccurate measurements. In this paper we present state of the art results in rainbow trout's (Oncorhynchus mykiss) length estimation within a water flow using image processing. For this purpose, we have designed, implemented and evaluated a novel measuring prototype which allows the fish swimming through out its channel in order to be measured and classified, taking advantage of the rainbow trout's instinctive behaviour in swimming against the water flow. Our prototype is provided with a vision component which is able to detect and measure the rainbow trout online by capturing and processing downward-view images when the fish passes below the camera. A fish is detected into the system when it cross a control point defined for this purpose and to activate our measuring function. Based on our previous work, we are approximating a third order regression curve to the fish's body to estimated its length. In our experimental evaluation we are achieving

1.413 cm mean absolute error (MAE) when estimating rainbow trout's lengths. This is an encouraging result and it allows us to draw different venues for future work based on our experimental findings.

Keywords	Measuring fish prototype; rainbow trout; image processing

Corresponding Author	Marcelo Romero

Corresponding Author's Institution

Universidad Autonoma del Estado de Mexico

Order of Authors	Jose Manuel Miranda, Marcelo Romero

Suggested reviewers	Daury Garcia-Pulido

A prototype to measure rainbow trout’s length using image processing

Jose Manuel Miranda, Marcelo Romero∗

Facultad de Ingenier´ıa, Universidad Auto´noma del Estado de M´exico.  Cerro de Coatepec s/n.  Ciudad Universitaria, 50100, Toluca, Estado de M´exico, M´exico

Abstract

In rainbow trout farming, automatic measuring for classification is an open problem, when in most of small farms this work is done manually within a laborious process and achieving inaccurate measurements. In this research we present state of the art results in rainbow trout (Oncorhynchus mykiss) length estimation within a water flow using image processing. For this purpose, we have designed, implemented and evaluated a novel mea- suring prototype which allows the fish to swim throughout its channel in order to be measured and classified, taking advantage of the rainbow trout instinctive behaviour in swimming against the water flow.   Our prototype   is provided with a vision component which is able to detect and measure    the rainbow trout online by capturing and processing downward-view images when the fish passes below a camera. A fish is detected into the system  when it crosses a control point, event that recalls our measuring process. To measure, we approximate a third order regression curve to the fish body to estimate its length. In our experimental evaluation we are achieving 1.413 cm mean absolute error (MAE) when estimating rainbow trout lengths. This is an encouraging result that allows us to draw different venues for future work based on our experimental findings.

Keywords:	Measuring fish prototype, rainbow trout, image processing

∗Corresponding autor:  Tel.  +52 722 214 08  55

Email address: mromeroh@uaemex.mx (Marcelo Romero )

Preprint submitted to Aquacultural Engineering	September 23, 2016

1    1. Introduction

2	Aquaculture is the farming of aquatic organisms in both coastal and in-

3  land  waters  that  involves  interventions  in  the  rearing  process  to  enhance

4    production (FAO, 2016).

5	From  last decades,  aquaculture has increased and extended    worldwide

6             in both production and consumption of aquatic products, demanding from

7  farmers better fish rearing processes for consumers who are expecting quality,

8    freshness and authenticity on their products (Saberioon et al., 2016;  Dowlati

9    et al., 2012; Mathiassen et al.,  2011).

10	Fish farming needs supervision to sort out the fish during different growth

11    stages (fry, fingerling and table-fish) to avoid severe low growth, cannibalism

12               and unfair competition;  and also to provide required amount of food and

13    medicine.  This sorting process is done through certain characteristics,   such

14    as:  size,  gender,  species,  weight  and shape to separate into consisting  fish

15            groups (Dowlati et al., 2012; Zion, 2012; Dowlati et al., 2012; Mathiassen

16    et al., 2011).

17	The estimated fish length allows the farmer to infer the fish size and

18    to sort out the specimen during their different growth stages.     The fish size

19    helps farmers to have better decisions about grading and harvesting. Besides,

20    farmers can supervise growth rate and plan accurate feeding to avoid  excess

21               of food as it is one of the main water pollution contributors (Beddow and

22      Ross, 1996).

23	Despite of  the advance in  technology (Northwest  Marine    Technology,

24    2016; VAKI, 2016), traditional manual methods are still used in small  farms

25    to estimate the fish length during their different growth stages.    Indistinctly,

26    those methods not only cause stress and physical damage to the fish, but also

27    they are usually time-consuming,  costly,  laborious and invasive  (Saberioon

28    et al., 2016; Romero et al., 2015; Zion, 2012).

29	Because in a manual sorting, one fish at a time is extracted from the

30    water  to  be  visually  inspected and measured.	Based  on  this judgement,

31    the farmer makes a decision, e.g.       whether the fish needs to be changed to

32    another pond or if it is ready for human consumption.      Unfortunately,  this

33    visual measurement is not necessarily accurate (Romero et al., 2015;  Shafry

34    et al., 2012).

35	About fish farming in Mexico, rainbow trout (Oncorhynchus mykiss)   is

36    one of the most cultivated fish for feeding.  Especially in high altitude  states

37    where the water’s conditions such as low temperature and cleanness are suit-

2

38    able to rear this specimen  (Romero et al., 2015; Montero, 2013).      Further-

39              more,  this fish is easily adapted to quite diverse environments,  hence,  it

40                is possible to thrive it in pond’s hatcheries (Frost  et al., 2013; Staley and

41      Mueller, 2000).

42	Researchers from different disciplines have been exploring possible    ap-

43    proaches to improve productivity and profitability of fish rearing.     Specifi-

44    cally,  image processing has been showed as a non-invasive and   economical

45    method for fish length measurement, which is able to achieve promising   re-

46             sults (Saberioon et al., 2016; Dowlati et al., 2012; Zion, 2012; Mathiassen

47      et al., 2011).       However,  image processing  has been mostly used on dead fish

48     within the food industry and it has not been widely explored in aquacul-

49    ture (Zion, 2012), this could be because of the natural challenge to   measure

50    accurately in an alive fish.

51	This research aim is to measure the rainbow trout length (table-size)

52    within a water flow, indirectly manipulated by using image processing.    For

53              this reason,  our experimentation was  performed in a small rainbow trout

54              farm named Rincon del Sol, which is located in Ocoyoacac, State of Mexico

55    (Mexico), where as in most of small farms, earth ponds are used for  rearing.

56    1.1. Related work

57	For decades, researchers have been advocated to investigate   morpholog-

58    ical characteristics such as fishes’ length and mass from digital images.     In

59    this section we highlight key  literature.

60	Zion et al. (1999) developed and tested an image process for  discrimina-

61    tion between images of three different species: common carp fish, St. Peter’s

62    fish and grey mullet.      This method is based on invariant  moments coupled

63    with geometric considerations from the fish head and tail.    Downward-view

64    fish images were acquired using their illuminated chamber, by placing   each

65    dead fish at the bottom of it.  They reported correlation coefficients  between

66    manually measured lengths with estimated lengths as 0.950, 0.997 and 0.983

67    for fish common carp, St. Peter’s fish and grey mullet,  respectively.

68	In a later work, Zion et al. (2000) used (Zion et al., 1999) the same

69             method to create what it is believed the first discrimination system among

70    fish species in vivo.  To do this, they created a concrete fish pond to take side

71    view images when fish passed through an elongated narrow and   transparent

72    channel. Although, their fish species identification reached 100, 91 and 91%

73    for grey mullet,  carp and St.   Peter’s fish,  respectively;  a systematic     fish

74            training was  required for the fish to know how  to swim along the channel

3

75    for identification.        For  this reason,  it’s thought that this is an impractical

76    approach to be implemented in fish farms.

77	Odone et al. (2001) described a trainable system capable of   determining

78    fish weight from image measurements.  They proposed a prototype that   was

79    installed as an automatic fish grading device at a fish farm, which was able to

80    grade fish at a rate of three fish per second.  From live trout sliding  through-

81    out a transparent channel, image measurements were taken from top and side

82    views.  They trained a support vector machine using linear and    polynomial

83          kernels to learn the relationship between fish shape and weight parameters.

84    For their intended fish-weight estimation, they used 13 different shape   mea-

85    surements that are based on fish area,  perimeter,  length and width.     Using

86              a quadratic kernel and their 13 fish features,  their best weight  estimation

87    scored a 3% absolute error with a 2% absolute standard deviation.         They

88    also noticed that fish area measurements were more accurate to estimate fish

89    weights than fish length measurements, which are typically used in  biology.

90	Karplus et al. (2003) created and tested three types of apparatus to induce

91   guppy swimming throughout narrow channels using their positive phototactic

92              and rheotactic innate response,  to be inspected and sorted by  a computer

93    vision system.     They obtain results of 96% and 100% in female and males,

94    respectively, when leading them to a selected channel  output.

95	White et al. (2006) created a computer vision machine (The CatchMeter)

96    to identify and measure different species of dead fish.  In this system, a   fish

97             is moved  through a conveyor  belt out,  where digital images are captured

98             to determine the fish orientation by  using the invariant  moments method.

99              They obtained 100% accuracy in determining whether the fish is rounded

100          or flat, with a standard deviation of 1.2 mm in length and a 99% successful

101      classification among seven  fishes.

102	Using stereo vision, Costa et al. (2006) proposed a submersible dual cam-

103    era module connected to a portable water PC equipped with two frame  grab-

104    bers.  To  obtain binary images, they used filtering and segmentation with    a

105    fixed threshold. Major length and circularity axis were analysed from images

106    segments and those that fit into pre-established ranges where considered    to

107    belong to the fish.  Then, landmark points in stereo image pairs were located

108    for geometry calculation.  Distances between key points were calculated   by

109    locating in 3D the fish boundary.  A 5% fish-length estimation error from   a

110    single measurement of a fish model is reported.

111	In a comparative study of nine fish-size sorting methods using  computer

112           vision,  Ibrahim and Sultana (2006) set up five complexity levels based on

4

113    how fish is fed to their system’s conveyor belt.      They concluded that, even

114    though, methods based on computer vision have  shown aspiring   prospects,

115    their detailed analysis found no satisfactory of fish-size sorting techniques in

116    terms of speed, accuracy and handling of complexities altogether. Therefore,

117    they said that there is an opportunity to do further research on   automatic

118    fish-size sorting for industrial use.

119	Zion et al. (2007) introduced a technique to sort out common carp, St.

120    Peter’s fish and grey mullet.  Side view fish images were acquired by a com-

121  puter vision system while swimming through a narrow channel with their side

122    to the camera at a relatively constant distance.  To  overcome water   opaque-

123    ness, the channel is backlit by a series of fluorescent lights to generate   high

124    image contrast.  Size- and orientation-invariant features are extracted    from

125    the fish silhouettes.  Using Bayes classification in a laboratory prototype,  an

126    accuracy of 98.9%, 94.2% and 97.7% is achieved by grey mullet, St.  Peter’s

127    fish and carp images, respectively.

128	Shafry et al. (2012) proposed a framework (FileDI) to measure the fish

129             length using optical theory (Hsu, 2008) and images processing techniques

130    that  automatically  measures  dead  fish  length  (Abdullah  et  al.,  2009), by

131    processing  an  input  image  to  detect  fish  head and tail.	This framework

132    consists of five main stages: Pre-processing, bending ratio plot, corner factor

133   calculation, fish length estimation from image and calculating the fish length.

134      This framework was used to measure R. Kanagurta and S.    Crumenophthal-

135    mus fish (twenty specimens each), using two types of camera,    two types of

136    illumination and three camera positions.  Although, an overall results  analy-

137    sis was not presented in detail, they claimed a 95% accuracy for fish   length

138     measurement.

139	Torisawa et al. (2011) developed a simple method using direct linear trans-

140    form and Move − tr/3DT M   software to obtain three-dimensional  measure-

141               ments of cultured Pacific blue-fin tuna, which were freely swimming in a net

142    cage.  They believed that information obtained from stereo images could   be

143    useful for managing the growth of tuna during rearing, for that purpose  they

144    particularly estimated the fork length of individual fish.  They tried to calcu-

145           late the size of all recorded tuna in which they could distinguish the snouts

146    and tail forks during monitoring.     However, they were able to estimate fish

147    length no farther than 5.5 m from the camera system.  Then, they only  could

148             accurately estimate 99% sizes (106/107) of tuna recorded at that distance,

149    with error ratios (standard error/mean) less than 5%.

150	Costa et al. (2013) created a methodological tool applicable to order

5

151    farmed seabass by size, sex and skeletal anomalies by using elliptic    Fourier

152    analysis on digital images and partial least squares modelling.  Their   RGB-

153    image processing includes seven steps: two-channels image transform: G and

154    V;  background  value  as the mean of G and  V on the darker  pixels    (5%);

155    Euclidean distances from the background value; elimination of the  extremes

156      (1%);  rescaling;  edging  canny;  and  dilate  (3)  &  fill  filtering. The  proposed

157  techniques  integration  produced  size  estimation  (in  weight)  with  a  better

158    regression efficiency (r=0.9772) than the commonly used logarithmic   mea-

159    sured body length (r=0.9443).       The two  partial least squares discriminant

160                analysis models used to select sex and malformed fish also returned high

161   discrimination efficiencies (82.05% and 88.21%, respectively).

162	Viazzi et al. (2015) tested and evaluated a computer vision technique that

163    estimates the mass of dead jade perch fish, capturing a set of 120 images out-

164    side the water and extracting different features using single-factor regression

165    on a fish area image without considering the fin tail.  Their image processing

166    consisted of four consecutive tasks: a) region of interest delimitation within a

167    white-background-image, by locating a black painted square using an  adapt-

168    ing threshold (Otsu, 1979) for each colour spectrums; b) fish   segmentation,

169    where they got binary images using adaptive thresholds.      They considered

170             the fish as the biggest connected object, and finally, the image was rotated

171    using the angle between the x-axis and the mayor axis of the ellipse that  had

172    the same second moment as the region;  c) tail fin removing,    by  extracting

173           the fish contour from the binary image and computing Euclidean distances

174             among contour points and the contour’s centroid, the tail fin is considered

175            the highest peak from the respective Euclidean distance plot; and d) shape

176    feature  extraction,  from  the  binary  image  the  fish  body area,  length and

177    height are computed (no detail is provided).      Regression analysis was used

178    for fish mass estimation, which revealed a coefficient of determination   (R2)

179    of 0.99.  Also,  when evaluating their data set,  a mean relative error of    6±

180    3% was obtained when comparing the value measured by a weighing    scale.

181             Regarding measured lengths and heights, the error for length ranged from

182    0.2% to 2.8% with a mean error of 1.2 ± 0.8% and the height error    ranged

183            from 0.4 to 3.7% with a mean error of 1.5%.

184	Additionally, some commercial systems are available for fish length mea-

185    suring,  counting  and  weighting during rearing.	Systems as Fish Counter

186      from  Vaky  (VAKI,  2014)  counts,  weighs  and  sizes  by  fish  gliding  through

187    channels and has a 99% accuracy in fish counting.  The Riverwatcher  Fish

188    Counter (VAKI, 2016), is a submersible system able to measure fish  length

6

189    and  swimming  speed.  For  that purpose,  a measuring component is  placed

190               where every fish in the river could pass,  capturing  side view images for fish

191    length estimation. This system shows a precision of 98% and 95% for count-

192    ing and measuring, respectively.  Currently,  the most complete   commercial

193      system is AutoFish Sorting from Northwest Marine Technology (2016),  which

194    is based on fish length estimation, automatically aligns, classifies and   sorts.

195    This system glides the fish through a ramp where upper view images are cap-

196    tured for fish length estimation using computer vision techniques,  achieving

197    a mean accuracy error of 1 mm among total  lengths.

198    1.2. Automatic classification problem

199	An  automatic  fish  classification  system  during  different  growth  stages is

200    desired to improve fish harvest and profitability in farms.  This system could

201    help farmers  to improve classification  and  harvesting.   Ideally,  this system

202    should  be able  to  extract  individual  fish  features,  such  as:  length, mass,

203    weight, sex and skin colour from one fish at a time (Karplus et al., 2003;

204      Zion, 2012).

205	Different technology can be use for an automatic classification.      How-

206    ever, this research is mainly interested in investigating an image   processing

207    approach.  Thus, this problem concerns to have a non-invasive technique    to

208  make specimens to cross throughout a system for individual image processing

209    analysis.  To do this, we visualise three main sequential  steps:

210     a) Fish alignment.  We  need  to  face  some  problems  concerning  fish be-

211	haviour in rearing ponds.  Because, specimens stay in dense shoals that

212	cause occlusion.      Then, we  need to divide and space out the shoal to

213	make every fish pass across a channel.  This step is considered   essen-

214	tial to get an automatic classification system based on accurate fish

215	measurements.

216   b) Fish analysis. In this step we need to establish the number of fish classes

217	and their features that are related to the problem.       Then,  in terms of

218	that,  it’s defined appropriate image acquisition conditions  and   image

219	processing  techniques.

220    c) Fish sorting. This step includes a physical separation based on the num-

221	ber of classes established in the previous  step.

7

222	In a previous work, we developed a prototype to classify rainbow trout in

223    fry, fingerling and table-fish sizes.  To do this, a pyramid prototype to capture

224    downward-view digital images was constructed.  From captured images,  the

225            fish body silhouette is extracted and it is used to approximate a third order

226    regression curve to estimate the fish length (Romero et al.,  2015).

227	Then, based on such experience and interest, we continue our research by

228    designing,  developing and evaluating a prototype based on a passive    tech-

229    nique to make specimens  to cross throughout a channel.        This is done by

230            using the rainbow trout instinctive behaviour in swimming against a water

231    flow, where downward-view images are taken and analysed for length   esti-

232    mation.      By doing this, we believed that it is a measuring non-invasive and

233      efficient technique.

234   2. Materials and methods

235	This section contains materials and methods used for this research.   Sub-

236  section 2.1 depicts our measuring prototype evolution. Subsection 2.2 presents

237    our measuring prototype.  Subsection 2.3 shows our prototype’s installation.

238    Subsection 2.4 describes our experimental sample. Subsection 2.5 details our

239    image processing for fish detection and measuring.  Finally,  Subsection   2.6

240    indicates our measurement analysis.

241   2.1. Protype’s background

242	Our measuring prototype has been evolving since 2013 towards a Bsc

243    Thesis (Miranda, 2014), where a pyramidal prototype was proposed to   cap-

244    ture upward view image to classify the rainbow trout based on estimated fish

245             length (Romero et al., 2015, 2014), until the prototype we  introduce here.

246    Figure 1 depicts our measuring prototype  evolution.

247    2.2. Measuring prototype

248	In fish hatcheries, the pond’s shape and hydraulic conditions influence

249    the distribution, position, growth, metabolism and behaviour of rearing   fish

250    (Ross et al., 1995; Duarte et al., 2011; Staley and Mueller, 2000).       Taking

251            this into consideration and attending problems related to this research (see

252    Subsection 1.2), we have designed a four-level prototype that can be set aside

253            a fish earth pound,  which receives the pond natural water flow and avoids

254               occlusion as one fish is fed at a time (see Figures 2 and 3).  Additionally,

255    to promote the fish displacement into the prototype’s channel, we are   using

8

Figure 1: Evolution of the proposed prototype to measure the rainbow trout’s length.

256             their instinctive behaviour in swimming against a water flow (Frost  et al.,

257    2013; Woynarovich et al., 2011; Blanco, 1993).

Figure 2: Components of the measuring prototype to estimate the rainbow trout’s length (lateral view):  Water supply (#1), vision (#2), channelling (#3), Fish feeder (#4),   water

outlet (#5), illumination (#6), distribution (#7), outlet channel (#8), water diffuser (#9) and Water switcher (#10).

258	Seven components integrates our prototype:  water switcher, water   sup-

259    ply,  channelling,  illumination,  fish deposit,  distribution and vision.   These

260    components are described below.

261    2.2.1. Water switcher

262	As we mentioned before, the rainbow trout is a fish that swims  upstream

263    and downstream in a water flow (Staley and Mueller, 2000).  Taking this into

264    consideration, we thought that an intermittent water flow could promote  fish

265      displacement upstream, this is done by a water switcher component (#10   in

266    Figure 2) installed in the fourth level of our prototype.  This a  20x20x20 cm

9

Figure 3: Upward view of our prototype to estimate the rainbow trout’s length. The componentes are vision (#2), channelling (#3), fish deposit (#4), distribution (#7),  water

diffuser (#9) and water switcher (#10).

267    component  located  in  the  fourth  level  of  the measuring prototype.	Two

268          floodgates have been included to regulate the water flow into the prototype

269    by opening and closing them  manually.

270    2.2.2. Water supply

271	This  component collects  and  distributes the water flow into  the   chan-

272    nelling component, by means of a water tank and five water diffuser  arrays.

273	The water tank (#1 in Figure 2),  is located in the third level of the

274    measuring prototype.  It is a trapezoidal shape with a 1452.5 cm2  base   area

275    and height decided by (Davila, 1974).

. 	QE	 .2

√

Hight water tank =

8A1 ∗ Cd

√

2g + A2 ∗ Cd	2g

(1)

276	Where QE  is the inlet water flow (0.000852 m3/s, estimated with the

277    volumetric method ),  A1  is the central diffuser area (0.000506 m),  A2  is    a

278    lateral diffuser area (0.0020 m), Cd  is the spending ratio (≈ 0.6) and g is the

279   gravity constant (9.81 m/s2). With those values, Equation 1 gives us a water

280    flow height of 0.0075 m, which remains constant for the water tank inlet and

281    outlet parameters.  We decide a water tank high of 15 cm to handle a  higher

282    inlet water flow QE , hence the water tank capacity is 0.02178 m3.

283	The five water diffuser arrays, located at the bottom of the water tank (#9

284    in Figure 2), are thought to generate a water flow into the prototype. This

285  is possible by feeding water at different heights (8, 11 and 11.5cm) along ’L’

286    shape pipes, placed on every side of the distribution component’s walls   and

287    at the channel’s centre.  Lateral and central pipes are 2.54 cm and   5.08 cm,

288      respectively.

10

289   2.2.3. Channelling component

290	The channelling component is located in the second level of the prototype

291               (#3 in Figure 2) and directs the water flow in a linear path up to an outlet

292      duct (#5 in Figure 2).  This is rectangular chamber and it has the    following

293    dimensions 0.15x0.14x1 m in height, width and length, respectively.

294	The main function for this component is to lead the rainbow trout  during

295      their swimming upstream from the fish deposit to an exit in the   distribution

296     component.

297     2.2.4.  Illumination component

298	Illumination plays an important role in the overall efficiency and accu-

299       racy  of  any  image  processing  system,  because  it  greatly  affects  the quality

300    of captured images (Gonzalez and Woods, 2008).   An appropriate    lighting

301    system can reduce the effect of reflection, shadow and some noise,    thereby

302       reducing the required time for image processing (Szeliski, 2011).  The    aspects

303    to be considered in an illumination system are location, lamp type and colour

304    quality (Saberioon et al., 2016).

305	Then, an illumination component is integrated in the prototype and it is

306           located at the first level (#6 in Figure 2).  In fact, this is a source of diffuse

307    light  at  the  bottom  of  the  channelling  component  that  increases contrast

308    in  the  acquired  digital  image,  highlighting  the  rainbow trout body.	The

309    light source height was defined experimentally (Romero et al., 2015) and    it

310    consists of four 90 lm lamps and they are allocated in rectangular base   with

311    a dimension of 1.8x0.14x0.16 m in height, width and length, respectively.

312    2.2.5. Fish deposit

313	The fish deposit component, located in the third level, was designed    for

314    the rainbow trout welfare by easily setting the fish down into the prototype’s

315    channel (#4 in Figure 2).

316	The fish deposit is trapezoidal shape and its dimensions are 40x30 cm  at

317    top and 30x15 cm at the bottom.

318    2.2.6. Distribution component

319	As we mentioned before, this research is based on the rainbow trout

320    instinctive behaviour in swimming against the current.  Hence, an   upstream

321    water flow permits the specimen to cross the channelling component leading

322     it to an exit.

11

323	Therefore, the distribution component is integrated as the exit part of

324  the prototype, that is located in the second level and contains two exits (#7

325    in Figure 2).  This component has a W  shape, providing two outputs for fish

326     separation (#8 in Figure 3).

327    2.2.7. Vision component

328	This  component is integrated into the prototype to capture  and   process

329      digital images by the time the rainbow trout pass through its    channel.

330	The capturing process is performed through a digital camera    physically

331      located  in the  prototype’s third  level  at the  central part  of the channelling

332              component (#2 in Figure 2).  This is a 2D LifeCam Studio camera able to

333    capture RGB images with a maximum resolution of 1920x1080 pixels, 30 fps

334      Microsoft (2014).

335	That digital camera is situated over a physical region that we called re-

336    gion of interest, which is a 40x12 cm area at the base of the channel   retro-

337    illuminated by the illumination  component.

338	Taking into account the region of interest size and the geometry based

339    on a projection camera model (Pears et al., 2012; Szeliski, 2011), the  digital

340    camera is 32 cm high from the channelling component base, allowing a field

341      of  view  widely  enough  to  capture  the  40 cm  region  of  interest  length (see

342    Figure 4).

Figure  4:   Geometry  based  on  a  projection  camera  model.   Where  xc   is  the  position  of  a point in the camera’s image plane,  f is the distance of the image plane to the camera     centre (focal length), Xc is a 3D world point through the  camera  centre  and  Zc is  the  direction  of  the  principal  axis  of  the  lens  that encodes  depth  from  the camera.

343	Additionally,  limiting the image section to be processed,  two   reference

344    walls (made of opaque glass) were allocated inside the channelling   compo-

345              nent along the region of interest at 80o, which was  calculated using basic

346    trigonometry (see Figure 4):

12

b

Cos A =

c

(2)

347	Where  b = 31.385 cm and  c = 32.9074 cm,  considering  a  3.85 mm focal

348    length and a 4.8 mm size sensor.

349	The vision component has a logical part for image processing, which is the

350    essence for this research. Therefore, two main functions were programmed in

351    Matlab for fish detection and measuring on captured images, those functions

352    are detailed in Section 2.5.

353    2.3. Prototype setup

354	Two steps are required to set up our measuring prototype in the experi-

355    mental farm to measure a rainbow trout   specimen.

356    a) Prototype assembly

357	The experimental farm has a linear arrangement of grounds ponds in

358	levels, which allows to feed a water flow in cascade among them.   Then,

359	the measuring prototype was  placed between two  consecutive ponds    at

360	convenient distances to provide it with electricity and water. Additionally,

361	a personal computer was installed over the measuring prototype for image

362	acquisition.

363	To  branch off the water flow towards the measuring    prototype,  an  hy-

364	draulic installation was  required,  by  connecting an array of PVC    ducts

365	from a pond water fall to the prototype’s water switcher as it is illustrated

366	in Figure 5.

Figure 5:  Prototype to measure rainbow trout’s length installed in   situ.

367   b) Water flow measuring

368	The water flow fed into the prototype was measured using the volumetric

369	method (Woynarovich et al., 2011), which measures the required time   to

13

| Fish # | length (cm) | Width (cm) | Thickness (cm) |
| --- | --- | --- | --- |
| 1 | 25.00 | 6.00 | 2.80 |
| 2 | 28.00 | 6.50 | 3.00 |
| 3 | 28.50 | 6.70 | 3.00 |
| 4 | 27.50 | 5.90 | 2.70 |
| 5 | 26.50 | 6.50 | 2.90 |
| 6 | 28.00 | 6.30 | 2.90 |
| 7 | 27.00 | 6.00 | 2.80 |
| 8 | 25.70 | 5.50 | 2.50 |
| 9 | 26.30 | 6.00 | 2.80 |
| 10 | 26.00 | 5.70 | 2.50 |
| Mean | 26.85 | 6.11 | 2.79 |

Table 1: Summary data set obtained from experiments with our experimental  procedure.

370	fill a container of known volume.  Using this method, a 0.851 L/s   water

371	flow was recorded causing a water depth of 2.5 cm along the channelling

372	component.  The water flow speed was also estimated by sampling the re-

373	quired time for an object to cross along the prototype’s channel, obtaining

374	a speed of 14.004  m/s.

375   2.4. Experimental sample

376	For this experiment ten table-size rainbow trout were used.  These   spec-

377    imens were visually selected by the farmer in the experimental farm.     After

378    that,  every specimen was  firstly measured by  the prototype.   Once a     fish

379           reached an exit, their length, width and thickness were manually measured

380    using a Vernier scale (see Table 1).

381	An indirect manipulation was performed by feeding one rainbow trout  at

382      the time into the fish deposit.  Then, an intermittent water flow was   created

383    into the channel,  this was  done by  opening and closing the water   switcher

384    floodgates at intervals of 1.6 s. Then, a downward-view video recording was

385               obtained for every fish passing through the prototype,  so that it gives ten

386    videos, which length corresponds to the fish swimming time to exit the   pro-

387    totype.  These videos were recorded using a Matlab script based on its image

388    acquisition toolbox.  This is the raw data used for our image processing   ap-

389    proach.

14

390   2.5. Image processing

391	This  section  describes  our  image  processing  approach  for  fish detection

392    and measuring.

393    2.5.1. Fish detection

394	For fish detection into the prototype it was defined named region of inter-

395    est, which in fact is the covered area within the camera’s field of view at the

396    channel’s base high. As observed in Figure 6, two areas have been defined to

397      assist this process:  Start indicator and End  indicator.

| Input area | Start indicator | End indicator |
| --- | --- | --- |

Figure 6: The three areas that integrated in the prototype’s region of interest that help to detect a fish into the channel.

398	The complete rainbow trout detection process is shown in Figure  7.

Begin

Obtain reference frames for start & end  indicators

No

Exist a new frame?

End

Yes

Extract the next frame

Compute SDAbegin

Yes

SDAbegin  >= 15%

Compute SDAEnd

No

Yes

SDAEnd<= 10%

Fish  detected

No

Estimate length

Figure 7: Process to detect a rainbow trout into a captured image while it crosses the prototype’s region of interest.

15

399	First  a  reference  frame  is  captured,  which  is  an  image  of  the  region of

400    interest containing only the water flow, which is compared against every new

401    captured frame by computing the square difference addition  (SDA):

.N

i=1(Img1[i] − Img2[i])2

(3)

N

402	Where Img1 and Img2 are [length x width] images, represented as  row

403    vectors, and N = length ∗ width.

404	Then,  given  a  new  frame,  the  start  indicator  SDA  is  calculated.	If

405              (SDAbegin) is greater than or equal to 15%,  the end indicator SDA  is ob-

406           tained (SDAend) and a fish is detected, if (SDAend) is less than or equal to

407    10%. Hence, the fish measuring function is executed.

408	Thresholds 15% and 10% were obtained experimentally by computing the

409      mean x¯ and standard deviation Sx¯  from ten rainbow tout head images.  Then,

410      threshold for the standard indicator area equals x¯ + 2Sx¯, while threshold for

411      the end indicator area equals x¯ − 2Sx¯

412	Note that fish counting is possible as it equals the number of detected

413    fish using this approach.

414    2.5.2. Fish measuring

415	For fish measuring, standard image processing algorithms have been used

416    to get the fish body silhouette (Gonzalez and Woods, 2008; Szeliski,  2011),

417   which is given to our statistical approach to estimate the fish length (Romero

418    et al., 2015). The detailed process is as follows.

419               a)  Convert the input RGB image to grayscale, using a standard weighted

420	addition of its components: 0.2989 ∗ R + 0.5870 ∗ G + 0.1140 ∗ B.

421           b)  A binary image is obtained using a 0.7058 threshold, which was exper-

422	imentally  calculated  based  on  a  rainbow  trout  histogram. Taking  into

423	account that the histogram is bimodal, where the first mode is the fish

424	body.

425	c) The rainbow trout body is emphasized by using three morphological oper-

426	ations: dilating, opening and closing (Gonzalez and Woods, 2008), which

427	operation masks are shown in Figure  8.

428    d)  Connected components are labeled using a N8  neighbourhood   (Szeliski,

429	2011) and the second largest connected component is considered as the

430	fish body silhouette.

16

Figure 8: Operation masks to emphasize the rainbow trout’s body, a) dilating, b) opening

and c)closing.

431	e) Taking the fish body silhouette as an (x, y) points set, a fish length is

432	estimated by interpolating a third order regression curve (Romero et    al.,

433	2015).

434	f)  To convert the regression curve units from pixels to millimetres, the cam-

435	era’s model intrinsic and extrinsic parameters are considered (Pears et al.,

436	2012; Zhang, 2000):

| X  |  x  |  αx | s | x0 | −1 |  r1 |
| --- | --- | --- | --- | --- | --- | --- |
| Y  = | λ  y  |  0 | αy | y0 |  |  r2 |





1	1	0	0	1

−1



t

(4)

437	Where [X, Y, 1] are coordinate points in millimetres;  [x, y, 1] are  coor-

438	dinate points in pixels;  λ is an arbitrary scale factor;  [r1, r2, t] are  the

439	extrinsic  parameters,  rotation  and  translation,  which  relates  the world

440	coordinates system to the camera’s coordinate system; (x0, y0) are the

441	origin point coordinates from a reference image; (αx, αy ) are scale factors

442	according to the reference image for the x and y axes;    and s is the skew

443	between the two images axes.

444      g)  Finally,  the rainbow trout length is computing by  adding the   Euclidean

445	distance among the set of points within the regression  curve.

446   2.6. Measurement analysis

447	According to Lehmann and Casella (1998), root mean square error, mean

448    absolute error and mean relative error are computed for length  measurement

449      analysis.

450	The root mean square error (RMSE) represents the sample standard    de-

451    viation of the differences between estimated values and measured values:

RMSE =

.

.N

i=1[ELi − MLi]2

N

(5)

17

452	The mean absolute error (MAE) shows how close forecasts or predictions

453    are to eventual outcomes:

MAE =

i=1 |ELi − MLi|

.N

N

(6)

454	The mean relative error (MRE) indicates how large the length error is   in

455      relation to the length  measured.

MRE =

.N

i=1

.|ELi−MLi| .

MLi

N

(7)

456	Where ELi is the i-th fish estimated length; MLi is the i-th manual fish

457    measured length; and N is the number of estimated   lengths.

458   3. Results and discussion

459	As mentioned in Section 2.4, one video per fish was recorded when it

460    passes throughout the prototype.  Then,  those videos were analysed    frame

461            by frame from the fish entrance to exit to measure every fish detected (see

462    Section 2.5).  As indicated by Armstrong and Collopy (1992), a performance

463    evaluation was  done by  computing  mean square,  mean absolute and  mean

464    relative errors using equations 5, 6 and 7, respectively.

465	From  this experiment, two  main results are obtained, fish detection   and

466    measuring.

467	To evaluate our fish detection process (Section 2.5.1), we visually counted

468  the number of times that a fish crosses throughout the region of interest mark-

469    ing upstream and downstream swimming. A total of forty-six fish swimming

470    were counted, each one with different number of images, implying   different

471      swimming  speed.    Then,  these  forty-six  fish  swimming  were  processed  by

472    our fish detection approach, from which, in 244 images a rainbow trout  was

473    detected and measured (Section 2.5.2), an illustration is shown in Figure    9.

474           Table  2 summarises our fish detection results,  where it is observed that in

475    one occasion specimens eight and nine were not detected.

476	Comparing our manual counting against our fish detection module re-

477               sults,  we  are achieving a weighted mean of 96.7% successful fish detection,

478    from that comparison we also observed that our fish detection module is   ro-

479    bust in detecting a fish regardless its swimming orientation (downstream and

480     upstream).

18

| Fish # | Swim | Orientation | # Images | Exit taken |
| --- | --- | --- | --- | --- |
| 1 | us ds |  | 3 11 | No |
| 2 | us |  | 5 | Yes |
| 3 | us ds us |  | 2 11 5 | No |
| 4 | us ds |  | 2 1 | Yes |
|  | us |  | 2 |  |
|  | ds |  | 6 |  |
|  | us |  | 2 |  |
| 5 | us ds |  | 1 8 | No |
|  | us |  | 2 |  |
|  | ds |  | 4 |  |
|  | us |  | 3 |  |
|  | ds |  | 19 |  |
|  | us |  | 9 |  |
|  | ds |  | 18 |  |
| 6 | us ds |  | 5 6 | Yes |
|  | us |  | 1 |  |
| 7 | us |  | 2 | Yes |
| 8 | us ds |  | 3 17 | No |
|  | us |  | 2 |  |
|  | ds |  | 2 |  |
|  | us |  | 2 |  |
|  | ds |  | 2 |  |
|  | us |  | 12 |  |
|  | ds |  | Not detected |  |
|  | us |  | 1 |  |
|  | ds |  | 2 |  |
|  | us |  | 1 |  |
|  | ds |  | 10 |  |
| 9 | us ds |  | 1 Not detected | No |
|  | us |  | 3 |  |
|  | ds |  | 3 |  |
| 10 | us ds |  | 2 6 | No |
|  | us |  | 3 |  |
|  | ds |  | 7 |  |
|  | us |  | 2 |  |
|  | ds |  | 6 |  |
|  | us |  | 5 |  |
|  | ds |  | 24 |  |

Table 2: Summary of experimental data set highlighting the number of times that a rainbow trout pass through the region of interest, upstream (us) and downstream (ds). Note some cases where the fish is upstream oriented, although it is swimming downstream.

19

Figure 9: An illustration when a rainbow trout is detected and measured by our system. In this figure one can note its length depicted by a green line and also the start and end indicators.

481	Our experimental results for rainbow trout length estimation into a  water

482    flow are summarised in Table 3, where a 1.413 cm mean absolute error   and

483    a 5.206% mean relative error are observed.  Although,  this performance    is

484    comparable with the state of the art, some large errors are observed and  now

485      we  discuss two  main causes:

486    a)  Freedom of movement:  Because of the channel’s width (#3 in Figure 2)

487	and the extreme rainbow trout’s flexibility, its body can be freely con-

488	tracted.  This contraction occludes part of the fish body, then, the   ad-

489	justed regression curve does not consider the complete body as we can

490	see in Figure  10.

Figure 10:  A sample when the rainbow trout’s body is  contracted.

491     b)  Trade off between water level and lighting:  We observed that a high water

492	level reduces contrast between the rainbow trout’s body and the illumi-

493	nated channels floor (#3 in Figure 2), causing that the fish body could

494	not be detected completely, particularly the trout’s caudal fin as shown

495	in Figure 11.

Figure 11: A sample when the fish body could not be detected completely due to high contrast.

496	Additionally, from this experimental research, a relevant discussion point

497    is the channelling component in our prototype’s design. Because of its width

20

| Fish # | R.L. (cm) | E.L. (cm) | Mean | Grand Mean | RMSE (cm) | Mean RMSE (cm) | MAE (cm) | Mean MAE (cm) | MRE (%) | Mean MRE (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 25.0 | 3 11 | 24.965 24.100 | 24.532 | 0.453 0.931 | 0.692 | 0.412 0.900 | 0.656 | 0.016 0.036 | 2.624 |
| 2 | 28.0 | 5 | 28.850 | 28.850 | 1.410 | 1.410 | 1.340 | 1.340 | 0.048 | 4.786 |
| 3 | 28.5 | 2 11 | 28.809 27.934 | 25.853 | 0.346 0.875 | 3.348 | 0.309 0.566 | 2.853 | 0.011 0.020 | 10.010 |
|  |  | 5 | 20.816 |  | 8.824 |  | 7.684 |  | 0.270 |  |
| 4 | 27.5 | 2 1 | 27.119 20.769 | 25.430 | 0.387 6.731 | 2.155 | 0.381 6.731 | 2.070 | 0.014 0.245 | 7.530 |
|  |  | 2 | 26.005 |  | 1.872 |  | 1.496 |  | 0.054 |  |
|  |  | 6 | 26.723 |  | 0.780 |  | 0.777 |  | 0.028 |  |
|  |  | 2 | 26.537 |  | 1.006 |  | 0.964 |  | 0.035 |  |
| 5 | 26.5 | 2 8 | 28.376 26.723 | 26.978 | 1.776 1.241 | 1.039 | 1.776 1.045 | 0.898 | 0.067 0.039 | 3.370 |
|  |  | 2 | 27.007 |  | 0.437 |  | 0.407 |  | 0.015 |  |
|  |  | 4 | 26.299 |  | 0.411 |  | 0.363 |  | 0.014 |  |
|  |  | 3 | 27.162 |  | 1.240 |  | 0.844 |  | 0.032 |  |
|  |  | 19 | 26.739 |  | 0.919 |  | 0.803 |  | 0.030 |  |
|  |  | 9 | 27.696 |  | 1.280 |  | 1.096 |  | 0.041 |  |
|  |  | 18 | 25.819 |  | 1.011 |  | 0.847 |  | 0.032 |  |
| 6 | 28.0 | 5 6 | 28.533 26.462 | 27.794 | 0.623 2.635 | 1.215 | 0.533 2.034 | 0.985 | 0.019 0.073 | 3.520 |
|  |  | 1 | 28.388 |  | 0.388 |  | 0.388 |  | 0.014 |  |
| 7 | 27.0 | 2 | 28.782 | 28.782 | 1.902 | 1.902 | 1.782 | 1.782 | 0.066 | 6.600 |
| 8 | 25.7 | 3 17 | 26.490 24.986 | 25.489 | 0.928 1.072 | 1.179 | 0.790 0.988 | 1.041 | 0.031 0.038 | 4.050 |
|  |  | 2 | 27.133 |  | 2.304 |  | 1.805 |  | 0.070 |  |
|  |  | 2 | 25.574 |  | 0.254 |  | 0.221 |  | 0.009 |  |
|  |  | 2 | 27.047 |  | 1.527 |  | 1.347 |  | 0.052 |  |
|  |  | 2 | 25.171 |  | 0.576 |  | 0.529 |  | 0.021 |  |
|  |  | 12 | 25.558 |  | 0.632 |  | 0.462 |  | 0.018 |  |
|  |  | 1 | 26.070 |  | 0.370 |  | 0.370 |  | 0.014 |  |
|  |  | 2 | 23.228 |  | 2.490 |  | 2.472 |  | 0.096 |  |
|  |  | 1 | 25.792 |  | 0.092 |  | 0.092 |  | 0.004 |  |
|  |  | 10 | 23.330 |  | 2.721 |  | 2.370 |  | 0.092 |  |
| 9 | 26.3 | 1 3 | 27.494 23.765 | 25.640 | 1.194 3.309 | 1.885 | 1.194 2.535 | 1.578 | 0.045 0.096 | 6.000 |
|  |  | 3 | 25.661 |  | 1.151 |  | 1.004 |  | 0.038 |  |
| 10 | 26.0 | 2 6 | 25.856 25.068 | 25.391 | 0.5759 0.9884 | 1.068 | 0.558 0.932 | 0.929 | 0.021 0.036 | 3.570 |
|  |  | 3 | 25.848 |  | 0.7709 |  | 0.758 |  | 0.029 |  |
|  |  | 7 | 23.092 |  | 3.5497 |  | 2.946 |  | 0.113 |  |
|  |  | 2 | 26.064 |  | 0.2081 |  | 0.198 |  | 0.008 |  |
|  |  | 6 | 25.687 |  | 0.8003 |  | 0.590 |  | 0.023 |  |
|  |  | 5 | 26.228 |  | 0.6895 |  | 0.561 |  | 0.022 |  |
|  |  | 24 | 25.285 |  | 0.9623 |  | 0.889 |  | 0.034 |  |
|  |  |  |  |  | Mean | 1.589 |  | 1.413 |  | 5.206 |

Table 3: Estimated lengths summary with respective estimation errors when using our measuring prototype.

21

498             and to be positioned without any slope, the fish can freely swim along the

499               channel and it is able to change its swimming direction (downstream and

500    upstream).      Although,  our measuring prototype has been proved robust in

501    estimating the rainbow trout’s length in both directions,  we  are     expecting

502    the fish to be detected and measured once.  Furthermore, such a freedom   to

503    swim upstream or downstream into the prototype’s channel could avoid   the

504    fish to exit it.      From  our ten rainbow trout specimens only four got an exit

505    channel by itself, the rest were taken out   manually.

506	It is also important to control the fish orientation and position when

507            passing across the channel, because the caudal fin is absolutely thin that if

508    it is parallel to the camera, the caudal will be visualised as a line and it   will

509    be lost when the image is processed.

510	Above all, we found that our measuring prototype is suitable for fish

511    detection and to estimate a fish length in presence of different fish orientations

512    and positions when swimming into water flow.

513   4. Conclusions

514	In this document,  we  have  presented state  of the art results in   rainbow

515    trout’s length estimation within a water flow using image  processing.

516	For    that purpose, we have designed, implemented and evaluated a novel

517  prototype, which is taking advantage of the rainbow trout’s swimming against

518    the water flow to measure it.  An essential part of this prototype is the vision

519           component,  which based on downward-view images to detect when a fish

520    crosses a pre-established control point.  Then, a third order regression  curve

521    is approximated to the fish’s silhouette to estimate its   length.

522	So far, our experimental results are scoring a 1.413 cm mean absolute error

523    (MAE) using our novel prototype to measure the rainbow trout’s length. This

524      is an encouraging preliminary result that is comparable with the state of  the

525    art and it is useful to draw different venues for future   work.

526    Acknowledgements

527	Jose Manuel Miranda, author, thanks the Mexican Council for Science

528    and Technology for his scholarship (CVU 634478).

22

529    Appendix  A.  Alternate prototypes

530	In this section we present our alternative prototypes to measure rainbow

531    trout’s length in a water flow.

532	Prototype A

533    The first prototype, shown in Figure A.12, consists of the following  compo-

534    nents:  fish tank inlet, channelling, reducer, output channel, water   diffusers,

535    gate, vision, illumination.  The water diffuser component supplies water  and

536  generates  flow  inside  of  the  prototype.   The  fish  is  fed  inside  the  inlet  fish

537    tank, which has a water flow.      The fish is directed throughout a reducer to

538    permit only one fish at a time to pass across the channelling component. The

539           vision component measures a fish length in the field of view of the camera

540    and switches the gate according to the length.  Only two output channels  are

541    considered in this  prototype.

Figure A.12: First prototype to measure rainbow trout’s length using image processing.  We  thought the integration of two  output channels    only.

542	Prototype B

543    The second prototype, shown in Figure A.13, consists of similar components

544    and functionality as Prototype A, but it has two  differences.      The first dif-

545      ference is that it could have more than two gates and output channels.     The

546    second difference is the position of gates and output channels, which in   this

547          case are lateral side, intended to integrate two or more output channels.

548	Prototype C

549    The third prototype,  shown in Figure A.14,  is an immerse system into    the

550    rearing fish pond. This prototype consists of a flexible mesh, gate, containers,

551    channelling and a vision component.  For  this prototype, a farmer needs    to

552   bury away the fish toward a pond corner with a flexible mesh, then, the fish is

553    expected to swim across the channelling component.  The vision  component

554    captures images for fish length estimation.  Based on the fish length, a    gate

23

Figure A.13: Second prototype to measure rainbow trout’s length using image processing. This prototype explores the integration of two or more output channels.

555    is switched to the corresponding container.  Note that this proposal needs  to

556    be water proof.

Figure A.14: Third prototype to measure rainbow trout’s length using image processing. This prototype explores an immerse system into the rearing pond.

557	Prototype D

558             A fourth prototype, shown in Figure A.15, consists of a system that glides

559    across a channel the fish to measure its length. Components of this prototype

560    are:  fish tank, channelling, vision, gate and output channel.  The fish is   fed

561    into the fish tank where one by one is glided across the channelling   compo-

562    nent.        The vision component capture images and estimate the fish length.

563            The gate will be positioned based on the decision of the vision component

564                and the fish enter in the corresponding container.   In this system the fish

565          tank is positioned at a height h above the ground and does not consider the

566    rainbow trout to swim against a water  flow.

567  References

568     Abdullah,	N., Mohd Rahim,	M., Amin,	I..  Measuring fish length  from

569	digital  images  (FiLeDI);  volume  403  of  ACM  International Conference

24

Figure A.15: Fourth prototype to measure the rainbow trout’s length. This system pro- poses to glide the fish across a   channel.

570	Proceeding Series. p. 38–43.

571    Armstrong,  J., Collopy,  F.. Error measures for generalizing about   forecast-

572	ing methods:  Empirical comparisons. International Journal of Forecasting

573	1992;8(1):69 – 80.

574    Beddow,   T., Ross,   L.G..  Predicting biomass of atlantic salmon from mor-

575	phometric lateral measurements.      Journal of Fish Biology 1996;49(3):469 –

576	482.

577    Blanco,  M..  La trucha:  cria industrial.  2nd ed.  Acribia S. A., 1993.

578    Costa,   C., Antonucci,   F., Boglione,   C., Menesatti,   P.,  Vandeputte,   M.,

579	Chatain,	B..  Automated sorting for size,  sex and skeletal anomalies   of

580	cultured seabass using external shape analysis.  Aquacultural  Engineering

581	2013;52:58 – 64.

582     Costa,	C., Loy,	A., Cataudella,	S., Davis,	D., Scardi,	M..   Extract-

583	ing fish size using dual underwater cameras.   Aquacultural     Engineering

584	2006;35(3):218 – 227.

585      Davila,   G.S..  Hidr´aulica General Vol. 1.  1st ed.  LIMUSA, 1974.

586    Dowlati,  M., de la Guardia,  M., Dowlati,  M., eid Mohtasebi,  S..   Applica-

587	tion of machine-vision techniques to fish-quality assessment. TrAC Trends

588	in Analytical Chemistry 2012;40:168 –   179.

589      Duarte,   S., Reig,   L., Masalo´,   I., Blanco,   M., Oca,   J..  Influence of tank

590	geometry and flow pattern in fish distribution.  Aquacultural   Engineering

591	2011;44(2):48 – 54.

25

592      FAO, . Food and agriculture organization of the united nations: Aquaculture.

593	Retrieved from http://www.fao.org/aquaculture/en/; 2016.  Last  access

594	26/04/2016.

595      Frost,   A.J., Thomson,   J.S., Smith,   C., Burton,   H.C., Davis,   B.,   Watts,

596	P.C.,  Sneddon,	L.U..	Environmental  change  alters  personality  in the

597	rainbow trout, oncorhynchus mykiss. Animal Behaviour 2013;85(6):1199 –

598	1207.

599    Gonzalez,   R.C., Woods,   R.E..  Digital Image Processing.  3rd ed. Prentice

600	Hall, 2008.

601     Hsu,	H..        Method for calculating distance and actual size of shot object.

602	2008.  US Patent App. 11/716,466.

603     Ibrahim,	M.Y., Sultana,	S..   Study on fresh fish sorting techniques.    In:

604	IEEE International Conference on Mechatronics. 2006. p. 462–467.

605 Karplus,  I., Gottdiener,  M., Zion,  B.. Guidance of single guppies (poecilia  606	reticulata) to allow sorting by computer vision.  Aquacultural Engineering 607	2003;27(3):177 – 190.

608    Lehmann,   E., Casella,   G..  Theory of Point Estimation.  2nd ed.   Springer,

609	1998.

610   Mathiassen,   J.R.,  Misimi,    E.,  Bondø,    M.,  Veliyulin,    E.,  Østvik,    S.O.. 611	Trends  in application  of imaging technologies to inspection  of fish   and fish 612	products.  Trends  in Food Science and Technology 2011;22(6):257 –     275.

613    Microsoft,	C..	Lifecam     studio.	Retrieved     from 614	http://www.microsoft.com/hardware/en-us/p/lifecam-studio;	2014. 615	Last access 26/05/2016.

616 Miranda, J.M.. Prototipo de un sistema clasificador de la trucha arco´ıris 617	utilizando un modelo estad´ıstico de su longitud obtenido de ima´genes 2d. 618	Bachelor of Science in the Autonomous University of the State of Mexico; 619	2014.

620    Montero,  R.. National aquaculture sector overview (mexico). Retrieved from

621	http://www.fao.org/fishery/countrysector/naso  mexico/en;  2013.  Last ac-

622	cess 15/3/2016.

26

| 623 | Northwest Marine Technology, C.. Autofish sorting. | Retrieved |
| --- | --- | --- |
| 624 | from http://www.nmt.us/products/afs/afs.shtml; 2016. | Last access |
| 625 | 24/06/2016. |  |

626    Odone,  F., Trucco,  E., Verri,   A..  A trainable system for grading fish  from

627	images.  Applied  Artificial  Intelligence 2001;15(8):735–745.

628    Otsu,   N..  A threshold selection method from gray-level histograms.   IEEE

629	Transactions on Systems, Man, and Cybernetics 1979;9(1):62–66.

630    Pears,   N., Yonghuai,   L., Bunting,   P..  3D Imaging, Analysis and Applica-

631	tions. Springer, 2012.

632  Romero,  M., Miranda,  J., Montes,  H., Acosta,  J..  A statistical measuring  633	system for rainbow trout.   In:  In proceedings on the    International Con- 634	ference on Image Processing, Computer Vision and   Pattern Recognition. 635	2014. p. 384 – 390.

636 Romero, M., Miranda,  J.M., Montes-Venegas,  H.A..  Measuring Rainbow  637	Trout by Using Simple Statistics; Elsevier Inc.  Emerging Trends in Image 638	Processing, Computer Vision and Pattern Recognition. p. 39 –   53.

639  Ross,  R.M., Watten,  B.J., Krise,  W.F., DiLauro,  M.N., Soderberg,  R.W..  640	Influence of tank design and hydraulic loading on the behavior, growth, 641	and metabolism of rainbow trout (oncorhynchus    mykiss).  Aquacultural 642	Engineering 1995;14(1):29 – 47.

643   Saberioon,   M.,  Gholizadeh,   A.,  Cisar,    P.,  Pautsina,    A.,  Urban,    J..  644	Application     of machine vision systems in aquaculture with emphasis on 645	fish: state-of-the-art and key issues. Reviews in Aquaculture 2016;:1–  19.

646  Shafry,   M.R.,  Rehman,   A.,  Kumoi,   R.,  Abdullah,   N.,  Saba,   T..   Filedi   647	framework  for  measuring  fish  lenght  from  digital   images.    International 648	journal of the physical Science 2012;7(4):607 –    618.

649    Staley,   K.,  Mueller,   J..  Rainbow trout (oncorhynchus mykiss).  Fish  and

650	Wildlife  Habitat  Management  Leaflet 2000;190(13):1–11.

651    Szeliski,  R.. Computer Vision. 2nd ed. Springer,  2011.

27

652   Torisawa,   S.,  Kadota,   M.,  Komeyama,    K.,  Suzuki,    K.,  Takagi,    T.. 653	A digital stereo-video camera system for three-dimensional  monitoring of 654	free-swimming pacific bluefin tuna,  thunnus orientalis,  cultured in  a net 655	cage. Aquat Living Resour 2011;24(2):107–112.

656     VAKI,  .	Vaki   aquaculture  systems  ltd:	Biomass  daily.	Retrieved

657	from  http://www.vaki.com/Products/BiomassDaily;  2014.	Last access

658	20/04/2015.

659    VAKI,  .  Vaki  aquaculture systems ltd:  The riverwatcher fish counter.   Re-

660	trieved from http://www.riverwatcher.is; 2016.  Last access  15/06/2016.

661  Viazzi,  S., Hoestenberghe,   S.V., Goddeeris,   B., Berckmans,   D..  Auto-   662	matic mass estimation of jade perch scortum barcoo by    computer vision. 663	Aquacultural Engineering 2015;64:42 – 48.

664   White,   D., Svellingen,   C., Strachan,   N.. Automated measurement of species  665	and length of fish by computer vision.  Fisheries   Research 2006;80(2–3):203 666	– 210.

667  Woynarovich,   A.,  Hoitsy,   G.,  Moth-Poulsen,   T..   Small-scale rainbow   668	trout farming.  Food and Agriculture Organization of the   United Nations 669	2011;Retrieved  from http://www.fao.org/docrep/015/i2125e/i2125e.pdf.

670   Zhang,   Z..   A  flexible  new  technique  for  camera  calibration.   IEEE  trans- 671	actions  on  pattern  analysis  and  machine  intelligence   2000;22(11):1330  – 672	1334.

673       Zion,   B..  The use of computer vision technologies in aquaculture – a   review.

674	Computers and Electronics in Agriculture 2012;88:125 – 132.

675  Zion,  B., Alchanatis,  V., Ostrovsky,  V., Barki,  A., Karplus,  I.. Real-time  676	underwater sorting of edible fish species.  Computers and    Electronics in 677	Agriculture 2007;56(1):34 – 45.

678    Zion,  B., Shklyar,  A., Karplus,  I..  Sorting fish by computer vision.    Com-

679	puters and Electronics in Agriculture 1999;23(3):175 – 187.

680    Zion,  B., Shklyar,  A., Karplus,  I.. In-vivo fish sorting by computer   vision.

681	Aquacultural  Engineering  2000;22(3):165  – 179.

28

# Cap´ıtulo 5

Conclusiones y trabajo  futuro

Este cap´ıtulo presenta los resultados obtenidos en esta investigacio´n, la cual consistio´ en disen˜ar, construir y evaluar un prototipo para medir la longitud de  la  trucha  arco´ıris  dentro  de  un  flujo  de  agua.  Adem´as,  se  establece  el trabajo futuro a seguir en la investigacio´n.

### Conclusiones

En esta tesis se ha propuesto un prototipo para estimar la longitud de la trucha arco´ıris dentro de un flujo de agua usando procesamiento de ima´genes digitales y tomando ventaja de su instinto de nado contra corriente. Para esto, se disen˜o´ y construyo´ un modelo f´ısico para la captura en l´ınea de ima´genes digitales de peces vivos, as´ı como el software requerido para detectar y medir a  los  espec´ımenes.  Debido  a  esto,  se  puede  concluir  que  se  ha  cumplido  el objetivo general propuesto para esta tesis.

Por otro lado, las caracter´ısticas de disen˜o del prototipo propuesto: a) forma lineal rectangular, b) dimensiones rectangulares 15x14x180 cm, c) flujo de agua (0.851 L/s), d) altura de agua (2 cm) y e) intermitencia del flujo de agua (1.6 s), motivaron el nado contra corriente del pez. En este desplaza- miento  se  obtuvieron  ima´genes  de  vista  superior  de  una  secci´on  fija  dentro del componente de canalizacio´n, con las cuales fue posible estimar la longitud del pez de talla adulta usando procesamiento de ima´genes digitales. Con lo

73

CAP´ITULO 5.   CONCLUSIONES Y TRABAJO FUTURO 	74

cual se cumple la meta de ingenier´ıa postulada para esta tesis.

Para el desarrollo de esta investigacio´n se propusieron cinco prototipos para desplazar la trucha arco´ıris dentro de un flujo de agua (Ver Anexo A, Cap´ıtulo 4), de los cuales se eligio´ el disen˜o documentado en el Cap´ıtulo 4 de esta tesis.

Este disen˜o fue propuesto con base en las caracter´ısticas f´ısicas de la trucha arco´ıris  de  talla  adulta,  espec´ıficamente:  el  largo,  ancho  y  tonalidad  de  su cuerpo,  as´ı  como  las  caracter´ısticas  hidra´ulicas  y  el  caudal  medido  in situ (ver Seccio´n 2.1, Cap´ıtulo 4). Adem´as, esta propuesta busco´ desplazar al pez dentro de un flujo de agua tomando como referencia su instinto natural de nado contracorriente.

El prototipo desarrollado consta de siete componentes: alimentador de pe- ces,  iluminacio´n,  visio´n,  distribucio´n,  canalizacio´n;  as´ı  como,  caja  de  inter- mitencia y difusor de agua. La caja de intermitencia genera un flujo de agua intermitente dentro del prototipo. El difusor de agua almacena el agua y la suministra en el componente de distribucio´n de forma directa. El componen- te de canalizacio´n permite al pez nadar contra corriente dentro del flujo de agua generado. El componente de visio´n permite capturar ima´genes de vista superior en una regio´n f´ısica dentro del componente de canalizacio´n (regio´n de inter´es). El componente de iluminacio´n retro-ilumina la regio´n de inter´es para  la  ayuda  de  la  captura  de  ima´genes.  El  alimentador  de  peces  permi- te  introducir  a  los  espec´ımenes  dentro  del  canal,  permiti´endoles  avanzar  a contracorriente.

El prototipo desarrollado se evaluo´ experimentalmente in situ realizando lo siguiente:  (1)  se  genero´  un  flujo  de  agua  dentro  del  prototipo,  alimentando agua dentro de la caja de intermitencia, la cual es introducida al canal a trav´es del difusor de agua. (2) se inicio´ la captura  de ima´genes, obteniendo ima´genes de forma consecutiva de la regio´n de inter´es dentro del componen- te  de  canalizacio´n.  (3)  Se  extrajo  un  peces  del  tanque  de  crianza  de  forma

indirecta  y  se  deposito´  en  el  tanque  de  peces  dentro  del  prototipo.  (4)  Se

genero´  un  flujo  intermitente  dentro  del  prototipo  usando  la  caja  de  inter-

mitencia, para permitir al pez avanzar a contra corriente. (5) Una vez que el  pez  sale  del  prototipo  a  trav´es  de  alguno  de  los  dos  canales  de  salida,  se detiene la captura de im´agenes en el componente de visio´n. Este proceso se repitio´ para el conjunto experimental, obteniendo 10 videos de peces que se desplazan a contracorriente.

CAP´ITULO 5.   CONCLUSIONES Y TRABAJO FUTURO 	75

El prototipo propuesto permite desplazar al pez dentro de un flujo de agua, utilizando el instinto natural de nado contra corriente de la trucha arco´ıris. Sin embargo, la evaluacio´n final de campo demostro´ que el pez puede invertir su orientacio´n para nadar con la corriente o ser arrastrado por esta. Esto nos indico´, que el comportamiento del pez no es controlable.

Utilizando los m´odulos de procesamiento de ima´genes propuestos (ver Seccio´n 2.4,  Cap´ıtulo  4)  se  procesaron  los  videos  capturados  en  la  experimentacio´n fuera  de  l´ınea  para  evaluar  la  medicio´n  de  la  trucha  arco´ıris  dentro  de  un flujo de agua. La deteccio´n exitosa de peces dentro del prototipo de medicio´n alcanza  una  precisi´on  de  96.7 %  y  se  ha  mostrado  robusta  a  cambios  en  la orientacio´n  de  nado  del  pez  dentro  del  canal.  Por  otra  parte,  se  obtuvo  un error  cuadra´tico  medio  de  1.413 cm y  una  gran  media  del  error  absoluto de  5.206 %  en  todas  las  estimaciones  de  longitud  realizadas  (ver  Seccio´n  3, Cap´ıtulo 4), los cuales son valores comparables a lo reportado en la literatura.

### Trabajo futuro

Finalmente,  en  esta  secci´on  se  presenta  una  visio´n  sobre  posibles  l´ıneas  de investigaci´on de trabajo futuro en el problema de medicio´n automa´tica de la trucha arco´ıris, a partir del alcance de esta tesis.

Debido a los problemas encontrados al permitir libertad de movimiento del pez dentro del prototipo, se recomienda redisen˜ar la forma del componente de canalizacio´n. Para esto, se sugiere experimentar una rampa de deslizamiento en forma de ‘V’ en lugar del canal rectangular, con lo cual se espera limitar el  movimiento  de  nado  del  pez,  alinea´ndolo  dentro  del  canal  y  evitando  el cambio de orientacio´n del pez. La forma rectangular del componente de ca- nalizacio´n del prototipo actual fue disen˜ada para experimentar con peces de talla adulta, cuya dimensio´n resulta excesiva para experimentar con peces de talla alev´ın y juvenil. El redisen˜o de la forma del canal, tendr´ıa la ventaja de permitir el flujo de peces de las tallas alev´ın, juvenil y adulto.

Al  cambiar  la  forma  del  componente  de  canalizacio´n,  implicar´ıa  un  ajuste del  componente  de  iluminacio´n  del  prototipo  propuesto  en  esta  tesis,  si  se desea mantener el contraste en las ima´genes capturadas para la medicio´n de la trucha arco´ıris. De igual forma, se requiere ajustar la posicio´n de la c´amara

CAP´ITULO 5.   CONCLUSIONES Y TRABAJO FUTURO 	76

de adquisicio´n de ima´genes digitales dentro del prototipo de medicio´n.

Tambi´en  debe  notarse,  que  investigar  una  nueva  forma  del  canal  del  pro- totipo, evidentemente requiere analizar materiales alternos compatibles al componente  de  iluminacio´n  que  permitan  resaltar  el  cuerpo  del  pez  dentro de las ima´genes adquiridas.

Por otro lado, el prototipo de medicio´n actual contiene dos posibles salidas para separar a los peces una vez estimada su longitud. Sin embargo, en esta versio´n del prototipo no se considero´ un sistema de separacio´n. Por lo tanto, se propone investigar una forma de separacio´n automa´tica de los peces usando compuertas una vez estimada su longitud, para decidir el estanque donde se depositara´ al pez.

Por otro lado, conforme a los tres problemas identificados de un sistema automa´tico  de  clasificacio´n  de  peces  (ver  Secci´on  1.2,  Cap´ıtulo  4),  se  reco- mienda  investigar  un  proceso  automa´tico  para  la  separacio´n  e  introduccio´n individual  de  los  peces,  el  cual  se  podr´ıa  integrar  al  prototipo  de  medicio´n propuesto en esta tesis para obtener un proceso de medicio´n automatizado.