UNIVERSIDAD AUTÓNOMA DEL ESTADO DE MÉXICO FACULTAD  DE INGENIERÍA

DETECCIÓN Y LOCALIZACIÓN AUTOMÁTICA DE LA SUPERFICIE FACIAL EN IMÁGENES 3D DE  UNA PROFUNDIDAD

TESIS

### QUE PARA OBTENER EL GRADO DE MAESTRO EN CIENCIAS  DE  LA INGENIERÍA

PRESENTA:

ING.  JUAN  PADUANO SALINAS

DIRECTOR  DE TESIS:

### DR.  MARCELO  ROMERO HUERTAS

CIUDAD UNIVERSITARIA TOLUCA, MÉXICO	OCTUBRE 2016

#### A mi familia . . .

Resumen

#### La detección del rostro es uno de los primeros pasos en las aplicaciones de procesamiento facial, cuyo propósito es identificar y localizar todos los rostros que se encuentran en una imagen, independientemente de su posición, escala y orientaciones. En esta tesis se reporta la investigación realizada en la Maestría en Ciencias de la Ingeniería de la Universidad Autónoma del Estado de México para detectar y localizar automáticamente la superficie facial utilizando imágenes 3D  de una profundidad con variaciones en número y posición de las personas en la escena. Para esto se revisó la literatura relacionada y se discute la problemática actual en la detección de la superficie facial en imágenes 3D. Se propone y evalúa experimentalmente una técnica para la detección de la superficie facial automáticamente con variaciones en el número de usuarios y su posición con respecto a la cámara. Esta técnica consta de cuatro etapas: a) Eliminación de partes planas en la imagen 3D utilizando el algoritmo MSAC, b) Análisis de curvatura para detectar puntos convexos, c) Selección automática de la punta de la nariz utilizando SpinImages y d) Clasificación rostro o no rostro. En esta investigación se utilizó el sensor Kinect One™, para colectar una base de datos de 1020 imágenes 2D/3D variando el número y posición en la escena de treinta y dos sujetos. Las imágenes 3D colectadas fueron utilizadas para la evaluación de nuestro proceso de detección de la superficie facial. Este proceso ha sido capaz de detectar con éxito el 91 % de los rostros en las imágenes colectadas que contienen más de un sujeto en la escena, y el 100 % y 98 % de los rostros contenidos en las bases de datos Face Recognition Grand Challenge y CurtinFaces, respectivamente. En ese sentido, se obtuvieron resultados que son alentadores para abordar el problema general en la detección de la superficie facial en condiciones no controladas, con más de una persona en la escena y variación en su posición con el objetivo de crear sistemas de procesamiento facial automáticos.

Publicaciones

#### Partes de la presente investigación se han publicado en:

#### Sección 4, Capítulo 3: J. Paduano, M. Romero, and R. M. Valdovinos, “Face detection in 3D images with more than one person” in Proceedings on the International Conference on Image Processing, Computer Vision and Pattern Recognition (IPCV’16), Jul 2016.

#### Sección 3, Capítulo 3: J. Paduano, M. Romero, and V. Munoz, “Toward face detection in 3D data” in Proceedings on the International Conference on Image Processing, Computer Vision and Pattern Recognition (IPCV’15), Jul 2015.

#### M. Romero, J. Paduano, and V. Munoz, “Point-triplet spin-images for landmark localisa- tion in 3d face data,” in Biometric Measurements and Systems for Security and Medical Applications (BIOMS) Proceedings, 2014 IEEE Workshop on, pp. 8–14, Oct 2014.

Índice general

- Introducción	1

  - Directriz de la tesis	2

  - Estructura de la tesis	2

- Protocolo de tesis	4

  - Estado del arte	4

  - Planteamiento del problema	6

  - Justificación	8

  - Objetivos	9

  - Hipótesis	9

  - Alcances y limitaciones	9

  - Metodología	10

  - Infraestructura requerida	11

  - Contribuciones esperadas	12

  - Referencias	13

- Human Face Detection in 3D images	15

- Discusión	47

Capítulo 1 Introducción

#### La detección del rostro se ha considerado fundamental para la interacción persona computadora además de ser una capacidad natural del ser humano. La detección facial es una etapa primaria de las aplicaciones de procesamiento facial (Bowyer et al., 2006), tales como: reconocimiento facial, seguimiento del rostro, estimación de la posición, reconocimiento de las expresiones faciales, reconocimiento de gestos, modelado facial, diseño de interfaces hombre-máquina, la comunicación multimedia y el diagnóstico médico (Yang et al., 2002).

#### Sin embargo, la mayoría de los métodos existentes en estás aplicaciones utilizan imágenes o una secuencia de imágenes que contienen básicamente un rostro.

#### Para poder construir un sistema completamente automático primero se requiere detectar el rostro y de esa manera poder extraerlo y continuar con un análisis (Hjelmås and Low, 2001). Para lograr esto se requiere desarrollar algoritmos robustos y eficientes para detectar el rostro.

#### Dada una imagen o una secuencia de imágenes, el objetivo de la detección de rostros es identificar y localizar todos los rostros humanos, independientemente de sus posiciones , escalas, orientaciones, poses y las condiciones del ambiente (Yang et al., 2002).

#### La detección facial es un problema difícil porque los rostros humanos son altamente no rígidos con un alto grado de variabilidad en tamaño, forma, color y textura (Zafeiriou et al., 2015).

#### Este trabajo de investigación propone detectar y localizar la superficie facial utilizando imágenes 3D. El interés de trabajar con imágenes 3D es que proporcionan información de profundidad explícita que se considera útil para estudiar la forma de los objetos digitalizados. Además, en comparación con las imágenes 2D, las imágenes 3D son más robustas a cambios   de iluminación del ambiente, posición y orientación del rostro de la persona en relación a los sensores 3D; proporcionando información más detallada sobre la forma y rasgos faciales finos.

#### 2	Introducción

#### La motivación de hacer este trabajo de investigación es apoyar la construcción de sistemas de procesamiento facial que trabajen con imágenes del rostro de manera automatizada, en ambientes no controlados, para lo cual se requiere desarrollar algoritmos robustos y eficientes para detectar la superficie facial.

#### Para cumplir el objetivo de esta investigación, se identificaron tres técnicas del estado del arte de la detección del rostro en imágenes 3D:(Colombo et al., 2006; Mian et al., 2006; Segundo    et al., 2007) las cuales dependen de condiciones controladas y solo detectan un rostro por imagen. En este trabajo se realizó el análisis he implementación de estas técnicas, donde se identificaron debilidades y fortalezas que permitieron proponer un proceso para la detección y localización del rostro en imágenes 3D con variaciones en el número y posición de los sujetos en la escena.

#### Por lo tanto, la investigación aquí reportada propone un proceso para la detección de la superficie facial basada en la detección de la punta de la nariz y haciendo uso de los clasificadores PCA y SVM en imágenes 3D de una profundidad adquiridas con el sensor Kinect One™.

#### Acorde al Art. 60 del Reglamento de Estudios Avanzados de la Universidad Autónoma del Estado de México la estructura de esta tesis es por artículo especializado.

#### Por lo tanto, los resultados obtenidos en esta investigación han sido enviados para revisión  y posible publicación en la revista Computer Vision  and Image Understanding, la cual tiene   un factor de impacto de 2.134 y esta indexada en Scopus, Computing Reviews, Current Con- tents/Engineering, Computing & Technology, Index to Scientific Reviews, Science Abstracts - Physics Abstracts y Science Citation Index.

## Directriz de la tesis

#### Se espera que ésta tesis se útil para todo aquel interesado en el procesamiento de imágenes faciales 3D utilizando imágenes de profundidad obtenidas con el sensor Kinect One™, específica- mente se presenta una alternativa para la detección del rostro que pudiera ayudar al desarrollo de aplicaciones finales.

#### Este trabajo de investigación les puede servir a personas interesadas en aplicaciones procesa- miento facial ya que la detección del rostro es una tarea primaria para estas aplicaciones.

## Estructura de la tesis

#### Adicionalmente a este capítulo de introducción, esta tesis contiene el siguiente capitulado:

#### 1.2 Estructura de la tesis	3

En el Capítulo II: “Protocolo de tesis”, se muestra el protocolo de investigación desarrollado para esta tesis.

En el Capítulo III: “Human Face Detection in 3D Images”, se presenta el artículo de detección facial enviado para su revisión a la revista Computer Vision and Image Understanding.

#### En el Capítulo IV: “Discusión”, se puntualizan los hallazgos relevantes obtenidos en esta investigación.

Capítulo 2 Protocolo de tesis

## Estado del arte

#### La detección facial es una tarea primaria en aplicaciones de procesamiento facial (Yang et al., 2002), puesto que es esencial detectar el rostro dentro de una imagen para proceder a su análisis. Al paso de las décadas cuando se habla de detección facial, los investigadores entran en controversia respecto ha trabajar con imágenes de profundidad o con imágenes de intensidad. Como se sabe, las imágenes de intensidad presentan factores que retrasan o impiden el análisis de la superficie facial (Bowyer et al., 2006) . Es por eso el interés del estudio de la superficie facial en imágenes 3D. La adquisición de la imagen facial 3D es invariante a cambios en la iluminación durante la captura, además se considera que las imágenes 3D representan explícitamente la forma y dimensión del rostro humano. Trabajos como los de Bowyer et al. (2006) y Xu et al. (2004) han comprobado que si se quiere analizar la forma de los objetos, la manera natural es utilizando una

#### imagen 3D.

#### El incremento del poder computacional así como la aparición de los sensores 3D de bajo costo, por ejemplo, el sensor Kinect™ (Microsoft, 2014) , han permitido incrementar el estudio en el procesamiento de imágenes digitales en particular en aplicaciones de procesamiento facial, tales como; reconocimiento facial, seguimiento del rostro, estimación de la posición, reconocimiento de las expresiones faciales y reconocimiento de gestos (Bowyer et al., 2006; Lei et al., 2014; Ortega-Garcia et al., 2004; Pears et al., 2012; Smeets et al., 2010; Xu et al., 2004).

#### La detección de la superficie facial es clasificada de acuerdo al tipo de imagen que se utilice (Bowyer et al., 2006):

#### Estado del arte	5

#### La detección facial 2D: es aquella que utiliza imágenes de intensidad/color para detectar el rostro.

#### La detección facial 3D: es aquella que utiliza imágenes de profundidad para detectar el rostro.

#### La detección facial multimodal: es aquella que utiliza tanto las imágenes 2D/3D.

#### En particular, este trabajo de investigación se centrará en la detección de la superficie facial en imágenes 3D.

#### En la literatura se encuentran estudios relacionados con aplicaciones de procesamiento facial 3D, sin embargo, muy pocos de ellos se dedican específicamente a la detección facial; Por ejemplo, Colombo et al. (2006) implementó un algoritmo de detección facial que fue experimentado en imágenes de posición frontal, el cual es capaz de determinar la presencia de un rostro y en caso afirmativo extraer la superficie facial. Primero las áreas que no pertenecen al rostro son descartadas, a continuación analizan las áreas cóncavas y convexas del rostro utilizando el análisis de curvatura con el fin de encontrar las esquinas de los ojos y la punta de la nariz con los cuales calculan n combinaciones de puntas de la nariz con esquinas de los ojos. Si al final de estos pasos no encuentra al menos un candidato como nariz y dos candidatos como ojos se asume que no existe un rostro en la imagen. El reporte de resultados no precisa el número de combinaciones obtenidas, sin embargo, es evidente que se obtiene una cantidad de combinaciones considerables.

#### Después de obtener las n combinaciones de esquinas interiores de los ojos y punta de la nariz se alinean las imágenes en una posición estándar utilizando transformaciones rígidas. Al final crean un nueva imagen 3D con la cual analizan la parte rígida del rostro utilizando eigenfaces para clasificar la imagen en rostros o no rostros.

#### Por su parte, Mian et al. (2006) presenta un sistema automático de reconocimiento facial 3D, en cuyas primeras etapas detecta y alinea la superficie facial, correspondiendo las imágenes 2D y 3D obtenidas por el sensor y utilizando la transformada de Hotelling y su algoritmo clasificador Spherical Face Representation (SFR). La experimentación aquí reportada utiliza la base de datos del Face Recognition Grand Challenge (FRGC), la cual contiene 9500 imágenes 2D/3D de 370 personas tomadas desde la parte superior del tórax a la cabeza. En este trabajo, para extraer la superficie facial de la imagen 3D se detecta la punta de la nariz, cortando la imagen horizontalmente y analizando el contenido del corte punto por punto para identificar la punta de la nariz. Los puntos que no correspondan a la región de la nariz son considerados atípicos y por lo tanto se remueven. Detectada la punta de la nariz, se utiliza un radio de ocho centímetros a partir de este punto para extraer el rostro de la imagen 3D.

#### 6	Protocolo de tesis

#### En un trabajo similar, Segundo et al. (2007), propone una metodología para la detección de la superficie facial a través de segmentación de un mapa de profundidad obtenido de la imagen 3D. Esta técnica fue diseñada para imágenes que contienen una persona. Para esto, primero discriminan los puntos que no pertenecen a la superficie facial, utilizando el algoritmo K-Means (k = 3) para obtener tres clusters de la imagen: fondo, cuerpo y cabeza. Sin embargo, se reporta que este paso no es suficiente porque existen otras regiones en el área de la cabeza que afectan el resultado, por ejemplo la presencia de cabello, oídos y cuello. Posteriormente, se detecta el contorno de la superficie facial utilizando la transformada de Hough, la cual se asume de forma elipsoidal. Al igual que Mian et al. (2006), en esta experimentación se utiliza la base de datos FRGC.

#### Adicionalmente se pueden identificar otras investigaciones que de forma indirecta realizan la detección de la superficie facial en imágenes 3D. Por ejemplo, Creusot et al. (2010) desarrolla un sistema para etiquetar los puntos antropométricos (Landmarks) de la superficie facial utilizando imágenes que solo contienen el rostro de una persona. En este sentido, Xu et al. (2004) propone un método para localizar la punta de la nariz utilizando un esquema de filtrado jerárquico que combina características locales. Posteriormente, en base a la punta de la nariz detectada, se estima la cresta utilizando un análisis de curva denominado Included Angle Curve (IAC). Un trabajo similar es el propuesto por Dibeklioglu et al. (2008), quien sugiere un método basado en análisis de curvatura para la localización de la punta de la nariz en imágenes 2D, que posteriormente corresponde a la imagen 3D.

#### Como se puede observar en la literatura, la detección de la superficie facial es una tarea primaria en aplicaciones de procesamiento facial, en la cual se requiere un estudio experimental detallado para la detección de la superficie facial en imágenes 3D para ser utilizado en aplicaciones de procesamiento facial 3D.

## Planteamiento del problema

#### La detección de la superficie facial es un problema de alta complejidad que debido a que influyen diversos factores, por ejemplo: Los rasgos faciales que varían de un individuo a otro (e incluso para una sola persona), dichas variaciones son debido a la posición, tamaño, ángulo de visión, entre otros. La investigación de visión por computadora ha recorrido un largo camino para hacer frente a estas dificultades, pero sigue habiendo oportunidades de mejora (Bowyer et al., 2006).

#### Planteamiento del problema	7

#### La detección de la superficie facial 3D, consiste en identificar y localizar cada uno de los rostros humanos en una imagen 3D, independientemente de su posición, escala y orientación. En su forma más básica, una imagen 3D representa la superficie de los objetos mediante una nube de puntos 3D, generalmente con coordenadas cartesianas (x, y, z), los cuales pueden ser procesados de acuerdo a su información contextual, proximidad y alineación.

#### Existen técnicas que permiten la detección de la superficie facial 3D, por ejemplo: Colombo et al. (2006), Mian et al. (2006), Nair and Cavallaro (2009) y Segundo et al. (2007). Lamentable- mente, estos trabajos dependen de la posición de la persona y la detección se realiza con imágenes 3D que contienen básicamente un rostro humano. Además, estas imágenes fueron tomadas en condiciones controladas de posición, ubicación y expresiones faciales.

#### La dificultad que representa la detección de la superficie facial depende de factores tales como: La posición del sujeto, tipo de sensor 3D utilizado para la captura de imagen, número de vértices que inciden en la superficie facial, el tipo de imagen 3D, la presencia de expresiones faciales y el tiempo de procesamiento:

#### La posición del sujeto juega un papel importante en la detección facial, ya que puede causar oclusión en la imagen 3D y esto complica la detección del rostro.

#### El tipo de imagen 3D puede ser no estructurada (la cual contiene solamente los vértices en el espacio tridimensional), y estructurada (que además de contener los vértices proporciona la conexión de los mismos). Si las imágenes 3D son no estructuradas dificulta el análisis de la superficie facial porque no existe la conexión entre los vértices.

#### El sensor 3D influye en el tipo de dato que se este utilizando. Como se mencionó en la Sección 2.1 existen dos tipos de imágenes 3D estructuradas y no estructuradas, algunos trabajos necesitan de una imagen estructurada para poder ser utilizados, lamentablemente no todos los sensores 3D proporcionan este tipo de imagen.

#### El número de vértices que inciden en la superficie facial dificultan la detección del rostro, puesto que a menor número de vértices se complica el análisis de la superficie. Por el contrario, a mayor número de vértices que inciden en la superficie facial se facilita su análisis. Es importante aclarar que se esta hablando de los vértices que pertenecen a la superficie facial y no de todos los vértices contenidos en la escena.

#### Las expresiones faciales son consideradas como problema fundamental para el análisis de la superficie facial, ya que cambia la superficie del rostro de una expresión a otra y ademas cada individuo es diferente.

#### 8	Protocolo de tesis

#### El tiempo de procesamiento es importante. Por ejemplo, si los resultados de la detección facial 3D son del 99 % en un tiempo mayor de 10 minutos, no serviría en aplicaciones   de seguridad o seguimiento del rostro. En este sentido, es necesario obtener mediciones cuantitativas del tiempo, el cual es un tema que no se ha analizado en la literatura.

#### Es importante la precisión con la que se puede detectar la superficie facial en imágenes 3D que contienen más de un sujeto en escenarios reales.

#### La mayoría de los algoritmos existentes de detección de la superficie facial han sido evaluados con bases de datos 3D que fueron colectadas con sensores 3D especializados, por ejemplo la base de datos FRGC. Sin embargo, a partir del año 2010 Microsoft inicio la comercialización del sensor Kinect™ (Microsoft, 2014), que es una sensor de profundidad de bajo costo que desde su lanzamiento ha sido investigado por la comunidad científica. Lamentablemente, estos estudios se han realizado con imágenes que contienen un solo sujeto en la escena. Esta será una característica de interés para la investigación propuesta, puesto que se estudiará el problema de detección y localización de la superficie facial en imágenes de una profundidad colectadas con el sensor Kinect™, que contengan variaciones en el número y posición de sujetos en la escena.

## Justificación

#### El interés de hacer este trabajo de investigación es ayudar a la construcción de sistemas de procesamiento facial que trabajen con imágenes 3D del rostro humano de manera automatizada, en diferentes variaciones en número y posición de sujetos en la escena, entonces se requiere desarrollar algoritmos robustos y eficientes para detectar la superficie facial en una imagen 3D de una profundidad puesto que en la literatura se observa como un posible punto de mejora.

#### Como parte de este trabajo, se evaluarán experimentalmente los cambios en el rendimiento de los algoritmos de detección facial ante variaciones como posición y número de personas en la escena. Para este propósito, se utilizará el sensor Kinect One™.

#### Como se puede observar en la literatura relacionada, se han publicado técnicas para detectar la superficie facial, sin embargo, éstas funcionan en condiciones controladas, por ejemplo: posiciones frontales, un solo usuario en la escena, a nivel de tórax y hasta la cabeza del usuario.

#### Con las nuevas tecnologías en especifico el sensor Kinect ™, se podrán estudiar imágenes con los siguientes factores: más de una persona por escena, variaciones de posición y escenarios no controlados. En estas condiciones los algoritmos propuestos para la detección facial que      se encuentran en la literatura no funcionarían por las condiciones en las que fueron diseñadas.

#### 2.4 Objetivos	9

#### Es por eso que se requiere de un estudio de la detección de la superficie facial para poder ser implementado en aplicaciones de procesamiento facial.

## Objetivos

### Objetivo general

#### Detectar y localizar la superficie facial automáticamente en imágenes 3D de una profundidad con variaciones en el número de usuarios y su posición con respecto a la cámara para aplicaciones de procesamiento facial.

### Objetivos particulares

#### Analizar experimentalmente las técnicas de detección facial en imágenes 3D propuestas en el estado del arte para proponer nuevos algoritmos para la detección y localización de la superficie facial.

#### Colectar imágenes experimentales con variaciones en el número y posición del sujeto en la escena utilizando el sensor Kinect One™ para evaluar los algoritmos de detección y localización de la superficie facial.

## Hipótesis

#### Las imágenes 3D de una profundidad adquiridas con el sensor Kinect™, permiten detectar  y localizar la superficie facial de los individuos presentes en una escena con variaciones en el número y posición de sujetos.

## Alcances y limitaciones

### Alcances

#### Un proceso para detectar la superficie facial en imágenes 3D con variaciones en el número y posición de los sujetos al sensor Kinect One™, capaz de detectar el 91 % de los rostros presentes en la escena con un error menor a 16mm.

#### 10	Protocolo de tesis

#### Un conjunto de imágenes faciales experimentales con variaciones en el número y distancia de los sujetos al sensor, variando de uno a ocho sujetos en la escena de un total de treinta y dos personas.

#### El proceso de detección de rostros propuesta en esta investigación solamente esta basada en la localización de la punta de la nariz.

### Limitaciones

#### El sensor utilizado para la colecta de imágenes experimentales tiene limitantes técnicas como: su campo de visión y baja resolución espacial.

#### Parámetros y umbrales pre-establecidos y calculados de forma experimental en el proceso de detección propuesto: detección de partes planas, cálculo curvatura Media y Gaussiana, cálculo de histograma de SpinImages.

## Metodología

#### Para el desarrollo de este trabajo de investigación, se utiliza una metodología de seis etapas:

    - Definición del problema: En esta etapa se define el problema que se quiere resolver partiendo de lo general a lo particular.

#### Revisión de la literatura: Se revisa el estado del arte en la detección y localización de la superficie facial en imágenes 3D. Se evalúa experimentalmente las técnicas existentes en la literatura para detectar la superficie facial 3D.

#### Análisis y diseño operacional: Analizar y diseñar procedimientos, técnicas y algoritmos para realizar de detección y localización de la superficie facial en imágenes 3D. Determinar algoritmos para la detección y localización de la superficie facial en imágenes 3D conside- rando variaciones en el número y posición de los sujetos en una escena para aplicaciones de procesamiento facial. Diseño de función de captura de un conjunto de imágenes expe- rimentales considerando variaciones en el número y posición de los sujetos en la escena. Entendiendo por variaciones de posición, la ubicación del sujeto con respecto al eje z del sistema de referencia del sensor de captura.

#### 2.8 Infraestructura requerida	11

    - Adquisición de datos experimentales: Adquirir un conjunto de imágenes experimentales 2D/3D utilizando un módulo de captura del sensor Kinect One™.

#### Experimentación: Probar los procedimientos, técnicas y algoritmos de detección facial propuestos en diferentes condiciones (posición de rostro, número de sujetos en la escena) utilizando bases de datos del estado del arte (FRGC y CurtinFaces) y el conjunto de imágenes experimentales colectadas en esta investigación.

    - Evaluación y análisis de resultados: Evaluar y analizar la robustez de métodos, técnicas o algoritmos propuestos, además definir los puntos de mejora.

## Infraestructura requerida

#### Para poder llevar a cabo este trabajo de investigación se requiere de la infraestructura siguiente:

#### Bases de datos de rostros faciales 3D, estas bases permitirán implementar las técnicas existentes de detección de la superficie facial.

#### Equipo de cómputo con las siguientes características mínimas: Procesador (Intel Core i7 quad core de 2.8 GHz, Turbo Boost de hasta 4.0 GHz), Memoria (16 GB SDRAM DDR3L de 1600 MHz), Espacio de almacenamiento (flash basado en PCIe de 512 GB). En este equipo de cómputo se programarán las técnicas de detección facial.

#### Dispositivos de almacenamiento de 3TB de espacio para poder guardar los archivos re- sultantes de las pruebas, además de servir como medio de respaldo para las bases de datos.

#### Un sensor Kinect One™ para implementar el escenario de experimentación y capturar las imágenes experimentales.

#### Servidor con las siguientes características: Procesador (12 núcleos de 2.7 GHz con 30 MB de caché L3), Memoria (64 GB de memoria RAM de al menos 1866 MHz) , Almacenamiento (flash de 1 TB basado en PCIe), Tarjetas gráficas (Dos GPUs AMD FirePro D700 con 6 GB de GDDR5 VRAM cada uno). En este servidor se realizarán evaluaciones masivas de los algoritmos propuestos para la detección de la superficie facial.

#### Acceso a bibliotecas digitales para poder estudiar las tendencias de esta investigación.

#### 12	Protocolo de tesis

## Contribuciones esperadas

#### Las contribuciones originales que se espera derivar de este trabajo de investigación son las siguientes:

#### Algoritmo para la detección y localización de la superficie facial en imágenes 3D de una profundidad con variaciones en el número y posición de los sujetos en la escena.

#### Conjunto de imágenes 3D capturadas con sensores de una profundidad en escenas no controladas.

# Referencias

#### Bowyer, K. W., Chang, K., Flynn, P., 1 2006. A survey of approaches and challenges in 3d and multi-modal 3d + 2d face recognition. Computer Vision and Image Understanding 101 (1), pp. 1–15.

#### Colombo, A., Cusano, C., Schettini, R., Mar 2006. 3d face detection using curvature analysis.

#### Pattern Recognition 39 (3), pp. 444 – 455.

#### Creusot, C., Pears, N., Austin, J., October 2010. 3d face landmark labelling. In: Proceedings of the ACM workshop on 3D object retrieval. ACM, Association for Computing Machinery, pp. 27–32.

#### Dibeklioglu, H., Salah, A. A., Akarun, L., 2008. 3d facial landmarking under expression, pose, and occlusion variations. In: Biometrics: Theory, Applications and Systems, 2008. BTAS 2008. 2nd IEEE International Conference on. pp. 1–6.

#### Hjelmås, E., Low, B. K., 2001. Face detection: A survey. Computer Vision and Image Understan- ding 83 (3), pp. 236 – 274.

#### Lei, Y., Bennamoun, M., Hayat, M., Guo, Y., 2 2014. An efficient 3d face recognition approach using local geometrical signatures. Pattern Recognition 47 (2), pp. 509–524.

#### Mian, A., Bennamoun, M., Owens, R., 2006. Automatic 3d face detection, normalization and recognition. In: 3D Data Processing, Visualization, and Transmission, Third International Symposium on. pp. 735–742.

#### Microsoft, C., Microsoft Kinect for Windows, 2014. Nov. 2014.

#### URL http://www.microsoft.com/en-us/kinectforwindows/

#### Nair, P., Cavallaro, A., 2009. 3-d face detection, landmark localization, and registration using a point distribution model. Multimedia, IEEE Transactions on 11 (4), pp. 611–623.

#### 14	2.10.  Referencias

#### Ortega-Garcia, J., Bigun, J., Reynolds, D., Gonzalez-Rodriguez, J., 2004. Authentication gets personal with biometrics. Signal Processing Magazine, IEEE 21 (2), pp. 50–62.

#### Pears, N., Liu, Y., Bunting, P., 2012. 3D Imaging, Analysis and Applications, 1st Edition. Vol. 1.

#### Springer-Verlag London.

#### Segundo, M. P., Queirolo, C., Bellon, O. R. P., Silva, L., 2007. Automatic 3d facial segmen- tation and landmark detection. In: Image Analysis and Processing, 2007. ICIAP 2007. 14th International Conference on. pp. 431–436.

#### Smeets, D., Claes, P., Vandermeulen, D., Clement, J. G., 9 2010. Objective 3d face recognition: Evolution, approaches and challenges. Forensic Science International 201 (1–3), pp. 125–132.

#### Xu, C., Wang, Y., Tan, T., L., Q., 2004. Depth vs. intensity: which is more important for face recognition? Pattern Recognition, 2004. ICPR 2004. Proceedings of the 17th International Conference on 1, pp. 342–345 Vol.1.

#### Yang, M., Kriegman, D., Ahuja, N., Jan 2002. Detecting faces in images: a survey. Pattern Analysis and Machine Intelligence, IEEE Transactions on 24 (1), pp. 34–58.

#### Zafeiriou, S., Zhang, C., Zhang, Z., 2015. A survey on face detection in the wild: Past, present and future. Computer Vision and Image Understanding 138, pp. 1 – 24.

Capítulo 3

Human Face Detection in 3D images

#### En este capítulo se presenta un artículo que reporta los resultados obtenidos en esta investi- gación, el cual ha sido enviado para su revisión y posible publicación en la Revista Científica Computer Vision and Image Understanding, la cual tiene un factor de impacto de 2.1 y se encuentra indexada en Scopus.

Elsevier Editorial System(tm) for Computer Vision and Image Understanding

Manuscript Draft

Manuscript Number:

Title: Human Face Detection in 3D Images Article Type: Research paper

Keywords: 3D face detection; 3D face processing; Biometrics Corresponding Author: Dr. Marcelo Romero, Ph.D.

Corresponding Author's Institution: Universidad Autonoma del Estado de Mexico

First Author: Juan Paduano

Order of Authors: Juan Paduano; Marcelo Romero, Ph.D.

Abstract: Face detection is one of the most studied topics in computer vision; however, current 3D face detection techniques depend on the position of the person and they are performed basically on images that contain mainly the	human face. In particular, three key face detection techniques were experimentally investigated to begin this research. The novelty of this research is to detect and locate every face present into a 3D image regardless of the number of subjects and their distance to the camera. For this purpose, we are introducing an eective straight forward approach that consists of four steps. We populated a 3D image database using the Kinect One sensor, varying positions from one to eight dierent subjects in the scene. In addition to our 3D image database, this face detection approach has been evaluated with state of the art databases, Face Recognition Grand Challenge (FRGC) and CurtinFaces that contain one subject per image. With localisation errors from 0 to 16 mm, our experimental results are 100%, 98% and 91% for the FRGC, CurtinFaces and our database, respectively.

Suggested Reviewers: Felipe Orihuela PhD

Full Time Researcher, Ciencias Computacionales, INAOE f.orihuela-espina@ccc.inaoep.mx

Active researcher in the field

Fernando Arambula PhD

Full Time Researcher, CCADET, Universidad Nacional Autonoma de Mexico fernando.arambula@ccadet.unam.mx

Active researcher in the field

Juan Carlos Cuevas PhD

Full Time Researcher, Facultad de Ingenieria, Universidad Autonoma de San Luis Potosi

cuevastello@gmail.com

Active researcher in the field

*1. Cover Letter

TITLE OF THE SUBMITTED MANUSCRIPT:

##### Human Face Detection in 3D Images

LIST OF ALL AUTHORS’ NAMES AND AFFILIATIONS:

##### Juan Paduano and Marcelo Romero Facultad de Ingeniería

##### Universidad Autónoma del Estado de México

A SHORT STATEMENT (<50 words) OF THE PRECISE PROBLEM OR OBJECTIVE ADDRESSED IN THE PAPER:

##### A simple four-steps process for face detection in 3D images with more than one person is presented.

A VERY BRIEF (<100 words) DESCRIPTION OF THE ESSENCE OF YOUR APPROACH:

A LIST OF THE SPECIFIC MAJOR NOVEL CONTRIBUTIONS (up to 3) REPORTED HERE:

##### A novel face detection approach

##### A state of the art dataset

##### Experimental results

LIST OF OTHER JOURNAL OR CONFERENCE PAPERS (if any) PUBISHED OR SUBMITTED BY YOU OR ANY CO-AUTHOR THAT HAVE A SIGNIFICANT OVERLAP WITH THE CONTRIBUTION SUBMITTED HERE ACCOMPANIED BY A BRIEF EXPLANATION OF THE NATURE OF THIS OVERLAP POINTING OUT CLEARLY WHICH NOVEL IDEAS SUBMITTED HERE HAVE NOT BEEN DISCUSSED IN THESE PRIOR PUBLICATIONS:

##### IPCV 2015, WorldComp 2015, http://worldcomp- proceedings.com/proc/proc2015/

##### IPCV 2016, WorldComp 2016, http://worldcomp-proceedings.com/proc/p2016/

A REFERENCE TO THE CLOSEST PRIOR ARTICLE (by others) UPON WHICH YOUR CONTRIBUTIONS IMPROVE:

##### None.

THE NAMES, EMAILS, AND HOMEPAGE URLS OF FOUR EXPERTS COVERING THESE AREAS

AND FIELDS (by expert, we refer to someone who has published several high quality papers in that technical area in the last five years and who is recognized internationally as an expert in one of the fields you listed above):

##### Kevin W. Bowyer, kwb@cse.nd.edu, https://engineering.nd.edu/profiles/kbowyer

##### Ajmal Saeed Mian, ajmal.mian@uwa.edu.au, http://www.web.uwa.edu.au/people/ajmal.mian

##### Nick Pears, nick.pears@york.ac.uk, https://www-users.cs.york.ac.uk/nep/

##### Ioannis A. Kakadiaris, ioannisk@uh.edu, http://www.ee.uh.edu/faculty/kakadiaris

THE NAMES OF THE TWO GMOD ASSOCIATE EDITORS WHO YOU BELIEVE ARE THE MOST QUALIFIED TO HANDLE YOUR PAPER (please

go to http://www.elsevier.com/wps/find/journaleditorialboard.cws_home/6 22839/editorialboard to see list of associate editors:

TECHNICAL AREAS AND FIELDS OF EXPERTISE NECESSARY TO FULLY UNDERSTAND YOUR CONTRIBUTION AND TO EVALUATE ITS

POTENTIAL AND NOVELTY (see list of areas and topics posted at http://ees.elsevier.com/gmod/ and feel free to add areas and fields as needed):

##### 1.   3D image processing

*2. Manuscript

Human Face Detection in 3D  Images

Juan Paduano, Marcelo Romero∗

Facultad de Ingenier´ıa, Universidad Auto´noma del Estado de M´exico.  Cerro de Coatepec s/n.  Ciudad Universitaria, 50100, Toluca, Estado de M´exico, M´exico

##### Abstract

Face detection is one of the most studied topics in computer vision; how- ever, current 3D face detection techniques depend on the position of the person and they are performed basically on images that contain mainly the human face. In particular, three key face detection techniques were exper- imentally investigated to begin this research. The novelty of this research is to detect and locate every face present into a 3D image regardless of the number of subjects and their distance to the camera. For this purpose, we are introducing an effective straight forward approach that consists of four steps. We populated a 3D image database using the Kinect OneTM sensor, varying positions from one to eight different subjects in the scene. In addition to our 3D image database, this face detection approach has been evaluated with state of the art databases, Face Recognition Grand Challenge (FRGC) and CurtinFaces that contain one subject per image. With localisation errors from 0 to 16 mm, our experimental results are 100%, 98% and 91% for the FRGC, CurtinFaces and our database, respectively.

Keywords:	3D face detection, 3D face processing, Biometrics

1    1. Introduction

2	Face detection is the first step in almost every face processing applica-

3    tion, where the face is localised and extracted from an incoming image  prior

4      to specific analysis (Hjelm˚as and Low, 2001; Bowyer et al., 2006; Zafeiriou

5    et al., 2015).  However,    face detection from a single image is a challenging

∗Corresponding author: Tel.  +52 722 214 08 55

Email address: mromeroh@uaemex.mx (Marcelo Romero)

Preprint submitted to Computer Vision and  Image Understanding	October 11, 2016

6  task because of variability in scale, location, orientation (up-right, rotated),

7    and pose (frontal,  profile);  also facial expressions,  occlusion,  and  lighting

8    conditions change the overall face appearance  (Yang et al., 2002; Yang   and

9 Ahuja, 2001; Bowyer et al., 2006; Pears et al., 2012; Paduano et al., 2015).

10   Evidently, face detection is the first step in any automated system that solves

11    problems about face recognition areas.  However,  most of the existing   face

12    processing methods, assume that only one human face is present in an image

13    or into a video (Yang and Ahuja, 2001; Bowyer et al., 2006; Zafeiriou et  al.,

14    2015; Ariz et al., 2016).

15	Although 2D detection algorithms have reached an acceptable level in face

16      detection,  they  have  a  lot  of  difficulties because  the  3D  world  is projected

17           down onto a 2D image, resulting on ambiguities and losing depth informa-

18    tion (Pears et al., 2012).        On the other hand,  3D images provide explicit

19            shape information and they are considered robust to illumination and pose

20 variations (Bowyer et al., 2006).

21	Pan et al. (2013) used heterogeneous feature descriptors and feature se-

22                 lection in combination with Adaboost (Yin et al., 2015; Hu et al., 2008),

23    although this is a robust and efficient face detection method, it depends on

24    frontal position. Other pose-dependent face detection investigations using 2D

25   image are: Subburaman and Marcel (2013); Aghaei et al. (2016); Bhandarkar

26    and Luo (2009); Wang and Ji  (2007).

27	Face detection is necessary for face processing application, for   example:

28    face recognition  (Wagner et al., 2012; Kumar et al., 2015; Kakadiaris et  al.,

29    2016), face tracking	(Smeulders et al., 2014; Aghaei et al., 2016; Bhan-

30   darkar and Luo, 2009), pose estimation (Wohlhart and Lepetit, 2015), facial

31    expressions and facial gestures recognition (Valstar  et al., 2012).     Unfortu-

32    nately,  different factors like position of the subject,  3D     sensor,  structural

33    components, occlusion, facial surface resolution and facial expression,  make

34    face detection a complicated problem (Bowyer et al., 2006; Pears et al., 2012;

35    Yang et al., 2014; Kumar et al., 2015; Chellappa et al.,  1995).

36	Although face detection is essential for many face processing applications,

37    it has received little attention, especially when using 3D data. In this paper,

38    we  propose an innovative  approach for 3D face detection,  that deals    with

39    face detection using one view 3D images of scenes with variations in number

40    of persons and the distance to the  sensor.

41	The rest of this paper is as follows. Section 2 describes previous work on

42    3D face detection. Section 3 reports our experimental research in 3D  images

43    with one person.        Section 4 introduces our face detection approach in 3D

2

44    images with more than one person.  Finally, Section 5 concludes this  paper.

45    2.  Related work

46	There is a number of papers in face processing applications, however,

47    only few of them are specifically dedicated to face detection.      This section

48    discusses relevant related work on face detection in 3D  images.

49	Mian et al. (2006) proposed a nose tip detection method by using hori-

50    zontally sliced contours along a face image. Each sliced contour is formed by

51    n vertex.  For each vertex in each sliced contour, circles of constant radius

52    were estimated with centre in each vertex. Next, circumscribed triangles were

53   formed in all the circles by using three vertices, one vertex is the centre of the

54    circle and two intersection vertices located by    coinciding the circle contour

55    and points in the slice.  After triangle altitude comparison, only one  triangle

56   is selected in each slice as the nose tip candidate, the one with maximum al-

57    titude from the centre of the circle.  Random Sample Consensus  (RANSAC)

58    (Trucco and Verri, 1998) was used next to further outliers removal and   from

59 all the remaining nose tip candidates, the one with the maximum altitude is

60    taken as the nose tip.      Finally,  the face is extracted using a pre-established

61    radius.        They reported 98.3% accuracy in face detection using the FRGC

62     database.

63	Colombo et al. (2006) proposed a technique based on curvature   analysis

64            for face detection, they started the process using a range image containing

65             a single face that is a representation of the (x, y, z) coordinates of the 3D

66  image in a location (i, j). Every 3D point’s location was expressed regarding

67    to the 3D sensor reference system.  Next, from the range images, the    Mean

68            and Gaussian curvature were computed by the first and second derivatives

69    in x and y.  They considered a bi-quadratic polynomial approximation of the

70    surface and the coefficients were obtained using least squares by fitting    the

71    points in a neighborhood of x and y.       The objective for calculating Mean

72    and Gaussian curvatures, is to use HK classification to obtain four segment

73    types:  convex, concave, and two saddle regions.  The output may contain

74    many candidates for facial features.  If no nose or less than two candidate

75   eyes are detected, they assume that no face is present in the scene. Finally, if

76      a face is detected, the output is a list containing its location and facial   area.

77      They experimented with their own collected data using a MinoltaTM    camera

78    and they achieved 96.85% successful face detection.

3

79	Segundo et al. (2007) presented a method to extract the face region of

80    a single person.  They also used range images generated from a point  cloud.

81    Their face segmentation algorithm consists of two  main stages:      First,  the

82    homogeneous regions localization  in the input image using    clustering  and

83    edge  detection is used.	Second,  the  candidates  are  identified  when they

84              belong to the face region, then this region is delimited by an ellipse using

85    the Hough Transform.   They applied the K-Means algorithm with k  =   3

86    for image segmentation.     So, the input image was segmented in three main

87    regions identified as:  background,  body,  and face.   However,  this step  by

88   itself is not enough for face segmentation, then it is necessary to extract the

89    face region by edge detection.  After performing region and edge   detection,

90            which can be made in parallel, they combine the two resulting images and

91    finally, by an and operation, the facial region was obtained.  They   reported

92    a 99.95% face detection using the FRGC  database.

93	Nair and Cavallaro (2009) present an accurate and robust framework

94           for face detection and segmentation using landmark localization and doing

95    fine registration of face meshes by fitting a facial model (polygons and   ver-

96    tices).  This model is based on a 3D Point Distribution Model (PDM) that

97    is fitted without  relying on texture,  pose,  or orientation information.     Fit-

98    ting is initialized using candidate locations on the mesh, which are  extracted

99    from low-level curvature-based feature maps. Face detection is performed by

100   classifying the transformations between model points and candidate vertices

101      based on the upper-bound of the deviation of the parameters from the  mean

102    model.  The performance of face detection is evaluated on a database of face

103    and no-face images achieving an accuracy of 99.6%. They also demonstrated

104    face detection and segmentation with different scale and  pose.

105	From literature reviewed above, we have experimentally investigated three

106               key face detection techniques (Paduano et al., 2015) using state of the art

107    databases:  Face  Recognition Grand Challenge and CurtinFaces for a    deep

108   understanding. Findings and knowledge from this experimental research al-

109    low us to propose our face detection approach (Paduano et al.,   2016).

110	Additionally,  one may identify a number of related face processing    re-

111    search using 2D and 3D images.  For example, Peng et al. (2011) propose   a

112    nose tip detection method that has three main characteristics. First, it does

113    not require training and does not rely on any particular model.  Second, it

114      can deal with both frontal and non-frontal poses.  Finally, it is quite fast,  re-

115    quiring only seconds to process an image of 100 to 200 pixels (in both x and

116    y dimensions).  Chang et al. (2006) set two thresholds for the two   curvature

4

117            maps to search for the nose tip, the two eyes and the nose ridge, they used

118    a database of 4000 images of 449 different subjects. Hutton et al. (2003) use

119    a hybrid iterative closest-point (ICP) and active shape model (ASM) fitting

120    for non-rigid registration of a dense surface model on 3D faces. This method

121    does not require texture so it imposes constraints on the orientation of the

122    face and it is not scale invariant.     On the other hand, Nanni et al. (2014)

123    presents a face detector based on Viola and Jones algorithm.  Soldera et    al.

124    (2015) proposes a method of facial recognition in 2D images based on   pro-

125    jections of high dimensionality face images into low dimensions.  In his face

126   recognition study, it is explained that remains a difficult task since 2D images

127    of the face can be affected by changes in the scene, for example: position  of

128    the face, facial expressions, or lighting.  Their recognition method is a  mod-

129    ification of the technique introduced by Cai et al. (2006), which proposes    a

130    face recognition method based on appearance, called orthogonal   Laplacian-

131    faces (OLPP). Zhang et al. (2008) proposes a method for detecting faces    in

132    2D images based on principal component analysis (PCA) and support vec-

133    tor machine (SVM). First, they filter areas that are potentially a face  using

134    a statistical distribution function of the local histogram.  Then,  Candidate

135               faces are reduced using a SVM classifier.  Chen et al. (2009) proposes a face

136    detection method using a 2D half-template face model.  In this research they

137                 explain that face detection is an important process for facial recognition.

138    Their model is an average face template containing:  eyes, nose, mouth   and

139    part of the cheek. The face detection was made using a template matching

140   method to determine the position of the face in the image. Their theoretical

141    analysis shows that the average face template can reduce the chances of local

142    density and that it can be adapted to side face images at an angle,   which

143    improves the accuracy of face detection in lateral position.  Belahcene et   al.

144    (2014) proposes a five stages face recognition system (FRS):   preprocessing

145      and detection, branch, fusion features, feature extraction and   classification.

146    The first stage, is to detect the face using horizontal and vertical curves inte-

147    grated projection. In the second stage, they divide the face using face patches

148    consisting of the nose and eyes.  In the third stage, they combine the   image

149      information of  the  2D  image depth,  which  is  used by  PCA  and Enhanced

150    model Fisher (EFM) to extract characteristic features.  These were used  to

151    the classification stage using two methods, measures based on distance    and

152    a support vector machine (SVM). Maes et al. (2010) presents an  adapted

153              SIFT algorithm for 3D surfaces (called meshSIFT) and its applications to

154    3D face pose normalization and recognition.  Their algorithm allows reliable

5

155    detection of scale space as local feature locations.  The meshSIFT  algorithm

156               then describes the neighbourhood of every scale space in a feature vector

157    consisting of concatenated histograms of shape indices and angles.       Then,

158    feature vectors are reliably matched by comparing the angle in feature space.

##### 159   3. Face detection in 3D images with one person

160	This  section  presents  an  experimental  analysis  on  three  key  face de-

161    tection approaches using 3D images with one person.       Subsection 3.1 ex-

162    plains Colombo et al. (2006) curvature analysis. Subsection 3.2 details  Mian

163    et al. (2006) slicing approach. Subsection 3.3 describes Segundo et al. (2007)

164    segmentation technique.  Finally, Subsection 3.4 shows experimental results.

165    3.1. Curvature analysis

166	Colombo et al. (2006) curvature analysis technique was experimentally

167    analysed and implemented considering some particularities, as follows:

16S8tep 1.-  Depth images were computed from its respective 3D point cloud using

169	equations:

f → U defined on an open set U ⊆ R2	(1)

S = (x, y, z)|(x, y) ∈ U ; z ∈ R; f (x, y) = z	(2)

17S0tep 2.-  A Gaussian filter was applied (Equation 3) to the input depth image to

171	discard high-frequency fluctuations in the surface, while salient   facial

172	features,  such as the eyes and nose,  are still clearly     distinguishable.

173	When the filter is applied to the whole image some regions as   corners

174	or face contours are not discarded due radial   distortion.

. 1 .

16

1  2 1

∗ 2 4  2	(3)

1   2  1

6

17S5tep 3.-  Mean (4) and Gaussian (5) curvature were calculated by the first (6)

176	and second (7) derivatives from every point of the depth map as in Trucco

177	and Verri (1998).

(1 + f 2)fxx − 2fxfyfxy + (1 + f 2)fyy

H(x, y) =

y	x

3

2(1 + f 2 + f 2)2

(4)

x	y

K(x, y) =

fxxfyy − f 2

xy

(1 + f 2 + f 2)2

(5)

x	y

178	where fx, fy, fxy, fxx, fyy   are the first and second derivatives of f  in

179	(x, y).  Then, a face is initially represented as a range image.  Since we

180	have only a discrete representation of S, the partial derivatives were

181	also calculated for each point of the depth  map:

f (x0 + h) − f (x0 − h)	2

ft(x0) =

+ O(h )	(6)

2h

f (x0 + h) − 2f (x0) + 2f (x0 − h)	2

ftt(x0) =

h2	+ O(h )	(7)

182	considering that x0  is a (x, y) point in the range image, h is an  integer

183	greater than zero, and O(h2) is the truncation error, caused by stopping

184	the polynomial approximation to second order, which tends to   zero.

18S5tep 4.-  HK classification were performed according to the Mean and Gaussian

186	curvature signs.  Then, elliptical convex points can be identified  when

187	K > 0 and H < 0, and those points could represent a nose (see Table

188	1).

K < 0	K = 0	K > 0

Table 1:  HK classification was introduced by Besl and Jain (1986).

H < 0   Hyperbolic concave	Cylindrical concave   Elliptical concave

H = 0   Hyperbolic symmetric   Planar	Impossible

H > 0   Hyperbolic convex	Cylindrical convex	Elliptical convex

7

18S9tep 5.-  A  thresholding  process  (Equation  8)  by  curvature  values  is  now  per-

190	formed.  Regions with highest curvature are isolated as shown in   Fig-

191	ure 3 as nose candidates, while areas with low curvature values are

192	discarded.

|H(u, v)| ≥ Th	|k(u, v)| ≥ Tk	(8)

193	where Th  and Tk  are our estimated  thresholds:

Tk  = (kmax − kmin)/4	(9)

Th  = (hmax − hmin)/4	(10)

19S4tep 6.-  Finally, in the last step the number of candidates was reduced by fil-

195	tering each elliptical convex candidate, considering average Mean  (Hl)

196	and  Gaussian  curvature (Kl):

Hl ≥ Hmin	(11)

197    3.2. Slicing analysis

Kl  ≥ Kmin	(12)

198	Mian et al. (2006) slicing technique was also studied in detail and our

199    implementation is as follows:

20S0tep 1.-  A  3D  image  is  horizontally  sliced  in  multiple  steps  using  a  threshold

201	dv. For each slice, spikes are removed using dt. The dt is automatically

202	calculated by using equation  13.

dt = µ + 0.6σ	(13)

203	where µ is the mean distance between eight neighbouring points   with

204	standard  deviation σ.

8

20S5tep 2.-  After removing spikes, the 3D image may result in holes.  Those holes

206	were refilled by cubic interpolation.  Then, circles were created centred

207	at multiple horizontal intervals dh on each slice for a region segmenta-

208	tion.  In each segmented area, a triangle was inscribed from the   centre

209	of the circle and two  intersection points coinciding by  the circle    and

210	slice segmented points.        The circles were created with a 2cm radius

211	associated to the mean nose size from the tip to the end of the nose.

21S2tep 3.-  In each slice must be a maximum altitude triangle.  This triangle can

213	be considered  as a potential nose tip.   A confidence  value,  equal   to

214	triangle’s altitude is assigned to it for further comparison with the

215	other maximum altitude triangles from other  slices.

21S6tep 4.-  After getting the confidence values from Step 3.- associated to all the

217	slices, one nose tip candidate point per slice is identified.  Those candi-

218	dates were analysed for nose ridge identification.  The canditate  points

219	that do not correspond to the nose ridge were considered as  outliers

220	and were removed using RANSAC (Trucco and Verri, 1998).

22S1tep 5.-  Finally, the few canditate points were evaluated and the one with max-

222	imum altitude was defined as the nose tip.  Then, the whole face can

223	be extracted from detected nose tip.

224  3.3. Segmentation technique

225	Our Segundo et al. (2007) slicing technique implementation was as follows:

22S6tep 1.-  The input depth images (see Figure 1-a) for this algorithm were gen-

227	erated  from  a 3D point  cloud  by  using equations  (1)  and (2).   The

228	complete process consists of six stages: K-Mean segmentation, face re-

229	gion extraction, Sobel operation, closing, ellipse detection, binarization

230	and segmentation (see Figure  1).

23S1tep 2.-  K-Means algorithm Krishna and Murty (1999) is applied by setting k =

232	3, so the input image was segmented in three main regions: background,

233	body,  and face (see Figure 1-b).   The whole background is in    black,

234	the body is in gray and the region of interest, the face, is in white (see

235	Figure 1-c). Only this white region is extracted and is used in following

236	steps.

9

23S7tep 3.-  In this step, edge detection is applied using Sobel operator and the face

238	region is now represented by edges (see Figure 1-d).

23S9tep 4.-  Then, only the bigger edge is kept (see Figure 1-e) to which an ellipse

240	is fit using Hough Transform.      The resulting image is a clean binary

241	image (see Figure 1-f) with the possible face  shape.

24S2tep 5.-  Finally, to obtain an edge map with the detected and segmented face, a

243	closing process was applied to the resulting image in the last step.   By

244	using an and operation with this image and the original input depth

245	image, the face region is obtained.

246   3.4. Experimental Results

247	We have performed an experimental procedure for comparison and analy-

248     sis of those techniques using CurtinFaces (Li et al., 2013) and FRGC (Phillips

249    et al., 2005) databases.  These databases were considered because, the FRGC

250    database is the most commonly used in face processing research, and Curtin-

251    Faces database was collected with Kinect 360 sensor and images are   similar

252    to those collected in this investigation.  A performance summary is shown in

253    Table 2.

Table 2:  Performance summary:  Face detection in 3D images with one person.

Technique	Overall performance FRGC [%]    Overall performance CurtinFaces [%]

| Curvature Analysis | 98.48% | 75.18% |
| --- | --- | --- |
| Slicing analysis | 92.74% | 57.47% |
| Face segmentation | 97.19% | 46.88% |

254	We implement every face detection approach by developing specific  pur-

255    pose functions or using standard available libraries in MATLAB version  15.

256    For  the three face detection algorithms,  their performance is calculated    by

257     counting those facial areas that contain the tip of the nose landmark ac-

258    cording reference ground truth Creusot et al. (2010).  For  slicing   technique

259    we used only the tip of the nose to detect a face.      For the curvature and

260            segmentation techniques we used the corners of the eyes and the tip of the

261     nose.

10

Figure 1:          Illustration of our Segundo et al. (2007) implementation:  (a) range image,

- K-Means algorithm result, (c) face region extracting, (d) Sobel operator result, (e) closing process result, (f) ellipse detection with Hough Transform (g), binary image after normalization, and (h) final  segmentation.

##### 262   4. Face detection in 3D images with more than one person

263	This  section  investigates  the face detection problem with  images    that

264    contains more than one person in the scene.     Subsection 4.1 introduces our

265    face detection approach.   Subsection 4.2 describes our experimental   image

11

266    acquisition.  Finally, Subsection 4.3 shows our experimental results.

267    4.1. Face Detection Approach

268	Taking in consideration results and knowledge obtained from the  experi-

269    mental analysis described in Section 3, we propose a novel approach that   is

270    able to detect every present face within an image that contains more  than

271    one person in the scene.  As illustrated in Figure 2 this approach consists   of

272    four steps:  a) Elimination of planar regions, b) Curvature analysis, c)   Nose

273      tip candidate selection, and d) Face/No-Face classification.  A description  in

274      detail of this steps is as   follows.

3D

Face Data Base

3D

Image

M-estimator Sample Consensus (MSAC)

1.- Elimination of planar regions in the 3D Image

3D image without plane parts

Mean and Gaussian Curvature Computation

2.- Curvature analysis

Points Within Elliptical Convex

KNN/KD-Tree SpinImages

3.- Face candidates (at nose tip vertex

level)

Face candidate histograms

(PCA or SVM) + KNN

Face

4.- Classification (Face/No Face)

Figure 2: Experimental procedure to detect every face in a 3D image consisting of four main steps.

27S5tep 1.-  For  every  3D  image,  planar  areas  were  eliminated  using  M-estimator

276	Sample Consensus (MSAC). MSAC is a modification of the RANSAC

277	technique, where the aim is to adjust planes in the point cloud using   a

278	distance d, when set the plane the distance of the points is calculated

279	to it in order to obtain two sets of points, inliers and outliers.

280	The inliers are the vertices that fit the calculated plane and the outliers

281	are those that distance to the plane is greater that d.       Our MSAC

282	technique implementation consists of five steps:

283	(a)  The input of the MSAC algorithm is a point cloud and a   distance

284	d.

12

285	(b) A plane P 1· X + P 2· Y + P 3· Z + P 4 = 0 is adjusted to a part of

286	the 3D image with the  operation:

X1	Y 1	Z1 1 P 1

X2	Y 2	Z2  1 	

	  P 2

= Ap¯

(14)

 ..	..	..

..  P 3

 .	.

.	.  	

Xn   Y n  Zn  1	P 4

- The optimal parameters are calculated. We defined the optimal parameters as the minimum values of plane   adjustment:

2

p¯opt = argmin||Ap¯||	(15)

2

Calculate the estimation error of the distance of each point to the plane:

e2  = ([x y z 1]p¯opt)

(16)

P 2	2	2

1 + P2 + P3

287	(e)  Select the inliers using a distance d.  If the distance of a point   is

288	less or equal than the value of d it is considered inliers,  otherwise

289	are outliers.

290	We  selected the outlier because they are points that are not    planar

291	regions.

29S2tep 2.-  Then, we used curvature analysis for detecting convex elliptical points,

293	the steps for implementation of the curvature analysis   are:

294	(a) We started with a range image that is a representation of the

295	coordinates (x, y, z) of the 3D image in a location (i, j).        The

296	depth images are computed from respective 3D point cloud  using

297	equations (1) and  (2).

298	(b)  For every point within the depth map, Mean (4) and Gaussian  (5)

299	curvature are computed.

300	(c)     Then, by analyzing the signs of the Mean and the Gaussian cur-

301	vature, we perform what is called an HK classification (Besl   and

302	Jain, 1986). For this case, we have only selected the convex  ellip-

303	tical points as illustrated in Figure 3.

13

Figure 3: The black vertices are convex elliptical  points

304	At this point, we mostly got those points around the subjects body  and

305	head.

30S6tep 3.-  For every vertex in the 3D point cloud, Spin Images, as prescribed by

307	Johnson and Hebert (1998) were computed using the following  steps:

308	(a)  Select a point P  in the point cloud.

309	(b)  Calculate the normal of P using a radius r to select neighbours  of

310	P

- Compute α and β values:

α = ,

x − p||

− (n · (x − p))2

(17)

2

||

β = (n · (x − p))	(18)

- Every alpha and beta coordinates contribute to its respective (i, j) spin image histogram:

i = [

βmax − β

bin

]   ,	j = [

α bin

]	(19)

311	where bin is a predefined constant, which in this research was

312	experimentally calculated as 0.2.

14

313	To speed up the selection process to create a spin image histogram,  we

314	are using Nearest Neighbour and KD-Tree algorithms. To calculate  the

315	nearest neighbours of each vertex KD-tree was used for partitioning

316	the space that organizes the points in a k-dimensional Euclidean space.

317	With this partitioning, we can get the neighbours of each vertex using

318	KNN, an 8cm distance was used for neighbours selection to generate the

319	spin image histogram. Figure 4 shows spin image histograms computed

320	from three different databases at the nose tip level.

Figure 4:  Spin image histogram samples:  (a) FRGC (b) Curtin Faces (c)  BDUAEMex.

32S1tep 4.-  Face/No-face  classification  was  done  using  PCA/SVM  classifiers  and

322	KNN.

323	For PCA classification we compare spin image histograms of remaining

324	nose tip candidates.  For  this,  we  see a [p x q] histogram as an   [m x 1]

325	feature vector in an m = pq dimensional space.  Separate training  and

326	testing sets were defined and we used a one-nearest neighbour  scheme

327	as proposed by Pears et al. (2012) based on Mahalanobis distance as

328	follows:

329	PCA training

(a) For the set of n training images, xi , i = 1...n, where each training face is represented as an m-dimensional column vector:

x = [x1, ..., xm]T	(20)

15

stack the n training face vectors together to construct the [n x m] training data matrix:

X

  T 

1



X = 



...

X

T

n

	(21)

330	where each column vector represents Spin Image histogram .    In

331	this experiment, [16x16] histograms from 200 tips of the nose were

332	used.

333	(b)  The mean of the training set is   calculated:

x¯ =

n

1

. xi	(22)

n

i=1

334	to form the zero-mean training data matrix:

Xzmean  = X − J(n,1)x¯T	(23)

335	where J(n,1)  is an n x 1 matrix of ones.

336	(c)  Perform a standard eigen descomposition. Several variants of PCA

337	based 3D face recognition or classification exist in the    literature

338	(Colombo et al., 2006; Bouzalmat et al., 2014; Gumus et al., 2010;

339	Pears et al., 2012). We used Singular Value Decomposition (SVD)

340	directly on the n x m zero-mean training data matrix,   Xzmean.

341	The advantage of using SVD is that it can often provide superior

342	numerical stability compared to eigen descomposition algorithms,

343	additionally, the storage required for a data matrix is often   much

344	less than a covariance matrix (Pears et al., 2012).  The SVD  is:

USV T = Xzmean	(24)

345	where  U  and  V  are  orthogonal  matrices  of  dimension n x n and

346	mxm respectively and S is a nxm matrix of singular values along

347	its diagonal.

348	(d)  Select the number of subspace dimensions for projection.  This  is

349	the dimensionality reduction step and is usually done by analysing

350	the ratio of cumulative variance associated with the first k dimen-

351	sions of the rotated image space to the total variance associated

352	with the full set of m dimensions in that   space.

16

353	(e)  Project the training data set into the k-dimensional    subspace:

X˜  = Xzmean  Vk	(25)

354	where Vk  is a m x k matrix containing the first k eigenvectors  of

355	V  and X˜  is a nxk matrix of n training faces in the k-dimensional

356	subspace.

357	PCA testing

358	Once the above PCA training phase is completed, it is straightforward

359	to implement a simple nearest neighbour face identification scheme,

360	within the reduced k-dimensional space.

(a) Project the testing set by subtracting the mean vector times the derived subspace PCA Vk:

X˜ T

t   = (Xt − X)T

Vk	(26)

361	(b) Compute the Euclidean distance between every face in the training

362	data set against the testing set.  The testing candidate with   the

363	smallest distance is considered the face nose  tip.

364	Support Vector Machine (SVM)

365	Support vector machine (SVM ) is considered a supervised learning

366	method. The  basics  of  SVM  are  rooted  in  the  work  on  statistical

367	learning theory and were introduced in the 90s by Vapnik et al. (1996).

368	Within the task of classification, SVM belong to the category of linear

369	classifiers, since they induce linear or hyperplanes separators:   separa-

370	ble,  quasi-separable,  transformed space (Gumus et al., 2010;  Bouzal-

371	mat et al., 2014; Tello et al., 2013). While most learning methods focus

372	on minimizing the errors computed by  the generated model from    the

373	training sample,  the inductive bias associated with the SVM      lies in

374	minimizing the structural risk (Vapnik et al.,   1996).

375	For  the implementation of SVM, the 3D vertexes have  been    filtered

376	previously as candidate to nose tip.  The idea is to select a separa-

377	tion hyperplane equidistant from the closest examples of every class,

378	in other words, to get what is called a margin maximum on either  side

17

379	of the hyperplane margin.   Moreover,  when defining the   hyperplane,

380	only the training examples of each class that lay in the border of   these

381	margins are considered (support vectors).  Therefore, support    vectors

382	are subsets of training observations that are used to support an  optimal

383	location decision surface (Vapnik et al., 1996).

384	We  train SVM with 200 correct nose tip histograms and 200 nose    tip

385	shaped histograms that were actually:  clothes, eyebrows or chins.    As

386	we can see in Table 3, correct and incorrect nose tip histograms are  la-

387	belled as 1 and -1, respectively. The SVM implementation were carried

388	out using Matlab Support Vector Machines toolbox for  Classification.

Table 3: Data encoding matrix exemplifying the distribution of the SVM training.

c1	c2	c3	c4	c5	c6	· · ·   c256	d1

P1	1	0	1	1	0	1	· · ·	1	1

P2	0	1	0	1	0	1	· · ·	0	-1

P3	1	0	1	1	0	1	· · ·	1	1

· · ·	· · ·   · · ·   · · ·   · · ·   · · ·   · · ·   · · ·	· · ·	· · ·

P400	1	0	1	1	0	1	· · ·	1	1

389	We used an optimized Matlab function (svmtrain) to identify   support

390	vectors si, weights αi, and bias b for a given kernel k to classify a  testing

391	set x:

c = . αik(si, x) + b	(27)

i

392	We use KNN to calculate the distances of our training set and candi-

393	dates to nose tip, the point with the shortest distance is selected.

394	Finally, from that selected nose tip the facial surface is extracted   from

395	the image using an 8cm radio.

396    4.2. Experimental data acquisition

397	As we know, state of the art 3D databases have only one person per

398  image and they were collected in controlled illumination, mostly neutral facial

399   expressions and front pose (Senthilkumar and Gnanamurthy, 2014). Hence,

400    there are no 3D experimental images for the purpose of this  research.

401	To investigate face detection in 3D images with one person, we are  using

402    two state of the art databases: Face Recognition Grand Challenge (Phillips

18

1m

0.80m

Figure 5: A one-depth image with eight people in the  scene

403  et al., 2005) and CurtinFaces (Li et al., 2013). From the FRGC and Curtin-

404    faces databases, we are using 4950 and 1437 3D images, respectively.

405	For our face detection experiment in 3D images with more than one per-

406    son,  a capture session was  performed using the Kinect     OneTM   sensor,  to

407           collect 2D/3D images varying the number and position of eight subjects in

408    the scene. Figure 5 shows how people were placed within the field of view of

409    the 3D sensor.

410	Our data collection  is limited to eight persons  per scene,     because  the

411    Kinect OneTM  sensor field of view is not able to capture more people without

412    reducing the number of vertices per face.  This is well known as a  challenge

413    in 3D face processing Bowyer et al. (2006).

414	Four sets of images were collected varying in each scene the number  and

415    the position of eight subjects, giving 28  different positions and counting 1020

416    images in total not considering images with no  subject.

417	For training and performance evaluation it was necessary to collect ground

418    truth data.  As we are assuming that a face exists into an image if its nose  is

419    present. Therefore, we meticulously collect ground truth values at nose tip

420    vertex level, by manually identifying the nose as the most prominent part  in

421    a face in a MATLAB 3D plot.

422	To  implement and evaluate our experimental procedure to detect   any

423    present face into a 3D image, using our set of 1020 depth images we defined

424  different training and testing sets as indicated in Table 4.

19

Table 4: Training ad testing sets using depth images containing from one to eight different people.

| Set | Number of images | Number of faces |
| --- | --- | --- |
| Training | 100 | 200 |
| Testing | 920 | 3896 |
| Total | 1020 | 4096 |

425   4.3. Experimental Results

426	Using our face data set containing from one to eight persons in each

427    scene we  have experimentally evaluated our face detection procedure.       In

428    this case, from the resulting spin image classification we find first the    local

429    maximum vertex for every cluster of candidates.  Then,  localization    errors

430    were computed as the Euclidean distance between the localised nose tip  and

431    its respective ground truth value.        Table  5 summarises our face detection

432      performance.       As we  can see, we  are successfully detecting 91% of faces in

433    our experimental face images, 100% of faces in the FRGC database and 98%

434    of faces in the CurtinFaces database within a localization error lower or equal

435    to 16mm (see Figure 6 and 7).

436   5. Conclusions

437	In this paper we have presented a novel pose invariant experimental   pro-

438    cedure for automatic face detection in 3D images, which consist of four steps:

439    Planar region elimination, curvature analysis, spin image analysis and classi-

440    fication.  This localisation approach was evaluated using our 3D image   data

441    set (that contains from one to eight different subjects) and state of the  art

442   databases (FRGC and CurtinFaces) that were collected considering only one

443    person per image.

444	To begin our research, three key approaches in related literature were

445    experimentally investigated toward our final aim.      The first approach uses

446          a curvature analysis to detect the most rigid facial area where the inner-

447                 eyes corners and the tip of the nose are expected.   The second approach

448  horizontally  slices  an  income image to  localize  the most likely  facial  area

449    that contains the tip of the nose.  The third approach uses a  segmentation

450    procedure to detect an elliptical area where a human face is   expected.

451	For  the  three  experimented  datasets  (Our  database  with  images  with

452    more than one subject, FRGC and CurtinFaces) we have gathered location

453             errors by computing the Euclidean distance from the selected nose tip and

20

Figure 6:  Performance summary in our face detection approach using  PCA

454      their respective ground-truth.  Our best experimental results, using a    SVM

455    classifier, shows that within a localization error between 0 to 16 mm,  100%,

456               98% and 91% of faces in every image in the FRGC, CurtinFaces and our

457      dataset are located.

458	Despite of the Kinect OneTM           low resolution and limited field of view,

459    our face detection approach has been shown robust for nose tip  localisation.

460    Then, our approach assume the presence of a face by locating its nose   tip.

461	On the other hand, our face detection approach as been experimented

462    using pre-stablish thresholds, which were experimentally calculated for  each

463     step.

464	As part of the future work, we are studying alternate techniques to im-

465    prove our face detection approach performance. Additionally, it is mandatory

466    to consider a face extraction technique for any counted face within an  image

467    to complete our process.

21

Figure 7:  Performance summary in our face detection approach using SVM

468    Acknowledgments

469	Authors thank to the Research Department of the    Autonomous  Univer-

470     sity of the State of Mexico (UAEMex) for their financial support, grant

471    3720/2014/CID. Also, our gratitude goes to CONACYT for Mr.    Paduanots

472      scholarship 634473.

Table 5: Face detection summary using FRGC, CurtinFaces and our point cloud images containing from one to eight subjects at 16mm.

| Data Bases Performance using PCA [%] Performance using SVM [%] |  |  |
| --- | --- | --- |
| FRGC | 98% | 100% |
| CurtinFaces | 97% | 98% |
| Our DataBase | 85% | 91% |

22

473  References

474    Aghaei, M., Dimiccoli, M., Radeva, P., 2016. Multi-face tracking by extended

475	bag-of-tracklets in egocentric photo-streams. Computer Vision and   Image

476	Understanding  149,  pp.  146  –  156,  special  issue  on  Assistive Computer

477	Vision and Robotics - Assistive Solutions for     Mobility,  Communication

478	and HMI.

479    Ariz, M., Bengoechea, J. J., Villanueva, A., Cabeza, R., 2016. A novel 2d/3d

480	database with automatic face annotation for head tracking and pose esti-

481	mation. Computer Vision and Image Understanding 148, pp. 201 –  210,

482	special issue on Assistive Computer Vision and Robotics - Assistive Solu-

483	tions for Mobility, Communication and  HMI.

484    Belahcene,  M.,  Chouchane,  A.,  Mokhtari,  N.,  Nov 2014. 2d and 3d  face

485	recognition based on ipc detection and patch of interest regions. In:   Con-

486	nected Vehicles and Expo (ICCVE), 2014 International Conference on. pp.

487	627–628.

488    Besl, P.  J., Jain, R. C., 1986. Invariant    surface characteristics for 3d ob-

489	ject recognition in range images. Computer Vision, Graphics, and    Image

490	Processing 33 (1), pp. 33 –  80.

491   Bhandarkar, S. M., Luo, X., 2009. Integrated detection and tracking of mul-

492	tiple faces using particle filtering and optical flow-based elastic  matching.

493	Computer  Vision  and Image Understanding  113 (6), pp. 708 –   725.

494    Bouzalmat, A., Kharroubi, J., Zarghili, A., 2014. Comparative study of   pca,

495	ica,  lda using svm classifier. Journal  of Emerging Technologies  in    Web

496	Intelligence 6 (1), pp. 64–68.

497            Bowyer, K. W., Chang, K., Flynn, P., 1 2006. A survey of approaches and

498	challenges in 3d and multi-modal 3d + 2d face recognition.   Computer

499	Vision and Image Understanding 101 (1), pp.   1–15.

500    Cai, D., He, X., Han, J., Zhang, H. J., Nov 2006. Orthogonal   laplacianfaces

501	for face recognition. IEEE Transactions on Image Processing 15 (11),   pp.

502	3608–3614.

503            Chang, K. I., Bowyer, K. W., Flynn, P.  J., Oct 2006. Multiple nose region

504	matching for 3d face recognition under varying facial expression. IEEE

23

505	Transactions on Pattern Analysis and Machine Intelligence 28 (10), pp.

506	1695–1700.

507  Chellappa, R., Wilson, C., Sirohey, S., May 1995. Human and machine recog-

508	nition of faces: a survey. Proceedings of the IEEE 83 (5), pp. 705–741.

509           Chen, W., Sun, T., Yang, X., Wang, L., Aug 2009. Face detection based on

510	half face-template. In:  Electronic Measurement Instruments, 2009. ICEMI

511	’09. 9th International Conference on. pp. 4–54–4–58.

512           Colombo, A., Cusano, C., Schettini, R., Mar 2006. 3d face detection using

513	curvature analysis. Pattern Recognition 39 (3), pp. 444 – 455.

514    Creusot, C., Pears, N., Austin, J., October 2010. 3d face landmark  labelling.

515	In:  Proceedings of the ACM workshop on 3D object retrieval. ACM,  As-

516	sociation for Computing Machinery,  pp.  27–32.

517    Gumus, E., Kilic, N., Sertbas, A., Ucan, O. N., 2010. Eigenfaces and support

518	vector machine approaches for hybrid face recognition. pattern recognition

519	8, pp. 9.

520      Hjelm˚as, E., Low, B. K., 2001. Face detection:  A survey. Computer Vision

521	and Image Understanding 83 (3), pp. 236 –    274.

522    Hu,  W.,  Hu,  W.,  Maybank,  S.,  April 2008. Adaboost-based algorithm for

523	network intrusion detection. IEEE Transactions on Systems, Man,   and

524	Cybernetics, Part B (Cybernetics) 38 (2), pp. 577–583.

525    Hutton, T. J., Buxton, B. F., Hammond, P., 2003. Automated registration of

526	3d faces using dense surface models. In:  In Proc. British Machine  Vision

527	Conference.  pp. 439–448.

528    Johnson, A. E., Hebert, M., 7 1998. Surface matching for object  recognition

529	in complex three-dimensional scenes. Image and Vision Computing 16    (9–

530	10), pp. 635–651.

531              Kakadiaris, I. A., Toderici, G., Evangelopoulos, G., Passalis, G., Chu, D.,

532	Zhao,  X.,  Shah,  S. K.,  Theoharis,  T., 2016. 3d-2d face recognition with

533	pose and illumination normalization. Computer Vision and Image   Under-

534	standing, –.

24

535    Krishna, K., Murty, M. N., 1999. Genetic k-means algorithm. Systems, Man,

536	and Cybernetics, Part B: Cybernetics, IEEE Transactions on 29 (3), pp.

537	433–439.

538      Kumar, A., Datta, M., Kumar, P. B., 2015. Face Detection and  Recognition:

539	Theory and Practice. Chapman and Hall/CRC.

540    Li, B. Y. L., Mian, A. S., Liu, W., Krishna, A., Jan 2013. Using kinect for face

541	recognition under varying poses, expressions, illumination and disguise. In:

542	Applications of Computer Vision (WACV), 2013 IEEE Workshop on.   pp.

543	186–192.

544            Maes, C., Fabry,  T., Keustermans, J., Smeets, D., Suetens, P.,  Vander-

545	meulen, D., 2010. Feature detection on 3d face surfaces for pose   normali-

546	sation and recognition. In:  Biometrics:  Theory Applications and  Systems

547	(BTAS), 2010 Fourth IEEE International Conference on. pp. 1–6.

548           Mian, A., Bennamoun, M., Owens, R., 2006. Automatic 3d face detection,

549	normalization and recognition. In:  3D Data Processing, Visualization, and

550	Transmission, Third International Symposium on. pp. 735–742.

551    Nair, P., Cavallaro, A., 2009. 3-d face detection, landmark localization,   and

552	registration using a point distribution model. Multimedia, IEEE    Transac-

553	tions on 11 (4), pp. 611–623.

554    Nanni,  L.,  Lumini,  A.,  Dominio,  F.,  Zanuttigh,  P.,  1 2014. Effective and

555	precise face detection based on color and depth data. Applied   Computing

556	and Informatics 10 (1–2), pp. 1–13.

557      Paduano, J., Romero, M., Mun˜oz, V., Jul 2015. Toward face detection in 3d

558	data. In:  International Conference on Image Processing, Computer Vision,

559	& Patter recognition. Vol. 15. pp.  473–479.

560    Paduano,  J.,  Romero,  M.,  Valdovinos,  R.,  Jul 2016. Face  detection in 3d

561	images with more than one person. In: International Conference on Image

562	Processing, Computer Vision, & Patter recognition. pp. 287–293.

563          Pan, H., Zhu, Y., Xia, L., 2013. Efficient and accurate face detection using

564	heterogeneous feature descriptors and feature selection. Computer   Vision

565	and Image Understanding 117 (1), pp. 12 –    28.

25

566    Pears, N., Liu, Y., Bunting, P., 2012. 3D Imaging, Analysis and Applications,

567	1st Edition. Vol. 1. Springer-Verlag London.

568    Peng, X., Bennamoun, M., S., M. A., 2011. A training-free nose tip detection

569	method from face range images. Pattern Recognition 44 (3), pp. 544 – 558.

570   Phillips, P. J., Flynn, P. J., Scruggs, T., Bowyer, K. W., Chang, J., Hoffman,

571	K., Marques, J., Min, J., Worek, W., 2005. Overview of the face recognition

572	grand challenge. Computer Vision and Pattern Recognition, 2005.   CVPR

573	2005. IEEE Computer Society Conference on 1, pp.   947–954.

574    Segundo, M. P., Queirolo, C., Bellon, O. R. P., Silva, L., 2007. Automatic

575	3d facial segmentation and landmark detection. In:      Image Analysis and

576	Processing, 2007. ICIAP 2007. 14th International Conference on. pp. 431–

577	436.

578    Senthilkumar, R., Gnanamurthy, R. K., April 2014. A detailed survey on   2d

579	and 3d still face and face video databases part i. In:  Communications  and

580	Signal Processing (ICCSP), 2014 International Conference on. pp. 1405–

581	1409.

582   Smeulders, A. W. M., Chu, D. M., Cucchiara, R., Calderara, S., Dehghan, A.,

583	Shah, M., 2014. Visual tracking:  An experimental survey. Pattern Analysis

584	and Machine Intelligence, IEEE Transactions on 36 (7), pp. 1442–1468.

585             Soldera, J., Behaine, C. A. R., Scharcanski, J., Sept 2015. Customized or-

586	thogonal locality preserving projections with soft-margin maximization for

587	face recognition. IEEE Transactions on Instrumentation and Measurement

588	64 (9), pp. 2417–2426.

589    Subburaman, V. B., Marcel, S., 2013. Alternative search techniques for  face

590	detection using location estimation and binary features. Computer Vision

591	and Image Understanding 117 (5), pp. 551 –    570.

592    Tello, J. C. C., Hernandez-Ramirez, D., Garcia-Sepulveda, C. A., 2013. Sup-

593	port vector machine algorithms in the search of kir gene associations  with

594	disease. Computers in Biology and Medicine 43 (12), pp. 2053    –  2062.

595           Trucco, E., Verri, A., 1998. Introductory Techniques for 3-D Computer Vi-

596	sion. Prentice Hall.

26

597              Valstar,  M. F., Mehu, M., Jiang, B., Pantic, M., Scherer, K., 2012. Meta-

598	analysis of the first facial expression recognition challenge. Systems, Man,

599	and Cybernetics, Part B: Cybernetics, IEEE Transactions on 42 (4), pp.

600	966–979.

601   Vapnik,  V., Golowich, S. E., Smola, A., 1996. Support vector method for    602	function approximation,  regression estimation,  and signal processing. In: 603	Advances in Neural Information Processing Systems 9.  Citeseer.

604 Wagner, A., Wright, J., Ganesh, A., Zhou, Z., Mobahi, H., Ma, Y., 2012. To- 605	ward a practical face recognition system:  Robust alignment  and illumina- 606	tion by sparse representation. Pattern Analysis and   Machine Intelligence, 607	IEEE Transactions on 34 (2), pp. 372–386.

608   Wang, P., Ji, Q., 2007. Multi-view face and eye detection using discriminant

609	features. Computer Vision and Image Understanding 105 (2), pp. 99   –  111.

610  Wohlhart, P., Lepetit, V., feb 2015. Learning descriptors for object recogni-  611	tion and 3d pose estimation. In:  Computer Science:  Computer Vision and 612	Pattern  Recognition.

613  Yang,  M.,  Ahuja,  N.,  2001. Face  Detection and Gesture Recognition for   614	Human-Computer  Interaction,  1st  Edition.  The  International   Series  in 615	Video Computing 1. Springer US.

616  Yang,  M.,  Feng,  Z.,  Shiu,  S. C. K.,  Zhang,  L.,  2 2014. Fast  and robust  617	face recognition via coding residual map learning based adaptive masking. 618	Pattern Recognition 47 (2), pp. 535–543.

619  Yang, M., Kriegman, D., Ahuja, N., Jan 2002. Detecting faces in images:  a  620	survey. Pattern Analysis and Machine Intelligence, IEEE   Transactions on 621	24 (1), pp. 34–58.

622 Yin, S., Ouyang, P., Dai, X., Liu, L., Wei, S., 2015. An adaboost-based face  623	detection system using parallel configurable architecture    with optimized 624	computation. IEEE Systems Journal PP (99), pp. 1–12.

625  Zafeiriou, S., Zhang, C., Zhang, Z., 2015. A survey on face detection in the  626	wild:  Past, present and future. Computer Vision and Image Understanding 627	138, pp. 1 – 24.

27

628   Zhang, J., d. Zhang, X., w. Ha, S., Oct 2008. A novel approach using pca and    629	svm for face detection. In:  Natural  Computation,  2008. ICNC   ’08. Fourth 630	International Conference on. Vol.  3. pp. pp.  29–33.

28

Capítulo 4 Discusión

#### En esta investigación se propuso un proceso para la detección de la superficie facial en imágenes 3D con variaciones en el número y posición de las personas con respecto al sensor Kinect One™. Este proceso consiste en cuatro etapas; La primera es la eliminación de parte planas de la imagen. La segunda es un análisis de curvatura para seleccionar puntos 3D convexos. La tercera es la selección de candidatos a punta de la nariz, y la cuarta es una clasificación de rostro/No rostro. Para esta clasificación se experimento el uso de dos técnicas; PCA y SVM.

#### Las contribuciones originales que se derivaron de este trabajo de investigación son: un proceso para la detección y localización de la superficie facial en imágenes 3D de una profundidad y un conjunto de 1020 imágenes 3D capturadas con el sensor Kinect One™ variando el número y la posición de treinta y dos sujetos. Con estás contribuciones se cumple el objetivo general de ésta tesis.

#### Se evaluó el proceso de detección propuesto en este trabajo de investigación utilizando dos bases de datos del estado del arte (Face Recognition Grand Challenge y CurtinFaces) y nuestra base de datos, obteniendo resultados competitivos contra los del estado del arte.

#### Para confirmar los resultados obtenidos con los clasificadores PCA y SVM, se corrieron tres pruebas experimentales variando los doscientos rostros de entrenamiento de nuestra base de datos considerando las tres resoluciones con las que fue colectada (alta, media y baja). La alta resolución la etiquetamos a las imágenes donde solo se encuentra una persona, la de media resolución es donde están cuatro personas y finalmente, la baja resolución es donde existen ocho personas en la escena.

#### Experimento 1: Esta prueba se realizó utilizando imágenes que contienen una, dos y tres personas. Esta selección se realizó de manera manual, con el propósito en que el algoritmo aprendiera la resolución de las imágenes. Entonces, la ideología de esta prueba es que el algoritmo

48	Discusión

#### conozca altas resoluciones. Los resultados fueron un 85 % (ver Figura 6 Capitulo 3, Sección 4.3) utilizando PCA y un 91 % (ver Figura 7 Capitulo 3, Sección 4.3) utilizando SVM como se pueden observar en la Tabla 4.1.

#### Experimento 2: Esta prueba se realizó utilizando imágenes que contienen una, dos y tres personas pero ahora la selección de las imágenes fue aleatoriamente. Esta prueba se realizó para confirmar los resultados del experimento uno. Aunque los resultados disminuyeron el 3 % como se puede observar en las Figuras 4.1. En este caso no se ve muy afectado al utilizar SpinImages en la generación de los histogramas, porque, no se esta modificando significativamente la resolución de las imágenes que se utilizan para generar el modelo medio.

#### Experimento 3: Esta prueba fue realizada utilizando imágenes 3D aleatoriamente, hasta tener un total de 200 rostros como las dos anteriores pruebas. En esta prueba se ve una disminución significativa en el rendimiento mostrado en la Figura 4.2, los motivos que nosotros estamos considerando del porque esta disminución en el rendimiento son los siguientes:

#### Se ve que se esta sesgando los resultados por la técnica de SpinImages, esta técnica depende de la posición y de la resolución de las imágenes; al modificar la distancia del sujeto al sensor se ve afectada el número de vértices que inciden en el rostro.

#### Existencia de oclusión en las imágenes; algunas imágenes de nuestra base de datos tienen perdida de información, esto es por el tipo de sensor utilizando ya que se quiere estudiar el número y la distancia de los sujetos al sensor.

#### El número de imágenes seleccionadas para el conjunto de entrenamiento; entre mayor cantidad de imágenes para entrenar se mejora la detección del rostro. Estos experimentos se realizaron con 200 rostros, pero el número de imágenes varía.

#### Tabla 4.1 Porcentaje de detección facial exitosa con un error máximo de 16mm.

| Experimento | PCA [%] | SVM [%] |
| --- | --- | --- |
| #1 | 85 % | 91 % |
| #2 | 82 % | 89 % |
| #3 | 62 % | 74 % |

#### Con la experimentación realizada en esta investigación al utilizar las imágenes de nuestra base de datos, se ha mostrado que es posible detectar y localizar la superficie facial de los individuos

49

(a)

(b)

#### Figura 4.1 Resultados seleccionando imágenes que tienen una, dos y tres personas en la escena aleatoriamente: La imagen a) muestra los resultados utilizando PCA en tres bases de datos, mientras que en la imagen b) muestra los resultados de las mismas bases de datos pero utilizando el clasificador SVM

50	Discusión

(a)

(b)

#### Figura 4.2 Resultados seleccionando imágenes aleatoriamente: La imagen a) muestra los resultados utilizando PCA en tres bases de datos, mientras que en la imagen b) muestra los resultados de las mismas bases de datos pero utilizando el clasificador SVM

#### presentes en una escena con variaciones en el número y posición de hasta ocho sujetos. Por lo tanto, se acepta la hipótesis de esta tesis.